| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | (Draft) 802.16n System Requirements Document |  |
| Date Submitted | 2010-09-16 |  |
| Source(s) | Eldad Zeira Acting Chair, GRIDMAN TG InterDigital | Eldad.zeira@interdigital.com |
| Re: |  |  |
| Abstract | This document captures functional requirements of the 802.16n amendment |  |
| Purpose | To serve as a basis for further development by GRIDMAN SG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Overview

The 802.16n amendment shall be developed in accordance with the P802.16 project authorization request (PAR) [1] with its accompanying Five Criteria Statement.

The standard shall be developed as an amendment to 802.16. The resulting standard shall fit within the following scope:

“This amendment specifies protocol enhancements to the IEEE 802.16 MAC for enabling increased robustness and alternate radio path establishment in degraded network conditions. Limited OFDMA PHY extensions are included for enabling operation with radio path redundancy and direct communication between subscriber stations. Also mobile base stations and mobile relay stations are supported. Support for enabling application specific specialized security suites is also provided.”

With the explanatory note:

“Operation in licensed, unlicensed and lightly licensed spectrum bands below 6 GHz with means and mechanisms to coexist with other radio access technologies (RATs) is supported.”

This document represents the high-level system requirements for the 802.16n amendment.  All content included in any draft of the 802.16n amendment shall meet these requirements. This document, however, shall be maintained and may evolve. These system requirements embodied herein are defined to ensure competitiveness of the amended standard against other mobile broadband radio access technologies in those areas defined by the PAR and Five Criteria Statement. These system requirements also call for significant gains and improvements relative to the preexisting IEEE 802.16 system that would justify the creation of the amendment. To accelerate the completion and evaluation of the standard, to improve the clarity and reduce complexity of the standard specification, and to further facilitate the deployment of new systems, the number of optional features should be minimized.

# References

- IEEE P802.16n PAR and 5C in 80216gman-10_0018r2.doc

# Definitions

|  | The definitions below have been agreed in the PAR |
| --- | --- |
| Degraded Network | The failure of one or more 802.16 network infrastructure nodes or network connectivity |
| Robustness | The capability of the network to withstand and automatically recover from degradation to provide the required availability to support mission critical applications (essential to the core function of society and the economy). E.g. the ability to recover from a single point of failure |
| Mobile Base Station | A base station which is capable of maintaining service while moving |
| Radio Path Redundancy | The ability to provide alternative paths between base stations, relay stations, and subscriber stations |
|  | Other definitions |
| HR-MS | A subscriber station that complies with the requirements for subscriber stations in this amendment |
| HR-BS | A base station that complies with the requirements for base stations in this amendment |
| HR-RS | A relay that complies with the requirements for relays in this amendment |
| HR-network | A network whose stations comply with their respective HR requirements in this amendment |
| HR-station | An HR-MS, HR-BS or HR-RS. |
| Infrastructure station | An HR-BS or HR-RS |

# Abbreviations and Acronyms

| SPOF | Single point of failure |
| --- | --- |
| HR | High Reliability (e.g. as in HR-MS) |

# General Requirements

## Backward Compatibility

HR-Network shall be backward compatible with the WirelessMAN-OFDMA or WirelessMAN-Advanced Air Interface.

## Complexity

IEEE 802.16n amendment should minimize complexity of the architecture and protocols and avoid excessive system complexity.  It should enable interoperability of access networks, support low cost devices and minimize total cost of ownership.

## Services

IEEE 802.16n should support services that require a higher degree of assurance of maintaining sufficient connectivity than can be provided by IEEE 802.16 legacy systems.  Examples of such services can be found in PPDR (Public Protection and Disaster Recovery) and M2M (Machine to Machine) communication networks for utility monitoring and control.

## Operating Frequencies

The HR-Network shall be specified to allow operation in all radio frequencies where 802.16 operates.

The HR-Network shall also be specified to allow operation in unlicensed and lightly licensed spectrum bands below 6 GHz with means and mechanisms to coexist with other radio access technologies (RATs).

As an example for frequencies used for PPDR, the following frequency bands have been identified by WRC 2003 (RESOLUTION 646 (WRC-03): Public Protection and Disaster Relief, The World Radiocommunication Conference (Geneva, 2003).)

For region 1

- 380-385 MHz

- 390-395 MHz

For region 2

- 746-806 MHz

- 806-869 MHz

- 4,940-4,990 MHz

For region 3

- 406.1-430 MHz

- 440-470 MHz

- 806-824 MHz

- 851-869 MHz

- 4,940-4990 MHz

- 5,850-5,925 MHz

## Operating bandwidths

HR-Network shall support the operating bandwidths of the WirelessMAN-OFDMA or WirelessMAN-Advanced Air Interface. This bandwidth may be supported by single or multiple RF carriers.

Other bandwidth may be considered as necessary to meet the government or operator requirements. ]

## Duplex schemes

The HR-Network shall be specified to support TDD and FDD.

# Functional Requirements

HR-Network devices shall comply with all Advanced Air Interface or 802.16 OFDMA functional requirements unless indicated below. Additional functional requirements are indicated in this section.

## Requirements related to construction and maintenance of network

This section contains requirements for IEEE 802.16n related to construction and maintenance of network. These requirements are intended to address multi-mode operation, link existence, infrastructure SPOF immunity, link reliability, mobility, security, and coexistence.

### Requirements related to multi-mode operation

Ability to dynamical change roles shall be included as defined in this clause.

#### Relay function for HR-BS

HR-Network shall support HR-BS to communicate with another HR-BS in order to support the relaying function to provide continuous network connectivity.

#### Relay function for HR-MS (RS Mode)

HR-Network shall support an HR-MS to change its role to relay data between other mobile or relay stations and a functioning HR-BS or HR-RS

#### Base Station function for HR-MS (BS Mode)

HR-Network may support an HR-MS to change its role to serve as a base station.

### Requirements related to infrastructure SPOF immunity

An HR-network shall be able to recover from any single point failure in any of its infrastructure nodes (i.e. all nodes excluding the subscriber station) or any of its radio links.

#### Standalone networks

HR-Network shall provide local connectivity to the HR-MSs within the coverage of the HR-BS without HR-BS’s connectivity to the backbone network.

When the HR-BS loses the backbone connection, the established service flow between HR-MSs within the coverage of the HR-BS should be maintained.

#### Multi-hop relaying

HR-Network shall provide a multi-hop relaying function.

#### HR-RS Sourcing and Sinking of Data

HR-Network shall support local source and sink of data at the HR-RS

### Requirements related to link existence and reliability

#### MS to MS Direct Communication In the absence of infrastructure stations

HR-MS shall provide direct communication i.e. the origination and termination of the data are at the HR-MS. HR-MS to HR-MS direct communication shall take place in the absence of infrastructure station. Association establishment procedure of an HR-MS to another HR-MS shall be supported.

##### Single hop

HR-MS shall support communication of user data and control signaling between an HR-MS and one or more HR-MSs that are 1-hop away.

##### Two-hop

HR-MS shall support forwarding of user data and control signaling between an HR-MS and one or more HR-MSs.

#### HR-MS forwarding to network

HR-MS forwarding is defined as the case where the origination and termination of data are at the HR-MS and network respectively and vice versa.

HR-Network shall support HR-MS forwarding of user data and control signaling between HR-MS and HR-BS and between HR-MS and HR-RS.  The control signaling and data transmission for the HR-MS to HR-MS direct link shall at least be capable of operating within the frequency band that the HR-BS operates.

An association establishment shall be supported.

#### Path discovery

HR-Network shall support neighbor and path discovery between HR-MSs as well as between HR-MS and HR-Network infrastructure stations. 
Path discovery can be accomplished with or without network support.

#### Path Management

HR-Network shall support establishment and maintenance of alternative paths to support fast recovery in the event of disruption. HR-Network shall provide the capability to choose the most reliable path.

#### Local Forwarding for RS and BS

HR-Network should allow local forwarding, which allows one HR-MS to communicate to one or more HR-MSs via infrastructure station without going through the backhaul.

### Requirements related to security

The HR-Network topology shall not degrade the security performance achieved with WirelessMAN-OFDMA or WirelessMAN-Advanced Air Interface in hierarchical network topology.

#### Security procedures for HR-Network

HR-Network shall support secure communication and session establishment among HR-stations, and between HR-stations and external AAA-servers.

##### Network aided mutual authentication of HR-MS and data security

HR-MSs shall be able to establish a security association with each other. A security server may be used to facilitate the establishment of security associations.

##### Autonomous (limited) mutual authentication of HR-MS and data security for direct communication

HR-MS shall be able to mutually authenticate themselves without access to a security server.

HR-MS shall be able to establish encrypted communication without access to a security server.

##### Security requirements for HR-Network nodes acting as relays

HR-station that functions as a relay shall forward security related messages between other HR-station and a security server, both during security association establishment and ongoing communications.

#### Multicast key Management

HR-Network shall provide the security architecture that provides a group of HR-MSs with authentication, authorization, encryption and integrity protection.

HR-Network shall provide multicast key management for the group of HR-MSs. The key shared within the group should be distributed securely and efficiently. HR-Network should support the group signaling procedure using multicast transmission for multicast key management efficiently.

### Coexistence requirements

#### Operation in unlicensed and lightly licensed bands

HR-Stations shall comply with regulators’ respective requirements for operation in unlicensed and lightly licensed spectrum.

#### Support for Multi-carrier operation in different licensing regimes

The HR-Network shall support multicarrier operation in licensed, unlicensed and lightly-licensed licensing environments.

An HR-MS that supports MC operation in different licensing environments shall be able of operating in all three types of spectrum at the same time.

## Requirements related to Services provided on network

### Enhancements to Unicast and Multicast communication

HR-Network shall provide optimized MAC protocols for unicast and multicast transmission to support applications of two-way communications such as Push to Talk (PTT) service among a group of HR-MS.

Examples of applications to be used in PTT service include:

- audio (e.g., speech, music)

- video

- still image

- text (formatted and non-formatted)

- file transfer

### High reliability control and reporting

The HR-network shall provide MAC enhancements to support high reliability latency intolerant control and reporting.