| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ## Text updates for Same-PHY signaling sections of Draft Document |  |
| Date Submitted | 2007-09-19 |  |
| Source(s) | John Sydor Communications Research Centre 3701 Carling Avenue Ottawa, Ontario | Voice: 613-998-2388 Fax: [Fax Number] john.sydor@crc.ca |
| Re: | Changes to Draft Standard |  |
| Abstract | Changes primarily related to comments from sessions 49 & 50 and use of the new CXCC concept. |  |
| Purpose | Editorial changes; technical changes due to new CXCC |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

## Text updates for Same-PHY signaling sections of Draft  Document

John Sydor

Communications Research Centre

Ottawa

Introduction

Changes to the operation of the  CXCC  necessitate re-writing and signaling structure changes to the operation of BSD, SSURF, and other related messages used with Same-PHY interference identification. Revisions to –085 have been made, primarily by deleting sections and adding text due to the comment. Deleted parts with respect to _085 have been noted below in the notes to the editor.

Given below are proposed changes to be made to the draft document.

### Italic script  are instructions to the editor

Red script strike-through  are deletions from the original Ref 1 text

Blue script are additions to the original Ref 1 text.

Black script is the original text of Ref 1.

Ref 1: Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems IEEE P802.16h/D2c, July 2007

(1) The following changes are proposed to Ref 1. on page 10, between lines 48 and 63. The changes conform to the new CXCC description. Additionally, changes are made to Ref 1, page 11 between lines10 and15 and to the Table 108aa on page 11 between lines 25 and 50.

6.3.2.3.62 Base Station Descriptor (BSD) message

The base station descriptor (BSD) message contains the identification, and other information about the BaseStation. This message is sent every CXCC repetition period  but only in the CX_CMI_D(n) slot of the CXCC (see 15.3.3.1,15.3.1.2.2 ) claimed by the Base Station and it is intended to be decoded as intelligible interference by subscriber stations associated to other systems (see 15.3.3.4). The BSD is also sent on an occasional basis in the CX_CMI_D(4) slot .

Additionally, changes are made to Ref 1, page 11 between lines10 and15

BS_RF_Sector_ID: The RF antenna sector ID is used to identify the RF transmitting antenna at the

base station where multiple RF antennas may be used or the azimuth direction of transmission if a Beam

Forming AAS antenna is used. It contains information about the azimuth direction (with respect to True

North) and -3 dB azimuth beamwidth of the antenna pattern. This contains information about:

-   the ID of RF transmitting antenna at the base station

- the antenna gain (dBi)

- the azimuth direction of transmission relative to the true North

- the antenna aperture at 3dB attenuation

- the antenna polarization (Vertical, Horizontal, Circular)

### Additionally, changes are made to Table 108aa- of Ref 1, page11 between lines 25 and 49

Table 108aa-BSD message format

| Syntax | Size | Notes |
| --- | --- | --- |
| BSD Message_Format( ) { |  |  |
| Management Message Type = 67 | 8 bits |  |
| BSID EIRP | 8 bits | dBm |
| BS ID | 48 bits |  |
| BS_GPS_LOC | 32 bits | 16 MSB for BS Lat 16 LSB for BS long |
| BS_HGHT | 16 bits | Height of BS antenna above sea level in meters |
| BS_RF_Sector_ID | 32 16 bits | Bits 0-7 For Azimuth of Beamwidth wrt true north, 2 degree steps Bits 8-15 for -3 dB Azimuth Beamwidth, 2 degree steps - ID: bits 0-3 - gain (dBi): bits 4-7, 2dB resolution - azimuth: bits 8-15, 2 degree resolution --3dB aperture: bits 16-23, 2 degrees resolution - polarization: bits 24-25, with: 00 - vertical, 01- horizontal, 11: circular - beam-forming: bit 26, 1=yes - bits 27-31: reserved |
| BS IP Proxy_Address_IE( ) | Variable | TLV specific |
| } |  |  |

(2) The following line should be deleted since it no longer is relevant to the new CXCC concept. This is line 51 on Page 11 of Ref 1.

The BSD message shall be preceded by a preamble and shall use QPSK 1/2, CC encoded.

(3) The following changes are proposed to Ref 1. between  page  11 Line 49 and page 12 Line 63. The changes conform to the new CXCC description. Additionally, as discussed in Session #49, the SSID is to be identified with the MAC ID of the subscriber terminal. The changes are due to comment 10 and 13 of Session 51 also

6.3.2.3.63 Subscriber Station Uplink Radio Frequency (SSURF) message

The Subscriber Station uplink radio frequency (SSURF) message is the complement to the BSD message

except it is sent on the uplink during the CMI interval claimed by the Base Station to which the SS is registered.

This message, if received by foreign (interfered-with) Base Stations, will identify the SS as being an interferer. (15.3.3.5).  The SSURF is sent on its working channel using the same power as a regular data transmission and with a sub-carrier allocation known to the interference neighbourhood.

A SSURF message shall include the following parameters to identify a subscriber station:

SSID: Subscriber station MAC (Media Access Control)  identifier, in the context of this message, identifies the transmitting SS. This SS is the source of co-channel interferences reported in this message.

BS ID: Serving Base Station associated with the SS.

BSS EIRP: The BSS EIRP is signed in units of 1 dBm. The EIRP at which the SSURF message was

sent; usually the maximum allowable EIRP for the operation of this station.

BSS_RF_Sector_ID: The RF antenna sector ID is used to identify the RF transmitting antenna at the

subscriber station. It contains information about the azimuth direction (with respect to True North) and -3

dB azimuth beamwidth of the antenna pattern transmitting the SSURF.

-   the ID of RF transmitting antenna at the subscriber station

- the antenna gain (dBi)

- the azimuth direction of transmission relative to the true North

- the antenna aperture at 3dB attenuation

- the antenna polarization (Vertical, Horizontal, Circular)

BS IP_Proxy address information: The BS IP address information uniquely identifies an associated

base station. The encoding of this field is given above in TLV format.

Table 108ab—SSURF message format

| Syntax | Size | Notes |
| --- | --- | --- |
| SSURF_Message_Format( ) { |  |  |
| Management Message Type = 68 | 8 bits |  |
| SS ID | 48 bits | MAC ID of sending SS |
| BS ID | 48 bits | Associated base station identifier |
| B SS EIRP | 8 bits | dBm |
| BSS_RF_Sector_ID | 3216 bits | Bits 0-7 For Azimuth of Beamwidth wrt true north, 2 degree steps Bits 8-15 for -3 dB Azimuth Beamwidth, 2 degree steps - ID: bits 0..3 - gain (dBi): bits 4-7, 2dB resolution -azimuth: bits 8-15, 2 degree resolution --3dB aperture: bits 16-23, 2 degrees resolution - polarization: bits 24-25, with: 00 - vertical, 01- horizontal, 11: circular - beam-forming: bit 26, 1=yes - bits 27-31: reserved |
| BS IP Proxy_Address_IE( ) | Variable |  |
| } |  |  |

The SSURF message is sent in the claimed uplink CMI  by a SS only if a CMI it  is scheduled. in its UL MAP.  The SSURF message shall be preceded by a preamble and shall use QPSK 1/2, CC encoded.

(4) The following changes are proposed to Ref 1. on page 19, Line 31 to Page 20, Line 42. The change is a deletion to an interference measuring approach superseded by the new CXCC operation and other changes related to the new CXCC concept. Additional changes undertaken to clarify operation with the context of the new CXCC; RSSI parameter modified as well. Table 108aj modified as a consequence….including the title which was incorrect.

BS RF_Sector_ID: The RF antenna sector ID is used to identify the RF antenna at the interfering

base station where multiple RF antennas may be used; this information is derived from the BSD. It contains

information about the azimuth direction (with respect to True North) and 3 dB azimuth beamwidth of the

antenna pattern that created the interference.

-   the ID of RF transmitting antenna at the subscriber station

- the antenna gain (dBi)

- the azimuth direction of transmission relative to the true North

- the antenna aperture at 3dB attenuation

- the antenna polarization (Vertical, Horizontal, Circular)

CX_CMI_D(n):  The Coexistence Messaging Downlink Interval (where n=1-34) during which the interference was received. If this variable contains zero, detection of interference was done during silent period of sub-channel 1 of CXCC, and not during CMI interval.

INT_BSD_Frq:  This parameter is used to quantify the number of interference instances  either  from other  same-PHY WirelessMAN-CX  sources or from non-WirelessMAN-CX sources. Quantification is done by counting interference events per CXCC repetition periods (5.12 sec;  see 15.3.1.1) The frequency of interference BSD events detected per  Coexistence Control Channel (Tcxcc) cycles (calculated as the number of BSD interference events per N Tcxcc cycles [1 cycle= 10 Sec]).  Same PHY WirelessMAN-CX sources are identified during CMI in Sub-channel 2; non-WirelessMAN-CX sources are identified during silent periods created in Sub-channel 1 (See 15.3.1.2.2 and 15.3.1.2.1).  The parameter sent  applies only  to: (a)   For  this  specific BSD and BSID, as forwarded by this  BS_CCID_RSP message ( in the case of same-PHY WirelessMAN-CX) or, (b) to the specific non-WirelessMAN-CX source identified in the DFS_LE_PWR_FRQ parameter. this value can be set by the home base station to make the SS less responsive to interference detection (such as highly sporadic and transient events).  This value is a threshold value determining when a BS_CCID_RSP needs to be sent by the interfered with SS. Only when this value has been exceeded will the BS_CCID_REQ message be sent.The threshold for the detection of an interference event is set by the BS_CCID_RSP message.

DFS_LE_PWR_FRQ: This parameter is used to identify the types of interfering non-WirelessMAN-CX devices and provide information that may be specific to those the particular devices. Up to 32 classes of devices are possible to identify. The parameter contains the mean RSSI of the radar signals or non-WirelessMAN-CX systems detected during the  (No+Io) measurement slots of the Tcxcc. radar signals may be detected at below Regulatory threshold values, and the RSSI level given will be the mean value. of such signals. Other radar Radar information such as PPS and Pulse duration will be given in the Device Detection Specific field Fields. If non- WirelessMAN-CX systems their signature will be given as number of detected interference events per N Tcxcc cycles. This parameter will be used to support specific interference detectors that may be mandated by for use in specific bands. Additional bit fields are provided in support of these requirements. The CCID is the identification number of this specific BS_CCID_RSP message and is used to tag this specific interference event. The number will be a rolling number from 1 to 65536.

RSSI report: The mean and standard deviation of the interfering signal reported by this message.  Measured only for interfering signals exceeding Interference RSSI threshold in BS_CCID_REQ message. See Section 8.2.2, 8.3.9,8.4.11 for details.

| Table 108aj—BS_CCID_REQRSP message format Syntax | Size | Notes |
| --- | --- | --- |
| BS_CCID_RSP_Message_Format( ) { |  |  |
| Management Message Type = 76 | 8 bits |  |
| DFS_LE_PWR_FRQ | 32 bits | Bits 0-3:Device Type Bits 4-15:Device detection specific Bits 16-23: 8 bit mean RSSI Bits 24-31: TBD Bit 0: Set to 1 for non-WirelessMAN-CX Bits1-5: Device Type Bits 6-15: Device detection specific Bits 15-31: CCID |
| INT_BSD_Frq | 16 bits | Bits 0-7 for WirelessMAN-CX detection: The frequency number of interference BSD detection events per N Tcxcc (Bits 8-15) CXCC repetition cycles. Bits 8-15 For non-WirelessMAN-CX detection: The number of interference events per Tcxcc cycles exceeding a threshold RSSI specific to the SS detector.). |
| BSID | 48 bits | Foreign Base Station ID; Set to 0 for Non-WirelessMAN-CX Interference |
| BS_RF_Sector_ID | 3216 bits | Bits 0-7 For azimuth of beam wrt True North, 2 degree steps Bits 8-15 for -3 dB azimuth Beamwidth, 2 degree steps - ID: bits 0..3 - gain (dBi): bits 4-7, 2dB resolution -azimuth: bits 8-15, 2 degree resolution --3dB aperture: bits 16-23, 2 degrees resolution - polarization: bits 24-25, with: 00 - vertical, 01- horizontal, 11: circular - beam-forming: bit 26, 1=yes - bits 27-31: reserved Set all to 0 for Non-WirelessMAN-CX Interference |
| BS EIRP | 8 bits | Nominal EIRP of interfering BS; Set to 0 for Non-WirelessMAN-CX Interference |
| CX_CMI_D(n) | 8 bits | Coexistence Messaging Interval ID In which WirelessMAN-CX interference detected. Set to 0 if interference event detected during Silent period of sub-channel 1. Set to 1-3 if detected during CX_CMI_D(1-3); Set to 4 if detected during CX_CMI_D(4). Otherwise =0 when detection (leading to this response) done in (No+Io) slot of CXCC |
| RSSI report | 16 bits | dBm for RSSI and dB for standard deviation |
| IP_BS Proxy_Address | Variable | (Proxy IP) Set to 0 for Non-WirelessMAN-CX Interference |
| } |  |  |

(5) The following changes are proposed to Ref 1. on page 20, Line 45  to Page 21, Line 35. The change is in response to improvements suggested in Session 49 and includes editorial corrections.

6.3.2.3.72 BS_CCID_REQ message

This message is sent to the SS initiating the BS_CCID_RSP message. It is sent by the BS and it is used to indicate whether the interference events identified in the BS_CCID_RSP (specifically identified by the CCID) have been resolved. For DFS and non-WirelessMAN-CX interference events, in addition to this message there likely will be other actions issued by the network management systems, which can entail moving to other channels. For WirelessMANCX systems the actions could include reducing EIRP at the interfering BS or assigning the interfered-with SS to a different sub-frame, etc. This message is also sent to adjust the RSSI threshold of interference detection at the SS, the number of CXCC repetition periods over which a interference measurement takes place, both to WirelessMAN-CX and other system interference. BS_CCID_REQ shall contain the following parameters: and to inhibit the sending of BS_CCID_RSP messages due to specifically identified interferers..

BS_CCID_REQ shall contain the following parameters:

BSID: The identity of the foreign BS noted in the BS_CCID_REQRSP. This can be null if the   message was originally due to DFS or non-WirelessMAN-CX sources.

RSP_FIELD: The response field indicates:

(1) Interference with foreign BS is/is not resolved

(2) Sub DFS threshold and non-WirelessMAN-CX users noted, no response at present

(3) TBD threshold/response adjustment variables (TBD).

(1) The specific BS_CCID_RSP that this message is in  reaction to, as indicated by the CCID  (bits 0-15). If this value is 0, this message is resetting threshold parameters.

(2) Status of the interference resolution (bits 16-17), which if equal to:

0= DL interference source identified  in the BS_CCID_RSP has been resolved/tolerated/pending  and SS must cease sending a BS_CCID_RSP until the given inhibition  period (applicable to the specific source) expires.

1= Stop sending all BS_CCID_RSP until given inhibition period expires.

2= Start sending BS_CCID_RSP, reset inhibition period.

3= Vacant.

- RSSI threshold. (bits 18-25) Interference measured higher than this mean RSSI level in the operational bandwidth of the receiver will be construed as an Interference Event and will trigger and RSSI measurement for the calculation of a mean level and standard deviation for the interfering signal. Levels are set between  -40 and -103 dBm in increments of 0.25 dB. See Sections 8.2.2, 8.3.9, 8.4.1.1 for details

- The number of CXCC repetition cycles (bits 26-33) for the SS measurement of interference before a BS_CCID_RSP message is to be sent, given that RSSI threshold is exceeded at least once.

- The length of the inhibition period during which the SS suppresses BS_CCID_RSP messages due to either a specifically identified interference source or to all interference in general.

Table 108ak—BS_CCID_RSPREQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| BS_CCID_REQ_Message_Format(){ |  |  |
| Management Message Type = 77 | 8 bits |  |
| BSID | 48 bits | Interfering BS station ID for which this response is sent to SS; set to zero if non- WirelessMAN-CX |
| RSP_Field | 1632 bits | Bits 0-15: CCID of BS_CCIC_RSP to which this message is reacting to. If 0, this message is for reset of threshold parameters. Bits 16-17: Resolution Status 0= Interference resolved/tolerated/pending 1= Stop sending all BS_CCID_RSP for given inhibition period. 2= Start sending BS_CCID_RSP messages, reset inhibition period. Bits 18-25: Threshold RSSI Setting. Bits 26-33: CXCC repetition cycles Bits 34-48: Inhibition Period The response field indicates: Bit 0: Response to Radar=1 Response to Non-WirelessMAN-CX =0 Bit 1-2: 0 - Resolved 1 - Pending Resolution 2 - Adjust threshold 3 - Inhibit Response Bit 3-9: Interference RSSI Power threshold Adjust [John to fix the table] Bit 10-15: TBD Threshold for number of interference events per CMI Cycle) |
| } |  |  |

(6)Ignore

(7) Ignore

(8) Ignore

##### (9)  Ignore

#### (10) Ignore

(11) Ignore.

(12) The following changes are proposed to Ref 1. on page 89, Lines 12-29. The changes are provided to give more detail as how this sub-channel supports same-PHY signaling for control of co-channel  interference

15.3.1.2.2 Sub-channel 2

Sub-channel 2 shall be used for  data transmission  of  interference identification messages and sub-frame claiming processes between systems using a same PHY profile. and their specific Master sub-frames. and used to transmit randomly chosen but periodic  A system claims by  continually occupying a specific master sub-frame in the  downlink and uplink by sending BSD and SSURF messages every sub channel 2 repetition period. Such transmissions(messages in CMI) should be undertaken only by  Only the system claiming a specific CXCC allocation, which transmit while all other systems will remain silent.

The DL CMI are identified as CX_CMI_D(n) for messages sent on the (n=1-3) Master sub-frames of sub-channel 2. Master sub-frames are claimed and continually used by the claiming WirelessMAN-CX system to indicate its operation and used of the channel and its associated sub-frames.

For n=4, the CMI is sent on the shared sub-frame, and only on an occasional basis by a system (this is used for identification of sporadic interference emanating from outside the coexistence community).

The complement of the downlink CMI are the uplink intervals, identified as CX_CMI_U(n).

BSD messages should be sent in the downlink CMI and SSURF messages are sent on the uplink.

The Shared sub-frames will only be used to transmit BSD and SSURF for purposes of  interferer identification .

The CXCC sub-channel 2, containing the CMI, uses the defined MAC Frames within the MAC Frame numbers 256-511.