| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to clause 15.1.3 |  |
| Date Submitted | 2007-09-07 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, HaBarzel St, 69710, Tel Aviv, Israel Shulan Feng Hisilicon Tech. Co., LTD Bld.17, No.8, Dongbeiwang West Road, Hai-Dian District, Beijing, P. R. China Wu Xuyong Huawei, Huawei Industry Base, Bantian, Longgang, Shenzhen, China 518129 | Voice: +972544225548 E-mail:mariana.goldhamer@alvarion.com Voice: +86-10-82829151 e-mail to : fengsl@hisilicon.com , fengsl@huawei.com Voice: +86-755-28976776 E-mail to : wuxuyong@huawei.com |
| Re: | Task Group Review |  |
| Abstract | Text consolidation |  |
| Purpose | Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes to clause 15.1.3

Mariana Goldhamer, Shulan Feng, Xuyong Wu

Alvarion Ltd, Hisilicon Tech. Co.Ltd, Huawei

# Summary of the relevant comments and proposed actions

## Comments from data-base IEEE 802.16h-07/14r3

### Comment 40 by David:

Referred text:

Similarly to the channel allocation, the IBS will then first try to find a vacant sub-frame in the potential

channels using the information of the distributed database; if it fails the IBS will then try to vacate an exclusive

existing sub-frame by sub-frame distribution optimization, if supported. If an exclusive existing subframe

is not available, the IBS will then try to negotiate with the systems inside the community to create a

new sub-frame. If all these attempts fail, the IBS will not be able to get any interference free resource for

operation.

Figure h 16 shows the initialization procedures for the WirelessMAN-CX BSs. Note, that the procedures

that the BS uses to create a Master slot or channel switching are also applicable for the operating stage. The

detailed negotiation and update procedures are described in section 15.5.1 and 15.4.1.2.

Comment:

### Creation of sub-frame is no more considered given the new CX Frame structure.

- Suggested Remedy

1) Remove following sentences:

- “If an exclusive existing subframe is not available, the IBS will then try to negotiate with the systems inside the community to create a new sub-frame.”

- “Note, that the procedures that the BS uses to create a Master slot or channel switching are also applicable for the operating stage.

2) Update accordingly the Figure h16   (fig. h23 in D2c)

### Comment 41 by Ken:

Referred text:

Otherwise (the country/region database is not available):

o The control channel CX_CMI_D/U(n) and the CSI method, to be described below, offer

time slots in which none of the members of the existing community transmits any signal.

A new BS uses CSI or CMI to broadcast the message containing the contact request and/or

the coexistence signal carrying the IP address

o A SS in the common coverage area, identifying the new BS signals, will forward the information

to its operating base station using REP-RSP message or BS_CCID_RSP message

o The operating BS updates its database and sends feedback information to the IBS, using

the IP network

o The new BS learns the IP identifier of the coexistence neighbor BS from the message sent

by the coexistence neighbor BS via the IP network

— Build the local image of the relevant information in the community BS’s, by copying the info from

those BSs

— Listen on multiple frequencies

o Identify the level of interference on each frequency channel;

— Decide the working frequency (ACS – Adaptive Channel Selection process);

o If no interference detected on some channels, select one randomly as working channel;

o If interference detected by the IBS or the OBS system on all the channels, then the IBS

should decide whether an optimized channel distribution, as described in section 15.4.1.1,

can allocate an exclusive channel for each BS, including the IBS, in the community.

o If every BS in the community can be allocated an exclusive channel without interfering

with the others, that means default interference-free Master slot is available for this initializing

BS.

— If available, select an interference-free Master sub-frame; if not, use the procedure for creating new

Master sub-frames;

— Search the Base Station data base for finding the BSs using the selected Master sub-frame;

— Request those Base Stations, by sending IP unicast messages, to listen during the BS_entry slot in

order to evaluate the interference from the new Base Station;

— Use the allocated slots for transmitting the “radio signature” at intended operating or maximum

allowed EIRP, whichever is less, and in all the used directions;

— Ask for permission of the Base Stations, using the sub-frame as Masters, to operate in parallel and

use the same sub-frames;

— If all of them acknowledge, the Base Station acquires a “temporary community entry” status; the

final status will be achieved after admission of the SSs;

— If no free Master slot sub-frame is found, use the procedure for creating new Master slot subframes.

Listening on multiple frequencies (line 33) and deciding the working frequency (line 37) should happen before the interaction on the control channel (lines 15-29)

- Suggested Remedy

Rearrange so the steps are in the correct order

### Comment 42 by Ken:

Referred text:

If available, select an interference-free Master sub-frame; if not, use the procedure for creating new

Master sub-frames;

Comment:

The portion of the bullet after the semicolon ";" is redundant with the last bullet at the top of page 64.  The one at the top of page 64 is in a better place in the order of events.

- Suggested Remedy

Delete "if not, use the procedure for creating new Master sub-frames;"

# Proposed general approach

There is lot of redundant text, which appears at the beginning of each clause. A better document will result if every mechanism will have its own clause and all the related ones will be just mentioned, instead to be described again. Here down is the proposed rationalization, providing instructions to 802.16h Editor.

- 15.1.3 Procedure flow

  - To be reduced to the description of how it works, not of how to be implemented; to use references the procedures described in 15.3 (Community entry) and 15.4 (interference avoidance during operation)

# Proposed reorganization and text changes

| Existing clause | Title | Text |  |
| --- | --- | --- | --- |
| 15.1.3 | Procedure flow | This chapter is highly redundant and some times not consistent with text in 15.3 and 15.4 Instead of a long description is better to refer procedures in 15.3 and 15.4 For SSU refer procedures in clause 6; the situation is difficult because the existing clause numbering in 6.3.15 is changed in 16h and a mix of old and new clauses appear in 6.4 Make the changes proposed below | P65/r35 … p72/r22 See below |

# Proposed text changes to 15.1.3

- Procedure in WirelessMAN-CX

- Procedure for BS

In general, coexistence detection, avoidance and resolution are performed in two stages, initialization stage and operating stage. In both stages, in bands where SSU are presents, need to be followed the procedures described in clause 6.3.15  in both initialization and operating stages.

(1) Initialization stage

The Base Station should follow the procedures described in 15.3.2 in order to assess the potential operating channel. However, only passive scanning may not be able to avoid the hidden neighbor system problem, i.e. the BS that cannot be heard directly by the initializing BS but may have harmful interference with the initializing systems SSs. Thus, with the knowledge of coexistence neighbor topology, available, for example, from a central country/region database, the Initializing Base Station (IBS) may be able to take the potentially hidden neighbor systems into account, therefore, avoid the possible interference from those neighbors.

Alternatively, in this stage, in addition of the passive scanning approach of the CCA/CMFA, the initializing BS may use the active scan approach within CXCC to broadcast its BSID and relative information to its potential neighbor using its maximum capable and allowed operating EIRP. In this way, potential victim SSswill measure the interference and get the information within the singalingsignaling/messaging in CXCC and forward to their serving BSs. After the neighbor BSs get the address via the SSs' reports, they will contact their new coming neighbor via the IP network and update the database on both sides. Therefore, using the information that the IBS has received from its neighbors, the IBS can get the information of the coordinated systems in a potential community.

In conclusion, a BS should use all the available information in order to choose a first operating channel and Master sub-frame.

If the IBS finds that there is no available channel for operation neither to be interference victim nor source, the information in the distributed database can be used to figure out the scheme of the channel allocation optimization in its neighborhood. If the channel optimization feature is supported within the neighborhood, the IBS should find out whether a free channel can be vacant vacated by channel reallocation within the community. If it is found a suitable the channel reallocation scheme is figured out, which will provide every system  within the neighborhood with exclusive channels, the IBS  should contact the BSs in the neighborhood, which need to reallocate the channels (the candidate BS's) and negotiate with the relative neighbor systems for the reallocation. After receiving the confirmation from all candidate BSs, the IBS should send a CXP message acknowledging the success and the final channel allocation. The BS in the neighborhood should then operate on the new set of channels. allocationOtherwise, if the IBS can't succeed after the effort of reallocation, the IBS should try to share a channel with  proper neighbors. The automatic channel reallocation procedures aim to replace the operator coordination, which can be a problematic process, with an automatic equipment procedure based on interference measurement.

Similarly to the channel allocation, the IBS will then first try to find a vacant master sub-frame allocation in the potential channels using the information of the distributed database; if it fails, the IBS will then try to vacate an existing master sub-frame allocation by master sub-frame allocation optimization, if supported. If all these attempts fail, the IBS will not be able to get any interference free resource for operation.

Error: Reference source not found shows the initialization procedures for the WirelessMAN-CX BSs. The detailed negotiation and updatine procedures are described in section Error: Reference source not found and Error: Reference source not found. Scan here st an action taken on the each candidate channel sequentially. The detail of different type of scanning is described in 15.3.2. ([the following statement moved into section addressing CCA/CMFA at the end of its first paragraph.]The basic distinction between passive scan and active scan is in the specific action of scanning. Passive scan means measurement or monitoring of the potential channels, without any transmission. Active scan leads to signal transmission on each potential channel, and waiting for the responses.)

- brief procedures of BS initialization and operation for coexistence— BS

(2) Operating stage

In the operating stage, the BS has SSs associated with it.

BS and its serving SSs should monitor the channel status on the working channel and the non-working channels and update its interference status in the information table (15.5.5.1). If interference status varies, system shall update its resource allocation to get interference free wireless resource and at the same time, use the free bandwidth efficiently once more bandwidth available. The changes of the interference status shall be triggered when  a primary user is detected, when a new free channel is found, or when the number of interference neighbor system varies.

If a new free channel is detected and the working channel of system is too crowded, a system may switch to the new free channel. A system may request its coexistence neighbors to delete it from their coexistence neighbor list using Delete Coexistence Neighbor Request message (15.5.1.7) via backbone or air. And the coexistence neighbor may occupy the released resource to improve the bandwidth efficiency.

If a system has no interference neighbor and finds a new free channel, it just records the new free channel as its alternative channel in the information table. The alternative channel may be used for the optimization of channel distribution or fast channel switching when a primary user is detected on working channel.

If a primary user is detected on current working channel and a system is not allowed to share the same channel with this user, the system shall stop working on that channel at the required switching time. At the same time, a system should try to find free working bandwidth for itself, as a free channel and/or a free master sub-frame, following the procedures descriebed for the initialization stage.

The reasons for changes in the interference status are the addition of a new uncoordinated interference system or the release of an interference neighbor system.. If a new interference BS or SS is identified, a system should negotiate with the corresponding interference neighbor to get interference free. For example, in the case of an interfering SS, should be allocated resources for the SS operation during its serving BS’s master subframe. If an interference relationship is released, the system should use the released resource effectively to improve frequency efficiency. As example, the SS which has released the interference relation may operate during its serving BS’s slave subframe. Or a BS may borrow the subframe released by the interference neighbor to another system.

- Procedure flow for SS

The procedure of system and community entry for a new SS in WirelessMAN-CX is shown below:

- The new SS starts the network entry procedures using the UL Master sub-frame

- The new SS listens to the Coexistence Control Channel (CXCC) slots to detect interference sources (see Error: Reference source not found)

- At BS request, the new SS reports to the BS the results of the CXCC measurements

- At BS command, the new SS transmits CMI at the appropriate slots in the CXCC

- If an operating Base Station (OBS) perceives interference from the new SSs, it will ask the new Base Station to find another sub-frame for that SS operation via the CXP;

- Channel Measurement in the Operating Stage

BS may request SS to measure one or more channels on its behalf in the operating stage. BS should schedule available measurement interval for SS via periodic_channel_measurement_IE (8.4.5.3.5). During scheduled measurement interval, BS shall not transmit MAC PDUs to that SS or request any uplink transmission from SSs. BS should schedule measurement interval properly so that no effect on normal traffic transmission between BS and SS.

Upon receiving a measurement requirement, SS shall start to measure the indicated channel during the scheduled measurement intervals. SS shall continue to measure the indicated channel during the scheduled measurement intervals until the measurement interval ends or serving BS schedules SS to receive and/or send signal during measurement interval.

BS may schedule one or more measurement pattern for SS. Measurement interval of different measurement pattern shall not be overlapped. Measurement patterns are identified by measurement_request_index parameter in periodic_channel_measurement_IE. SS should report measurement result corresponding to each measurement request from BS.