| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to clause 15.3 |  |
| Date Submitted | 2007-09-07 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, HaBarzel St, 69710, Tel Aviv, Israel Harry Bims Apple, Inc. 1 Infinite Loop, Cupertino, CA 95014 | Voice: +972544225548 E-mail:mariana.goldhamer@alvarion.com Voice: +1-650-283-4174 Email: harrybims@att.net |
| Re: | Task Group Review |  |
| Abstract | Summary of the Ad-Hoc activity |  |
| Purpose | Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes to clause 15.3

Mariana Goldhamer, Harry Bims.

Alvarion Ltd, Apple

# Proposed general approach

There is lot of redundant text, which appears at the beginning of each clause. A better document will result if every mechanism will have its own clause and all the related ones will be just mentioned, instead to be described again. Here down is the proposed rationalization, providing instructions to 802.16h Editor.

- 15.3 Interference assessment and basic connectivity creation

  - Introduction text

  - 15.3.1 CXCC Description

  - 15.3.2  Candidate Channel and Candidate Master Frame assessment

    - 1) Introduction

- a) Passive scan (for IBS in UL assessment as potential interference victim)

- b) Active scan (for IBS in DL assessment as potential interference source)

    - 2) Passive scan [former]

a) RSSI detection in channel and master sub-frames [m?]

b) CXCC Sub-channel 4 [o?]

c) CXCC Sub-channel 2 SSUF [o?]

    - 3) Active scan [after]

a) CXCC CSI sub-channel [o?]

b) CXCC Sub-channel 2 BSD [o?]

    - 4) All the results come above, which have been supported in IBS will be collected into distributed information database and for further interference resolution in 15.4.

    - use as basis the John’s text from 15.3.3.2 (it is now in a place related to messaging, however this is the best available text on channel selection)

  - 15.3.3   Coexistence Messaging Mechanism

    - Inter-system communication

    - Interferer Identification using messaging

    - BSD Description

    - SSURF Description

  - 15.3.4 Coexistence Signaling Mechanism

    - Energy keying in time-domain

    - Inter-system communication during CSI

  - 15.3.5  Mechanisms based on energy keying in frequency domain

    - 15.3.5.1 Signal definition

    - 15.3.5.2 Synchronization using CXCC

    - 15.3.5.3 Inter-system communication

    - 15.3.5.4 Special procedures for backhaul-less systems

# Proposed reorganization and text changes

| Existing clause | Title | Text |  |
| --- | --- | --- | --- |
| 15.3 | Interference assessment and basic connectivity creation | Needs re-organization Update the introductory text of 15.3 | updated text will come in later revision |
| 15.3.1 | Change title to “Coexistence Control Channel” | 15.3.1.2.3.2 (NUR-BS using freq keying) goes to 15.3.5.3 | P89/r49 to p90/r6 to 15.3.5.3 |
|  |  | 15.3.1.2.3.3 (signaling to backhaul-less) goes to 15.3.5.4 | P90/r6 to p91/r9 to 15.3.5.4 |
|  |  | 15.3.1.3 (signaling using fre-keyed) goes to 15.3.5.1 and 15.3.5.2 | P91/r52- to p92r44 goes to 15.3.5.1 (Signal Definition) P92/r44 to p93r34 goes to 15.3.5.2 (Synchronization using CXCC) |
| 15.3.2 | Change title to “Coexistence Signaling Mechanism” | Goes to new clause 15.3.4 Needs up-date | P93/r37 to p102/r 8 |
| 15.3.3 | Coexistence messaging mechanism | 15.3.3.2 (candidate channel determination) goes to new 15.3.2; delete “using GPS…” in the title 15.3.3.3 becomes 15.3.3.2; add at the beginning of the clause a sentence saying that the channel selection is described in 15.3.3 | p103/r3 to p104/r44 which goes to new clause 15.3.2 |
|  |  | Create 15.3.2 Candidate Channel and Candidate Master Frame assessment; use the text below |  |
|  |  | Create 15.3.4 Coexistence Signaling Mechanism - Energy keying in time-domain - Inter-system communication during CSI |  |
|  |  | Create 15.3.5 Mechanisms based on energy keying in frequency domain - 15.3.5.1 Signal definition - 15.3.5.2 Synchronization using CXCC - 15.3.5.3 Inter-system communication - 15.3.5.4 Special procedures for backhaul-less systems |  |

# Proposed text changes to 15.3.2 based on John’s text from 15.3.3.2

(change all the references to 15.3.3.2 to the new clause 15.3.2)

### 15.3.2 Candidate Channel and Candidate Master Frame assessment(CCA and CMFA)

BASIC STRUCTURE 15.3.2（using the idea in Mariana’s first relative mail）:

YELLOW highlighted is notes for the reader only, not proposed text.

- Introduction

  - Passive scan (for IBS in UL assessment as potential interference victim)

  - Active scan (for IBS in DL assessment as potential interference source)

- Passive scan [former]

  - RSSI detection in channel and master sub-frames [m?]

  - CXCC Sub-channel 4 [o?]

  - CXCC Sub-channel 2 SSUF [o?]

- Active scan [after]

  - CXCC CSI sub-channel [o?]

  - CXCC Sub-channel 2 BSD [o?]

- All the results come above, which have been supported in IBS will be collected into distributed information database and for further interference resolution in 15.4.

[SUMMARIZING]

Candidate Channel Assessment (CCA) is a process used by WirelessMAN-CX systems in a given frequency range, to evaluate the radio environment within each candidate frequency channel, and  sub-frames within the CX-Frame. The BS monitors the candidate channels and sub-frames within that band, to find a frequency channel and Master sub-frames that provide the most capacity. This process is used, for example, by an IBS prior to forming a Coexistence Community.

The CCA/CMFA process utilizes the passive scanning process to determine if it is a potential interference victim, and utilizes the active scanning process to determine if it is a potential interference source. Passive scanning involves monitoring of uplink transmission intervals and measurement of radio activity by measuring the potential interference signal power [I] and thermal noise power [N]. Active scanning involves broadcasting a signal that advertises the presence of the initializing BS, so that a potential interference victim system that supports this feature can measure the interference created and potentially contact the initializing BS neighbor for further collaboration.

CXCC sub-channel 4 is a method that a potential interference victim utilizes to measure the level of interference present during each potential Master sub-frame by passive scanning.  Alternatively, SSURF provides for a method of passive scanning by monitoring messages in UL sub-frames within CXCC sub-channel 2. While the CXCC CSI sub-channel provides a method of active scanning using signal broadcasting and the CXCC sub-channel 2 provides a method of active scanning by message broadcasting. Also, it is possible for the BSIS to be used to calculate the expected interference in the BS area.

[PASSIVE SCANNING PART1]

In passive scanning, each candidate channel and Master sub-frame should be ranked in terms of  interference-to-noise ratio (INR). Those channels and Master sub-frames with the lowest INR ratio or lower than -6dB will be marked as available by the base station.  They will therefore become candidates for entry by an IBS, since such channels and Master sub-frames have the lowest amount of discernable activity on them, which implies that they are likely to have lower interference.

[ACTIVE SCANNING]

In active scanning, the BS should broadcast its identification and relative information using signaling/messaging within each available channel. All of the SSs in potential interference victim systems can measure the impact of interference from the initializing BS from their specific CXCC allocation and store the ID and relative information of the initializing BS (see [15.3.2.5] & [6.3.2.3.62]). By receiving a report from the victim SSs, the OBS in the victim can have an entire view of the interference environment of each SS within its system and contact the interference source IBS for further interference resolution (15.4).

Channels should be further ranked according to the number of available Master sub-frames. A higher number of available Master sub-frames indicate a less crowded channel.

[GENERAL]

Interference [I] and noise [N] should be determined using the RSSI measurement capability of the receiver BS as detailed in Section 8.4.11.2. After synchronization to a universal or regional timing standard and initialization of the base station's operating parameters, the BS should select a channel and undertake noise floor and interference measurements during the CXCC suitable allocations ( see 15.3.1.2) that  may also be used by non-WirelessMAN-CX system.

The noise floor [N] is the noise power spectral density of the received channel (No) multiplied by the channel bandwidth. Measurements not deemed as Gaussian and/or RSSI measurements that result in a combined noise [N] and interference [I] floor higher than 1dB above [N] alone ((N+I)/N>1 dB) will be an indication that channel may be occupied by non-WirelessMAN-CX users. In this instance the value of the mean interfering RSSI will be taken as the [N+I] created by the occupying non-WirelessMAN-CX interference source system and the given channel will be discarded from further consideration unless the interference source system support a compatible collaborative mechanism with the WirelessMAX-CX system.(The discarded channel's noise plus interference level will be stored in the BS interference table). Otherwise, the measurement will provide a value for [N].

[note:not a standard issue.]

[PASSIVE SCANNING PART2]

For uplink interference assessment using messaging, interference [I] measurements for the potential uplink interference victim on channels occupied by WirelessMAN-CX systems may be undertaken by calculating the mean signal strength and variance due to uplink SSURF messages summed over the uplink sub-frames in CXCC sub-channel 2 . Measurement of the RSSI will be done in accordance with Section 8.4.11.2. The mean RSSI and variance calculated for the summed SSURF occurrences over the  uplink sub-frames in CXCC sub-channel 2 of the channel are construed as interference values [I] and interference level variation [Var I] for the channel. In essence, what this measurement represents is the total interference power that the Base Station measures on a given channel due to the total of all Subscriber Stations operating on that channel. In the case of the OFDM PHY, because of the granularity of the measurements within the CXCC sub-channel 2 (each interference source SS may be separately detected), so that the BS can obtain a more sophisticated understanding of the interference environment beyond what is simply given by [I] and [Var I]. The incorporation of advanced interference detection approaches will not be considered in the context of the current discussion.

Then, for uplink usage, the channels and sub-frames can be further ranked, with the channel having the lowest I/N and smallest [Var I] [question: result of these 2 criteria are not comply with each other, what we can do if mean of I/N are lowest but Var I are higher than other channel??] measurements over a maximum number of Master sub-frames to be likely selected for IBS entry into a Coexistence Community[question: the later portion of sentence add a 3rd criteria for the channel selection. Notes: What will be used to channel selection is belong to 15.4, while 15.3 should only provide different rank result in different dimensions. These 3 output<mean I/N; Var I; numbers of available master allocation>will be separately provided as the uplink assessment result, and together with the downlink assessment result using CSI/BSD/RS, the interference prevention(15.4) will provide resolution.]. This process is undertaken for each channel that is specified for the band of operation for the WirelessMAN-CX system and in essence identifies availability for the uplink sub-frames usage within CX-Frame. . All the passive scan results(in RSSI monitoring and uplink CXCC sub-channel 2 SSURF messaging) and active scan results(in CXCC CSI sub-channel and CXCC sub-channel 2 DL subframe BSD messaging), which supported by current system, should be provided to the BS in this system for further interference resolution.(15.4)

Candidate Channel Determination Process (needs changes) [notes: passive scan for available channels and sub-frames in CX-Frame, then active scan within these available channels for confirming its really usable. These should be addressed in CCD.]shows the CCD process.

- Candidate Channel Determination Process

[Notes: The figures below are propose to replace the figures in 15.1.3.1, and the figure above (figure h48) are propose either to be deleted and insert a reference here, or to be updated accordingly as a extended level of detail.]

[FYI, the CCA/CMFA procedure described in this section is highlighted in the following figure.]