| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to clause 15.4 |  |
| Date Submitted | 2007-09-07 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. Xuyong Wu Huawei Shulan Feng Huawei | Voice: +972544225548 E-mail:mariana.goldhamer@alvarion.com |
| Re: | Task Group Review |  |
| Abstract | Summary of the Ad-Hoc activity |  |
| Purpose | Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes to clause 15.4

Mariana Goldhamer, Xuyong Wu, Shulan Feng

Alvarion Ltd, Huawei

# Proposed general approach

Here down is the proposed rationalization, providing instructions to 802.16h Editor.

  - Interference prevention

    - Coexistence Frame

    - Coordinated Contention-based protocol

    - Working channel allocation optimization

    - Master Frame allocation optimization

    - Interference control in sub-frames

    - Credit token-based coexistence protocol (CT-CXP).

# Proposed reorganization and text changes

| Existing clause | Title | Text |  |
| --- | --- | --- | --- |
| 15.4 | Interference prevention | Most of the information in the introduction is redundant with 15.1.3; delete the redundant text | Delete p110/r14 to p111/r8 |
| 15.4.1 | Adaptive channel selection | Move to 15.4.3 and change title: Working channel allocation optimization | P111/r33 to p115/r39 |
| 15.4.2 | Adaptive sub-frame allocation | Delete the title | P115/r45 |
| 15.4.2.1 | Sub-frame sharing and scheduling | Delete the title | P115/r48 |
|  |  | Insert title: 15.4.1 “Coexistence Frame” |  |
| 15.4.2.1.1 | Introduction | Move to 15.4.1.1 | P115/r50 to p116/r18 |
| 15.4.2.1.2 | Scheduling of interference free… | Move to 15.4.1.2 Add the text on CXZ from p.127 | P116/r23 to p120/r45 P127/r24 to p127/r59 |
| 15.4.2.1.3 | Scheduling of restricted … | Move to 15.4.1.3; delete the text on BS information data-base | Move P120/r48 to p126/r46 Delete p126/r48 to p127/r23 |
| 15.4.2.2 | Sub-frame distribution opt | Move to 15.4.4 and change title: Master Frame allocation optimization | P127/r60 to p131 r 3 |
| 15.4.2.3 | Interference control in sub-frames | Use contribution C802.16h/07-89r2 | P131/r8 to 134/r34 |
| 15.4.2.4 | CT-CXP | Move all the existing text to the new 15.4.6 | P134/r36 to p143/r35 |

In addition:

- Comment 50 resolution to be inserted in 15.4.1 before the fig. h53

To enable SSs which do not support CX-Frame usage the resource allocation should be scheduled within each CX-frame in WirelessMAN-CX systems. The Preambles and MAPs should only be transmitted in the DL master sub-frames. The MAPs in master sub-frames shall be used to indicate the resource allocation of this system in a CX-Frame, possibly including the resource schedule in the master sub-frame, shared sub-frame, slave sub-frame, borrowed sub-frame, CXCBP sub-frame and optional common sub-frame. SSs with harmful interference within their neighborhood shall be scheduled to use master subframes. SSs without harmful interference may be scheduled to use any of sub-frames within the CX-Frame.

The frame duration IE in DL-MAP message should be assigned to the duration of the CX-Frame.