| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC header format for radio signature |  |
| Date Submitted | 2008-01-15 |  |
| Source(s) | Shulan Feng Hisilicon Tech. Co., LTD Bld.17, No.8, Dongbeiwang West Road, Hai-Dian District, Beijing, P. R. China | Voice: +86-10-82829151 Fax: +86-10-82829075 e-mail to : fengsl@hisilicon.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #29 |  |
| Abstract | This contribution proposes a MAC header format for radio signature. |  |
| Purpose | Discussion and accept. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Header Format for Radio Signature

Shulan Feng

HiSilicon

# Introduction

Radio signatures are used for inter-system interference identification. Downlink radio signature may carry a MAC message including BSID information and uplink radio signature may carrier a MAC message including SSID.

If we use the traditional MAC PDU format, to carry BSID information in radio signature, at least 136 bits is needed as shown in following table.

GMH

(48bits)

Management message type

(48bits)

BSID

(48bits)

CRC

(32bits)

Table 1

But many IEs in GMH are useless for inter-system communication. For example, CID IEs in GMH is for intra-system communication, it is useless for inter-system communication.

The bandwidth for inter-system communication is very limited. It is necessary to redesign the MAC header for inter-system communication to improve the bandwidth efficiency.

If no MAC PDU appended, then the following table may be suitable for radio signature. ID field is the transmitter’s ID. For downlink, it is the MAC address of BS. For uplink, it is the MAC address of SS. HCS is CRC-8.

ID

(48bits)

HCS

(8bits)

Table 2

If MAC PDU appended, the table 3 may be more suitable. ID field is the transmitter’s ID. For downlink, it is the MAC address of BS. For uplink, it is the MAC address of SS.

CRC

(Optional)

(32bits)

Length

(8bits)

HCS

(8bits)

MAC PDU(Optional)

(variable)

Inter-system communication

Management Message Type

(8bits)

Inter-system communication

Management Message Payload

(variable)

ID

(48bits)

Table 3

If used for radio signature only, then table 2 is better. If used for both radio signature and SSURF, then table 3 is better.

Or considering radio signature and SSURF are transmitted in specific zone and can be identified by the receiving slot, we may let MAC message in radio signature following the table 2 and SSURF message following the table 3.

Considering we may add more inter-system communication message in the future, I suggest take the table 3 as the inter-system communication management message format.

So for the downlink radio signatures, they may carry the MAC massage as table 4. Since the value of length includes the length of header itself, the length of MAC PDU if MAC PDU is appended and the length of CRC if CRC is appended. So we may set the length value to 8 which indicates no MAC PDU and CRC.

Length

[value:8]

(8bits)

HCS

(8bits)

BSID

(48bits)

Table 4 MAC message format carried by downlink radio signature

For the uplink radio signatures, they may carry the MAC massage as table 5.

CRC

(32bits)

Length

[value:19]

(8bits)

HCS

(8bits)

Type

(8bits)

BSID

(48bits)

SSID

(48bits)

Table 5 MAC message format carried by uplink radio signature

# Proposed Text

[tbd]

Reference

[1] C802.16h-07/106, Action Items from Session #52

[2] 80216h-07/053r2, Comment database on 16h draft D3