| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | WirelessMAN Coexistence System Profiles |  |
| Date Submitted | 2008-01-07 |  |
| Source(s) | Wu Xuyong Huawei, Huawei Industry Base, Bantian, Longgang, Shenzhen, China 518129 Paul Piggin NextWave Broadband Inc. 12670 High Bluff Drive San Diego CA 92130 USA Mariana Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv, Israel | Voice: +86-755-28976776 Fax: +86-755-28972352 wuxuyong@huawei.com， Voice: 1 858 480 3100 Fax: 1 858 480 3105 ppiggin @ nextwave.com mariana.goldhamer@alvarion.com |
| Re: | IEEE 802.16-07/050: IEEE 802.16 Working Group Letter Ballot #29: Announcement (2007-10-05) |  |
| Abstract | Proposal to the Adhoc for Coexistence Profile |  |
| Purpose | To consolidate the 16h draft. |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

WirelessMAN Coexistence System Profiles

Wu Xuyong, Paul Piggin

Huawei, Nextwave

Overview

In letter ballot #29, there is several comment addressing the concern of lack of profile for coexistence enhancement functionality, the group agree that we need profiles to address this issue, and make coexistence profile for the industry usage depending on different usage scenario.

The following comment is taken from database IEEE 80216-07_053r2 [1], which show the concern from IEEE802.16 working group on this issue.

Comment #092 by Mariana Goldhammer for chapter 12:

Comment:

No System Profiles

Suggest Remedy:

Make a poll of the basic features to be supported and get the feeling of the group; start with the first draft based on this.

Reply comment:

Wu Xuyong: agree

David Grandblaise: Accepted

Group Resolution:

Ad-hoc chaired by Mariana to establish the Mandatory profile

Comment #093 by Jianmin Lu for chapter 12:

Comment:

Do we have system profile specified for 16h system?

Suggest Remedy:

Provide system profiles for 16h systems in chapter 12..

Reply comment:

Wu Xuyong: superseded by 092

David Grandblaise: Accepted

Group Resolution: Superseded

By comment 092

Comment #144 by Mariana Goldhammer for chapter 15.2.2:

Comment:

Fix: Inter-system communication may be initiated using messages over the air, as specified in this standard.

Suggest Remedy:

Inter-system communication may be initiated using messages over the air, as specified in this standard.

Inter-system communication is effective between systems using the same PHY profile.

Reply comment:

David Grandblaise: Accepted

Group Resolution: Accept Modified

Inter-system communication may be initiated using messages over the air, as specified in this standard.

Inter-system communication over the air is achieved effective between systems using the same PHY profile.

Move the profiles in 15.2.2 in the Profile Section (clause 12)

Discussion

Chapter 12 in IEEE802.16 defines system profiles that list sets of features to be used in typical implementation cases. The introduction paragraph of this chapter is shown below:

12. System profiles

This clause defines system profiles that list sets of features to be used in typical implementation cases. Each profile is assigned an identifier for use in such documents as PICS proforma statements. Features specified in the standard as optional may be listed in a profile as “required” or “conditionally required.” Profiles do not change “mandatory” status if specified in the standard itself. Any feature that is specified in the standard as optional and does not appear in certain profile is optional for the profile; thus absence of this feature in specific implementation does not affect conformance to the profile. Optional features shall be implemented as specified in the standard.

According to current status, this section has introduced 4 categories of profiles:

- MAC profiles (profM)

- PHY profiles (profP)

- RF profile (profR)

- Power Class profile (profC)

Each category addresses one domain of system specification. The integration of them will be used to specify the features list for typical implementation case.

However, on 16h point of view, the coexistence features is not covered by these category, and can compound with other category of system profiles, so we should add another profile domain to provide different implementation cases for coexistence. 16h may be not the only user of this domain; any future amendment may use this category to add coexistence system profiles.

Since this category includes multiple and mixed technology use cases, so it can not be appended into any existing sub-clause of section 12. So we propose to add a separate sub-clause addressing coexistence system profiles, which add: 5) WirelessMAN-UCP and WirelessMAN-CX system profiles.

Following the primary standard IEEE802.16-2004 style, the Yes/Conditional/No in the following table indicate mandatory/conditional mandatory/optional in the dedicate coexistence profile component.

Reference:

[1]  IEEE 80216-07_053r2：Letter Ballot #29 Comment Database (2007-11-15 )|

[2]  IEEE P802.16h/D3: 802.16h draft 3(2007-10-01)

[3]  IEEE 802.16-07/050: IEEE 802.16 Working Group Letter Ballot #29: Announcement (2007-10-05)

[4]  IEEE C80216h-07_106： Action Items from Session#52 (2007-11-15)

[5]  IEEE 802.16-2004: IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for

Fixed Broadband Wireless Access Systems (2004-10-01)

[6]  IEEE 802.16e-2005: IEEE Standard for Local and metropolitan area networks Part 16: Air Interface

for Fixed and Mobile Broadband Wireless Access Systems Amendment 2: Physical and Medium Access

Control Layers for Combined Fixed and Mobile Operation in Licensed Bands and Corrigendum 1

(2006-02-28)

Proposed Changes accordingly:

## Add a new sub-clause in section 12.

## 12.5 WirelessMAN-UCP and WirelessMAN-CX system profiles

This subclause defines typical system profiles for WirelessMAN-UCP and WirelessMAN-CX systems which implement coexistence enhancement in license exempt operation.

A system profile of WirelessMAN-UCP or WirelessMAN-CX system consists of six components: a MAC profile, a PHY profile, a RF profile, an optional coexistence profile, a duplexing selection, and a power class.

The coexistence profile is a special profile domain addressing coexistence enhancement features. The following sections define typical profile components in coexistence domain, which provide feature sets for variation of usage scenario.

All these profile components, specify additional feature sets for coexistence enhancement, which shall or may be implemented additionally once was selected as part of the target system profile. Table xxx1(609a?) show a list of coexistence profiles:

Table xxx1－ Profile definitions

| Identifier | Description |
| --- | --- |
| profX_UC | Uncoordinated Coexistence profile (12.5.1.1) |
| ProfX_CC1 | Coordinated Coexistence profile1 (12.5.2.1) |
| profX_CC2 | Coordinated Coexistence profile2 (12.5.2.2) |
| profX_CC3 | Coordinated Coexistence profile3 (12.5.2.3) |
| profX_CC4 | Coordinated Coexistence profile4 (12.5.2.4) |
| profX_CC5 | Coordinated Coexistence profile5 (12.5.2.5) |

Since the features dedicate to coexistence enhancement is not specified to all 802.16 system, but for different coexistence case requirement, the mandatory/conditional required feature sets are specified respectively based on cases below.

12.5.1 Coexistence profiles for WirelessMAN-UCP system

This subclause defines profiles for coexistence features in WirelessMAN-UCP systems.

12.5.1.1 ProfX_UC: Uncoordinated coexistence profile features

This profile specifies a set of capability requirements when a system is operating in the Uncoordinated coexistence mode. Table xxx2 lists the optional features and designates whether they shall or may be implemented to comply with this profile.

Table xxx2 Feature requirements profX_UC

| Feature | Reference | Required? | Conditions/Notes |
| --- | --- | --- | --- |
| Coexistence with SSU | (6.4.2.2) | Conditional | Local regulatory |
| DCS | (6.4.2.3.2) | Yes | － |
| Frame Selection | (6.4.2.3.4) | Yes | － |
| LBT with DMA | (6.4.2.3.7) | Yes | － |
| Synchronization to Neighbor system |  | Conditional | TDD |

12.5.2 Coexistence profiles for WirelessMAN-CX system

This subclause defines profiles for WirelessMAN-CX systems.

12.5.2.1 ProfX_CC1: Coordinated coexistence profile 1 features

Table xxx4 Feature requirements profX_CC1

| Feature | Reference | Required | Comment |
| --- | --- | --- | --- |
| CX with SSU | 6.4.4.2 | Y | To be requested only in frequency bands in which SSU have a defined regulatory status |
| DCS (passive channel selection) | 6.4.2.3.2 | Y |  |
| UCP | 6.4.2.3.7 | N |  |
| Provisioning | 15.2 | Y | GPS Sync |
| CXCC Timing | 15.3.1 | Y |  |
| CXCC Sub-channel 1 | 15.3.1 | Y |  |
| CXCC Sub-channel 2 | 15.3.1 15.3.3 | N |  |
| CXCC Sub-channel 3 | 15.3.1 | N |  |
| CXCC Sub-channel 4 | 15.3.1 | Y |  |
| CXCC CSI Sub-channel | 15.3.1 15.3.4 | N |  |
| Candidate Channel and Master Frame Assessment (CCMFA) | 15.3.2 | N |  |
| CX-Frame | 15.4.1 | Y |  |
| CX-CBP | 15.4.1.4 | Y |  |
| Channel optimization | 15.4.2 | N |  |
| Master frame optimization | 15.4.3 | N |  |
| Interference control | 15.4.4 | N |  |
| Token Protocol | 15.4.5 | N |  |
| CXP | 15.5 | N |  |
| Inter-system communication | 15.6 | Y |  |
| Network-based identification of SSU | 15.8 | N |  |

12.5.2.2 ProfX_CC2: Coordinated coexistence profile 2 features

Table xxx6 Feature requirements profX_CC2

| Feature | Reference | Required | Comment |
| --- | --- | --- | --- |
| CX with SSU | 6.4.4.2 | Y | To be requested only in frequency bands in which SSU have a defined regulatory status |
| DCS (passive channel selection) | 6.4.2.3.2 | Y |  |
| UCP | 6.4.2.3.7 | N |  |
| Provisioning | 15.2 | Y | GPS Sync |
| CXCC Timing | 15.3.1 | Y |  |
| CXCC Sub-channel 1 | 15.3.1 | Y |  |
| CXCC Sub-channel 2 | 15.3.1 15.3.3 | N |  |
| CXCC Sub-channel 3 | 15.3.1 | Y |  |
| CXCC Sub-channel 4 | 15.3.1 | Y |  |
| CXCC CSI Sub-channel | 15.3.1 15.3.4 | N |  |
| Candidate Channel and Master Frame Assessment (CCMFA) | 15.3.2 | Y |  |
| CX-Frame | 15.4.1 | Y |  |
| CX-CBP | 15.4.1.4 | Y |  |
| Channel optimization | 15.4.2 | Y |  |
| Master frame optimization | 15.4.3 | Y |  |
| Interference control | 15.4.4 | Y |  |
| Token Protocol | 15.4.5 | Y |  |
| CXP | 15.5 | Y |  |
| Inter-system communication | 15.6 | Y |  |
| Network-based identification of SSU | 15.8 | N |  |

12.5.2.3 ProfX_CC3: Coordinated coexistence profile 3 features

Table xxx8 Feature requirements profX_CC3

| Feature | Reference | Required? | Conditions/Notes |
| --- | --- | --- | --- |
| Coexistence with SSU | 6.4.4.2 | Conditional | Local regulatory |
| CX-Frame | 15.4.1 | Yes | － |
| CXCC Sub-channel 1 | 15.3.1 | No | － |
| CXCC sub-channel 2 | 15.3.1 15.3.3 | No | － |
| CXCC sub-channel 3 | 15.3.1 | No | － |
| CXCC sub-channel 4 | 15.3.1 | No | － |
| CXCC CSI sub-channel | 15.3.1 15.3.4 | Yes | － |
| Synchronization to Neighbor system | 15.2 | Yes | － |
| coexistence messaging/ inter-system communication over the air | 15.3.3 15.6 | No | － |
| energy pulse signaling in time domain | 15.3.4 | Yes | － |
| energy pulse signaling in frequency domain | 15.3.5 | No | － |
| BSIS messages | 15.3.6 | No | － |
| coexistence proxy | 15.1.5 | Yes | － |
| Working channel allocation optimization | 15.4.2 | No | － |
| Master Frame allocation optimization | 15.4.3 | No | － |
| Secondary Master Priority | 15.4.1.3 | No | － |
| Interference control in Slave and Shared | 15.4.1 | No | － |
| CT-CXP over the air | 15.4.5 | No | － |
| CT-CXP over backhaul | 15.4.5 | No | － |

Systems implementing ProfX_CC3 shall meet the minimum performance requirements listed in Table xxx9.

Table xxx9 Minimum Performance basic requirements

| Capability | Minimum performance |
| --- | --- |
| Precision of Neighboring BS Synchronization | TDD:[+/-10us] FDD: no requirement |
| TDD DL/UL ratio | [3:2 (+/-2%)] |
| Precision of SS Synchronization to neighbor BS | No requirement |
| SS interference sensitivity | [-97dBm/10MHz] |
| BS interference sensitivity | [-97dBm/10MHz] |
| Interference detection duration | [10s] |
| Interference detection probability | [>90%] |
| Interference source identification sensitivity | [2 dB SINR degradation (INR -2.3dB)] |
| Interference source identification duration | [10s] |
| Interference source identification probability | [>90%] |

12.5.2.4 ProfX_CC4: Coordinated coexistence profile 2 features

Table xxx6 Feature requirements profX_CC2

| Feature | Reference | Required | Comment |
| --- | --- | --- | --- |
| CX with SSU | 6.4.4.2 | Y | To be requested only in frequency bands in which SSU have a defined regulatory status |
| DCS (passive channel selection) | 6.4.2.3.2 | Y |  |
| UCP | 6.4.2.3.7 | N |  |
| Provisioning | 15.2 | Y | GPS Sync |
| CXCC Timing | 15.3.1 | Y |  |
| CXCC Sub-channel 1 | 15.3.1 | Y |  |
| CXCC Sub-channel 2 | 15.3.1 15.3.3 | Y |  |
| CXCC Sub-channel 3 | 15.3.1 | N |  |
| CXCC Sub-channel 4 | 15.3.1 | N |  |
| CXCC CSI Sub-channel | 15.3.1 15.3.4 | N |  |
| Candidate Channel and Master Frame Assessment (CCMFA) | 15.3.2 | Y |  |
| CX-Frame | 15.4.1 | Y |  |
| CX-CBP | 15.4.1.4 | Y |  |
| Channel optimization | 15.4.2 | N |  |
| Master frame optimization | 15.4.3 | N |  |
| Interference control | 15.4.4 | Y |  |
| Token Protocol | 15.4.5 | N |  |
| CXP | 15.5 | Y |  |
| Inter-system communication | 15.6 | Y |  |
| Network-based identification of SSU | 15.8 | N |  |

12.5.2.5 ProfX_CC5: Coordinated coexistence profile 5 features

Table xxx12 Feature requirements profX_CC5

| Feature | Reference | Required | Comment |
| --- | --- | --- | --- |
| CX with SSU | 6.4.4.2 | Y | To be requested only in frequency bands in which SSU have a defined regulatory status |
| DCS (passive channel selection) | 6.4.2.3.2 | Y |  |
| UCP | 6.4.2.3.7 | N |  |
| Provisioning | 15.2 | Y | GPS Sync |
| CXCC Timing | 15.3.1 | Y |  |
| CXCC Sub-channel 1 | 15.3.1 | Y |  |
| CXCC Sub-channel 2 | 15.3.1 15.3.3 | Y |  |
| CXCC Sub-channel 3 | 15.3.1 | Y |  |
| CXCC Sub-channel 4 | 15.3.1 | Y |  |
| CXCC CSI Sub-channel | 15.3.1 15.3.4 | N |  |
| Candidate Channel and Master Frame Assessment (CCMFA) | 15.3.2 | Y |  |
| CX-Frame | 15.4.1 | Y |  |
| CX-CBP | 15.4.1.4 | Y |  |
| Channel optimization | 15.4.2 | Y |  |
| Master frame optimization | 15.4.3 | Y |  |
| Interference control | 15.4.4 | Y |  |
| Token Protocol | 15.4.5 | Y |  |
| CXP | 15.5 | Y |  |
| Inter-system communication | 15.6 | Y |  |
| Network-based identification of SSU | 15.8 | Y |  |