| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Primitives in 802.16h – high level approach |  |
| Date Submitted | 2008-05-01 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv, Israel | E-mail: mariana.goldhamer@alvarion.com |
| Re: | LB 29 |  |
| Abstract | The document is a proposal for profiles based on the poll taken in November 2007 meeting, in relation with comment 092 in database IEEE 802.16/07-53r2 |  |
| Purpose | [Description of what specific action is requested of the 802.16 Working Group or subgroup.] |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Primitives in 802.16h – high level approach

Mariana Goldhamer

Alvarion Ltd.

# Introduction

In response to comments addressing the existence of the IP messages in the 802.16h Draft P802.16h/D4, are proposed a number of changes to enforce the primitives approach. The changes are based on the approach in P802.16Rev2/D4, which includes the 802.16g solutions.

Below is extracted the relevant text for the 802.16h work.

1. NCMS

We will need to add a box named: Coexistence Services.

2. Network  Reference Model

The model includes two Service Access Points: C-SAP and S-SAP. See fig. 4.

The M-SAP uses a higher latency transport and may be used for:

The Control-SAP will be 802.16h preferred SAP.  It includes now a number of functions (see below).

We need to add the functions which are contained in CXP.

In clause 14.1 we have the primitive definition template. It includes the following:

The actual primitive template is provided below for the operation type.

In 16g the attribute list is provided as an array, each attribute having its code.

## Proposed changes

Clause 1:

Add in Fig.3 a box named “Coexistence Services”

Clause 15.6  Coexistence Primitives for WirelessMAN-CX

This section defines the Coexistence (CX) service primitives for use at C-SAP and M-SAP at BS side. The specific mapping of service primitives to protocol messages in the backhaul network is out of scope of 802.16.  For easing the understanding of the document the protocol primitives as a group are named CXP.

15.6.1 Coexistence (CX) primitives

- This section defines CX primitive for Management (M-SAP) and Control (C-SAP), following the naming convention in clause 14.1.

(Note for the Editor: follow fron here the C802.16h-08/25r2)

Instruction for the Editor: move the attribute table in the general part

Table h9 gives a list of possible attributes.

- Attributes for CXP primitives

| Type | Parameter Description | Length (bytes) | Comment |
| --- | --- | --- | --- |
| 01 | BSID of source BS | 6 |  |
| 02 | GPS coordinates | 2 |  |
| 03 | BS Network Address of source BS | Variable | Depends of the network addressing within a protocol. For example, in case of for IP usage: 4 bytes if IPv4 is supported 16 bytes if IPv6 is supported |
| 04 | MAC Frame duration | 1 |  |
| 05 | MAC Frame number | 1 |  |
| 06 | Sub-frame number | 1 |  |
| 07 | Repetition interval between two Master sub-frames, measured in MAC-frames | 1 |  |
| 08 | Time shift from the MAC Frame start | 2 | (in microsec) |
| 09 | Duration of the time slot | 2 | (in microsec) |
| 10 | Repetition information for the Base Station radio-signature transmission | 2 | In MAC Frames number |
| 11 | Repetition of the Subscriber Station radio-signature transmission | 2 | In MAC Frames number |
| 12 | List of other used sub-frames, in the interval between two Master sub-frames |  |  |
| 13 | Country Code | 1 | (in ASCII - bytes) |
| 14 | Operator contact - phone | 1 | (in ASCII - bytes) |
| 15 | Operator contact – E-mail | 1 | (in ASCII - bytes) |
| 16 | Channel information | 2 | The channel information of the requested BS. Containing Modulation mode, alternative Channel Flag |
| 17 | Maximum coverage at Max. power |  |  |
| 18 | Tx power | 1 | in dBm |
| 19 | Channel Center Frequency(ChannelCenterFrequency) | 4 | in 10kHz |
| 20 | Transmit antenna type (1 - omni, 2 - directional) | 1 |  |
| 21 | Transmit antenna gain (dBi) | 1 | Signed value |
| 22 | Acceptance indication: | 1 | 1 - accepted 2 - rejected 3,... - message dependent |
| 23 | Channel width | 2 | In 0.01MHz |
| 24 | Type | 1 | message specific |
| 25 | Reduction of transmit power | 1 | in dB |
| 26 | Gap DIUC/UIUC insertion | 1 | 1 - for this system 2 - for systems which do not comply with the criteria at type |
| 27 | Number (message specific) | 1 | Number of distinct BS configurations < 20 Number of SSs < 200 |
| 28 | Two field structure | 2 | Each element in the structure is coded with 8 bits) |
| 29 | Radio signature type | 1 | Radio Signature type: 1 - RSS1 of this system, signature from SS1 (logical numbering) 2 - RSS2 of this system . . 199 - RSS199 of this system 200 - RBS1 of this system (BS in configuration 1, logical numbering) 201 - RBS2 of this system, configuration 2 . . 220 - RBS20 of this system, configuration 20 221 - maximum interference from all the CX_SS (compatible with WirelessMAN-CX), which operate inside the frequency block 222- maximum interference from all the CX_BS, which operate inside the frequency block 223 - maximum interference from all the CX_SS, which operate in a channel, which overlaps the operating channel of the requesting BS 224 - maximum interference from all the CX_BS, which operate in a channel, which overlaps the operating channel of the requesting BS 225 - maximum interference from all the NON-CX_SS, which operate in a channel, which overlaps the operating channel of the requesting BS 226 - maximum interference from all the NON-CX_BS, which operate in a channel, which overlaps the operating channel of the requesting BS 227 - maximum interference from all the CX_SS, which operate in the upper adjacent channel relative to the operating channel of the requesting BS 228 - maximum interference from all the CX_BS, which operate in the upper adjacent channel relative to the operating channel of the requesting BS 229 - maximum interference from all the CX_SS, which operate in the lower adjacent channel relative to the operating channel of the requesting BS 230 - maximum interference from all the CX_BS, which operate in the lower adjacent channel relative to the operating channel of the requesting BS |
| 30 | Renting_out_start_time | 2 | in millisecond |
| 31 | Renting_out_end_time | 2 | in millisecond |
| 32 | Negotiation_Mode_Bit_Flag (NMBF) | 1 | scalar |
| 33 | T_renting_sub-frame | 2 | in millisecond |
| 34 | Start_negotiation_time | 2 | in millisecond |
| 35 | End_negotiation_time | 2 | in millisecond |
| 36 | Pricing_Bit_Flag (PBF) | 1 | Scalar |
| 37 | Minimum number of Credit Token (MNCT) | 6 | in number of credit tokens |
| 38 | Requester_bid | 6 | in number of credit tokens |
| 39 | Rented_resource_amount | 1 | Scalar |
| 40 | Renting_in_start_time | 2 | in millisecond |
| 41 | Renting_in_end_time | 2 | in millisecond |
| 42 | Minimal_payoff | 6 | scalar |
| 43 | Maximal_payoff | 6 | scalar |
| 44 | Requester_bid_update | 6 | in number of credit tokens |
| 45 | Resource_Granting_Bit_Flag (RGBF) | 1 | scalar |
| 46 | Clearing_price | 6 | in number of credit tokens |
| 47 | Renting_sub-frame_start_time | 2 | in millisecond |
| 48 | Renting_sub-frame_end_time | 2 | in millisecond |
| 49 | Acceptation_Bit_Flag (ABF) | 1 | scalar |
| 50 | LC | 1 | scalar |
| 51 | Coexistence_community_BSID | 24 | List of the BSID of the systems belonging to the community of the offeror for CT-CXP |
| 52 | Radio Application Identifier (guidance from ECC Decision 25) | 8 | ASCII coding: AGA - Air Ground Air BSS - Broadcasting Satellite Service CRS - Central Radio Station DCS 1800 - Digital Communication System DECT - Digital Enhanced Cordless Telecommunication System DME - Distance Measuring Equipment DSI - Detailed Spectrum Investigation DVB-T - Terrestrial Digital Video Broadcasting EESS - Earth Exploration-Satellite Service EGSM - Extended GSM ENG - Electronic News Gathering EPIRB - Emergency Position-Indicating Radiobeacon ERMES - European Radio Messaging System FSS - Fixed Satellite Service FWA - Fixed Wireless Access GMDSS - Global Maritime Distress and Safety System GNSS - Global Navigation Satellite System GSM - Global System for Mobile Communications HAPS - High Altitude Platform Systems HDTV - High Definition Television HIPERLAN - High Performance Radio Local Area Network IBCN - Integrated Broadband Communications Network ILS - Instrument Landing System IMT-2000- International Mobile Telecommunications ISM - Industrial, Scientific and Medical applications JTIDS - Joint Tactical Information Distribution System MIDS - Multifunctional Information Distribution System MLS - Microwave Landing System MSI - Maritime Safety Information MSS - Mobile Satellite Service MWS - Multimedia Wireless Systems NGSO - Non-geostationary Satellite Orbit OB - Outside Broadcasting OR - Off-Route PAMR - Public Access Mobile Radio (PMR) PMR - Professional Mobile Radio, Private Mobile Radio R - Route RA - Radio Astronomy SAB - Services Ancillary to Broadcasting SAP - Services Ancillary to Programming S-PCS - Satellite Personal Communication System SRD - Short Range Device TETRA - Terrestrial Trunked Radio RFID - Radio Frequency Identification systems RLAN - Radio Local Area Network RTTT - Road Transport & Traffic Telematics SNG - Satellite News Gathering SRD - Short Range Devices SSR - Secondary Surveillance Radar T-DAB - Terrestrial Digital Audio Broadcasting TACAN - Tactical Air Navigation System VLBI - Very Long Baseline Interferometry (Radio Astronomy) VOR - VHF Omni-directional Range VTS - Vessel Traffic System (radar) VSAT - Very Small Aperture Terminal |
| 53 | Radius of protection area | 1 | In km |
| 54 | Date | 3 | Format: day (5 bits): month (4 bits):year(8bits) - decimal digits, day being most significant |
| 55 | Absolute time | 4 | Format: hour(5bits): minutes (6 bits): seconds (6 bits) - decimal digits, hour being most significant |
| 56 | Antenna direction | 2 | In degrees, clock-wise, reference North (180 for South) |
| 57 | Long duration | 3 | In minutes, hex number |
| 58 | Adjacent Channel Leakage Ratio (ACLR) | 1 | Ratio of the transmitter power to the leakage power in the adjacent channel, in dB |
| 59 | Number of structures | 1 | Number of structures to be listed in continuations |
| 60 | Number of TLVs in a structure | 1 | Used in conjunction with the Number of structures |
| 61 | ID of the destination forwarding SS | 6 |  |
| 62 | Notification Bit Flag (NBF) | 1 | This flag indicates whether the forwarding SS is selected to complete the CT-CXP operations or not: 1: forwarding SS is selected 0: forwarding SS is not selected |
| 63 | BSID of destination | 6 |  |
| 64 | BS IP of destination BS | variable | 4 bytes if IPv4 is supported 16 bytes if IPv6 is supported |
| 65 | BS EIRP | 1 | dBm |
| 66 | BS_GPS_LOC | 4 | 16 MSB for BS Lat 16 LSB for BS long |
| 67 | BS_HGHT | 2 | Height of BS antenna above sea level in meters. |
| 68 | BS_RF_Sector_ID | 5 | bits 0-3: ID bits 4-7: gain (dBi),2dB resolution bits 8-15: azimuth, 2 degree resolution bits 16-23: 3dB aperture, 2 degrees resolution bites 24-25: polarization, with: 00 - vertical, 01- horizontal, 11: circular bit 26: beam-forming, 1=yes bits 27-30 no.of antenna elements bits 31-34 no.of possible beams bit 35 - Interference cancellation capability bit 36 - MIMO capability (which can imply interference cancellation to the same PHY cases) bits 37-39: reserved Note: In the case of beamforming the antenna gain refers to the gain of a single beam The azimuth and 3dB aperture refers to the whole sector covered Beam directions will be considered as uniformly spaced within the sector Beamwidth will be approximated as the sector beamwidth divided by the number of antennas A "1" in either MIMO capability or Interference cancellation capability would mean that the BS/SS is able to devote antenna resources to cancel external interference. |
| 69 | SSID | 6 |  |
| 70 | SS EIRP | 1 | dBm |
| 71 | SS_RF_Sector_ID | 5 | bits 0-3: ID bits 4-7: gain (dBi), 2dB resolution bits 8-15: azimuth, 2 degree resolution bits 16-23: 3dB aperture, 2 degrees resolution bits 24-25: polarization, with: 00- vertical, 01- horizontal, 11- circular bit 26: beam-forming, 1=yes bits 27-30 no. of antenna elements bits 31-34 no. of possible beams bit 35 - Interference cancellation capability bit 36 - MIMO capability (which can imply interference cancellation to the same PHY cases) bits 37-39: reserved Note: In the case of beamforming the antenna gain refers to the gain of a single beam The azimuth and 3dB aperture refers to the whole sector covered Beam directions will be considered as uniformly spaced within the sector Beamwidth will be approximated as the sector beamwidth divided by the number of antennas A "1" in either MIMO capability or Interference cancellation capability would mean that the BS/SS is able to devote antenna resources to cancel external interference. |
| 72 | DFS_LE_PWR_FRQ | 4 | Bit 0: Set to 1 for non-WirelessMAN-CX Bits1-5: Device Type Bits 6-15: Device detection specific Bits 15-31: CCID |
| 73 | CX Proxy network address 4 bytes | 4 | to support for example IPv4 |
| 74 | CX Proxy network address 6 bytes | 16 | to support for example IPv6 |
| 75 | GPS location for Fixed SS | 2 | provides additional information regarding SS location for a protocol sniffer |
| 76 | DL Master sub-frame | 1 | Indicator of the MAC Frame in CX-Frame |
| 77 | Coordinated Coexistence type | 1 | 1: CX-Frame 2. CX-Frame and CX-CBP |

Instruction to Editor: move the text at the right place

The detailed description of the service primitives is provided in continuation.

15.6.1.1 M-CX-REQ (Action Type= Search Neighbour)

Function:

This primitive is used by a Base Station to request BSIS to provide the geographical information of the neighbor Base Stations.

Semantics:

M-CX-REQ

(

Operation Type : Action ;

Action Type: Search Neighbour;

Destination: BSIS

Attribute_List:

(

Null

)

When generated:

This primitive is generated when the BS is installed for the first time when the BS wants to up-date its internal data-base.

Effect of Receipt:

BSIS shall respond with a message based on the M-CX-RSP primitive.

15.6.1.2 M-CX-RSP (Action Type=Search Neighbours)

Function:

The foreign BSIS responds to the requesting BSIS to Search Neighbors Request with a Search Neighbors Response message.

The query results are in the format of a list of structures, each structure describing a Base Station using the attributes defined in Table h11. .

Semantics:

M-CX-RSP

(

Operation type : Action

Action type : Search Neighbours

Destination: BS

Attribute List:

Number of Structures

for (i=0, i < Number of structures)

(

BSID

Channel Center Frequency

Channel Width

Channel information

Latitude

Longitude

Altitude

Maximum coverage at Max. power

)

)

- Search Neighbors Response message attribute

| Attribute | Contents |
| --- | --- |
| Number of structures | The number of Base Stations for which is provided the information |
| BSID | The BSID of the requested BS. |
| Channel Center Frequency | in10kHz |
| Channel Width | in10kHz |
| Channel information | The channel information of the requested BS. Containing Modulation mode, alternative Channel Flag ??? |
| DL Master sub-frame | Indicator of the MAC Frame in CX-Frame |
| Coordinated coexistence type | The coordinated coexistence type (CX-Frame or CX-Frame + CX-CBP) |
| Latitude | The latitude information of the BS. |
| Longitude | The longitude information of the BS. |
| Altitude | The altitude information of the BS. |
| Maximum coverage at Max. power | The maximum radius at maximum allowed/designed power that the BS intends to detect its coexistence neighbors. |

When generated:

The primitive is generated as response to the message using the M-CX-REQ primitive, with ActionType=Search Neighbours.

Effect of Receipt

BS should up-date the list of its neighbors.