| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CMI Related Primitives |  |
| Date Submitted | 2008-05-07 |  |
| Source(s) | John Sydor Communications Research Centre Ottawa, Canada | Voice: 1-613-998-2388 Fax: e-mail to : john.sydor@crc.ca |
| Re: | IEEE 802.16 Working Group Letter Ballot #29 |  |
| Abstract | Use of Layer 2 communications to support the CXP has required the definition of Primitives. This document details the primitives related to the CMI process. |  |
| Purpose | Discussion and accept. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# CMI Related Primitives

John Sydor

Siva Palaninathan

Communications Research Centre, Ottawa, Canada

Introduction

In order to support BS to BS coexistence (possibly with the help of a Network Management Control System (NMCS)),there is a requirement to describe the primitives that will provide the basis for backhaul communications between coexisting BSs.

The use of CMI techniques will therefore require a set of CMI related primitives to pass interference messages and reports. This document provides details of these messages and proposes to include these in Ref 1.

REF 1: IEEE P802.16h/D5 Mar 2008

Proposed Changes

Notes to the Editor (in italic):

Black text is the original from REF 1.

Red Strikethrough text is to be deleted from REF 1.

Blue Underlined text is to be added to REF 1.

The following changes are proposed:

Lines 6-9 Page 144, Table h18 of REF 1

| 19 | CMI_Interference_Resolution_Indication_Request (C-CX-INTRF-RES-RSP) | CX-REQ | UDP | BS->BS | Basic | C-SAP | High | Low |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | CMI_Interference_Resolution_Response (C-CX-INTRF-RES-RSP) | CX-RSP | UDP | BS->BS | Basic | C-SAP | High | Low |

The following changes are to be made to Sections 15.6.1.19 and 15.6.1.20 of REF 1.

15.6.1.19 CMI_Interference_ Resolution_Indication message _Request (C-CX-INTRF-RES-REQ)

Function

This primitive is sent by the interfered-with BS to either the Interfering BS or the NMCS.

It indicates a need to resolve interference.

This message    is sent by the BS to indicate the intensity and type of interference it is receiving as a system. The information content of this message is derived from foreign SSURF and BSD messages the system receives. Non-WirelessMAN-CX interference detected by the system is similarly reported. The message specifies the threshold of interference sensitivity of the victim BS or SS and assigns a CCID report number to specific sources of interference, allowing higher levels of the CX protocol to track the resolution or mitigation of the interference to the victim terminals.

The attributes are indicated in the Table h 34.

Semantics:

The parameters of the primitive are as follows:

C-CX-INTRF-RES-REQ

(

Operation Type:Action

Action Type:Indicate interference for resolution

Destination: BS,NMCS

Attribute List:

(

Interference Victim Device Identity

Interfering Source/DeviceTypes

Inhibition Period

Interference Threshold

Interference Incidence frequency

Interference RSSI Report

CX_CMI

RF Antenna Sector ID

EIRP

IP identity

Device Identity

GPS

BS Height

CCID

)

Table h34- CMI_Interference_ Resolution_Indication message Request (C-CX-INTRF-RES-REQ)

| Attribute | Contents |
| --- | --- |
| Interference Victim Device Identity | BSID or SSID of victim system devices |
| Interfering Source/DeviceTypes | (1) Type or identity of non-WirelessMAN-CX interfering source. (2) BSID or SSID of interfering WirelessMAN-CX device |
| Inhibition Period | Number of CXCC cycle interfered-with device has been inhibited prior to sending an interference report. |
| Interference Threshold | Threshold in dBm for a valid interference event for this interfered–with device. |
| Interference Incidence frequency | Number of interference events per specified CXCC cycles, from indicated interference sources sustained by the device |
| Interference RSSI Report | Mean RSSI level and standard deviation of interference of each interference source. |
| CX_CMI | CMI intervals in which interference detections occur for the device. |
| Interferer Proxy IP address | Proxy addresses of BS systems originating the interference. |
| RF Antenna Sector ID | Radiation pattern characteristics of Interfering system’s BS or SS |
| EIRP | EIRP of the interfering system’s device |
| IP identity | IP identity of the interfering system’s BS proxy address |
| Device Identity | The SSID or BSID of the interfering device. |
| GPS | GPS coordinates of interfering system |
| BS Height | Height of BS of interfering system |
| CCID | Interference report number relating to the specific interference event. |

When generated:

This primitive  is sent by the victim BS to indicate the intensity and type of interference it is receiving as a system. The information content of this primitive is derived from foreign SSURF and BSD messages the system receives. Non-WirelessMAN-CX interference detected by the system is similarly reported. The primitive specifies the threshold of interference sensitivity of the victim BS or SS and assigns a CCID report number to specific sources of interference, allowing higher levels of the CX protocol to track the resolution or mitigation of the interference to the victim terminals.

Effect of Receipt:

This primitive is received by the interfering base station or by  the NMCS. The primitive will result in alterations to the signals originally creating the identified interference (for example, by changing the EIRP, scheduling, or emission characteristics of the interference emitter). If the interference is created by other sources, for example, by primary users, other actions such as channel change may be implemented.

15.6.1.20 CMI_Interference_Resolution_Response message (C-CX-INTRF-RES-RSP)

Function:

This primitive is sent by the interference source BS or the NMCS to the interfered-with BS. It indicates the resolution of an interference occurrence or provides an inhibition to interference at the victim terminal.

This message  is sent to the BS  generating  the CMI_Interference_ Indication message This message sets the threshold of interference detection at the victim device and indicates to that device the status of interference resolution. Other actions related to the Coexistence Protocol and interference mitigation may be taken as well and may precede and/or preclude the sending of this message. Such actions may be related to CINR measurements and other channel quality assessments or involve rapid channel changes as a result of primary user requirements.

The attributes are listed in Table h35.

Semantics:

The parameters of the primitive are as follows:

C-CX-INTRF-RES-RSP

(

Operation Type :Action

Action Type : Modify interference detection thresholds

Destination: BS

Attribute List:

(

Interference Victim Device Identity

CCID

Inhibition Period Set

Interference Threshold Set

Interference Incidence count cycles

Interference Resolution Status

)

Table h35- CMI_Interference_Resolution _Response TLV encoding(C-CX-INTRF-RES-RSP)

| Attribute | Contents |
| --- | --- |
| Interference Victim Device Identity | BSID or SSID of victim device(s) |
| CCID | Interference report number to which this message applies |
| Inhibition Period Set | Inhibit interference report of device by given number of CXCC cycles |
| Interference Threshold Set | Set interference threshold of device to given level. |
| Interference Incidence count cycles | Specify number of CXCC cycles for interference incidence count purpose |
| Interference Resolution Status | Resolution of interference to which stated CCID applies: resolved/tolerated/pending |

When generated:

This primitive is sent to the BS  generating  the CMI_Interference_Resolution_Request (C-CX-INTRF-RES-REQ).  This primitive sets the threshold of interference detection at the victim device and indicates to that device the status of interference resolution. Other actions related to the Coexistence Protocol and interference mitigation may be taken as well and may precede the sending of this message. Such actions may be related to CINR measurements and other channel quality assessments or involve channel changes as a result of primary user requirements.

Effect of Receipt:

The victim BS (or its subscriber stations) is given the status of  its interference condition. The victim station may be have its interference detection  thresholds and response parameters changed by this message. .