| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Changes to Fig h30, etc. |  |
| Date Submitted | 2008-05-13 |  |
| Source(s) | John Sydor Communications Research Centre Ottawa, Canada | Voice: 1-613-998-2388 Fax: e-mail to : john.sydor@crc.ca |
| Re: | IEEE 802.16 Working Group Letter Ballot #29b |  |
| Abstract | Since cross-layer communication is not allowed, changes will be made to D6 with regard to how backhaul communications in support of the CXP are undertaken. These changes affect the IBS entry process, which now must be have text changes undertaken in (Section 15.3.3.3) and in Figure h30 of D5 |  |
| Purpose | Discussion and accept. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Changes to Figure h30,etc.

By

John Sydor

Communications Research Centre, Ottawa, Canada

Introduction

Backhaul communications are needed for the CMI operation, especially for  IBS entry process during which an IBS identifies itself to the OBS within a prospective Coexistence Community using over-the-air messaging (BSD). Once this was done, it was assumed that cross-layer communications ( using the IP layer) would be used to negotiate coexistence  between the IBS and the OBS. Since cross-layer communications is not allowed, alternative approaches to negotiated coexistence are required using only Layer 2 communications. Primitives supporting Layer 2 communication  have been specified in Section 15.6 of Ref 1 and elsewhere.  To reflect the use of these and the approach of using only Layer 2,  the IBS entry process for CMI operation needs revision.

The revisions need to be made to parts of Section 15.3.3.3  and to Figure h30.  These are changes are detailed below.

Proposed Changes

REF1: IEEE P802.16h/D5 Mar 2008

The proposed changes are to:

- Second paragraph of Subsection (C) of Section 15.3.3.3;  Starting at Line  28 and ending at Line 65 on Page 89 of Ref 1.

- Figure h30 located on Page 90 of Ref 1.

Notes to the Editor:

Black type is the original type

Red Strikethrough type is to be deleted.

Blue type is to be added.

Note that the flowchart in the C80216h-08_024 submission is in Microsoft Word

(a) Changes proposed to Second paragraph of Subsection (C) of Section 15.3.3.3;  Starting at Line  28 and ending at Line 65 on Page 89 of Ref 1. Here is the text in question:

To begin the claiming procedure the IBS broadcasts at maximum operational EIRP its BSD (see 15.6.1.3 and 15.3.3.4) message. This message, when received by foreign SS belonging to adjacent networks that form the Interference Neighborhood, will result in those SS informing their home base stations (the OBS) of the presence of a new base station (the IBS) using the MAC message CCID_IND (see 6.3.2.3.62). All OBS having SS that detect the BSD during the claimed CX_CMI_Dn downlink interval will via an IP  backhaul link respond back to the IBS (or its NCMS) using C-CX-REQ primitives (15.6.1.19),  informing it that it has been detected and is a de facto interferer on the downlink. The IBS will include in its BS Information Table the IP network addresses of all of the OBS that have sent  C-CX-REQ primitives in essence redirected their CCID_IND messages to it. The IBS continues its BSD broadcast routine until no new CCID_IND messages primitives are detected. With the receipt of the totality of redirected CCID_IND messages  primitives the IBS will have then have determined the extent of its Interference Neighborhood which is now determined by the set of foreign SS which have received the IBS's BSD messages transmitted during the claimed CX_CMI_Dn. Having this information the IBS (and/or its NCMS) will now know with which OBS it must negotiate coexistence, if indeed it is warranted. If it is not warranted (for instance, because the interference created is too difficult to resolve by the Coexistence Protocol), the IBS abandons the CX_CMI_Dn and chooses an alternative channel that is identified as part of the CCMFA process (15.3.2).

(d) Capacity Negotiation and membership in the Coexistence Community

The OBS networks which the IBS creates or sustains interference to/from become listed in the BS Information Table (See Table h A1) of the IBS. This table contains the BSIDs and IP proxy network addresses derived either from uplink SSURF messages that the IBS demodulated during its monitoring phase (above (a)) or from the primitives REP-RSP messages that it received via the IP network backhaul from the OBS as part of the claiming procedure ((c) above). Communication and negotiation with each OBS listed in the BS Information Table is undertaken via the Coexistence Protocol (CXP). Coexistence entails allocation of uplink and downlink transmission intervals in a manner that eliminates co-channel interference amongst users that would otherwise experience it and sustain degraded communications. This is done by parsing uplink and downlink intervals and establishing shared, master, and slave sub-frames (see 15.4.1.2). Each OBS that the IBS has listed in its BS Information Table as an interfering network must partake in such resolution of interference by scheduling. By undertaking this process the IBS thus creates a Coexistence Community for itself, and consequently becomes accommodated by the neighboring networks of its interference neighborhood.

(b) Changes proposed to Figure h30 located on Page 90 of Ref 1. Here is the new Figure.

IBS Entry

- GPS Timing Synchronization

- Operational integrity check

Wait for CCID_IND C-CX- REQ forwarded from foreign OBS. Extract foreign OBS information and build BS-information table.

CCMFA:Determine set of Candidate Channels

Monitor a candidate channel’s CX_CMI_U(n) occupancy for 10 Tcxcc cycles (5.12 sec)

Broadcast BSD until no new CCID_IND C-CX-REQ received. Finished?

N

Has CXCC Silent Interval interference been detected on candidate channel with   ((N+I)/I>1dB) ?

Y

N

Y

Y

N

Examine BS-information-table,

CXP negotiation process will negotiate the co-existence of this IBS with other OBS, and determine shared, master, and slave sub-frame scheduling.

For every unoccupied CX_CMI_U(n), demodulate the SSURF messages, extract all foreign BS OBS information and put into BS Information Table

IBS broadcasts its BSD msg using the chosen CX_CMI_D(n)

Peak RSSI <= noise floor +1dB?

For every unoccupied CX_CMI_U(n), interval, measure its uplink RSSI for a duration of 10 Tcxcc cycles

Clear CX_CMI_U(n) identified?

IBS entry complete, coexisting community formed complete

N

Y

Y

Y

N

Choose clear CX_CMI_U(n)

N

All negotiation processes complete with every OBS ?

Figure h30---IBS Community Entry Process Using CMI Messages over the CXCC