| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | WirelessMAN coexistence function primitives consolidation |  |
| Date Submitted | 2008-05-02 |  |
| Source(s) | Wu Xuyong Huawei, Huawei Industry Base, Bantian, Longgang, Shenzhen, China 518129 Shulan Feng Hisilicon Tech. Co., LTD Bld.17, No.8, Dongbeiwang West Road, Hai-Dian District, Beijing, P. R. China Harry Bims Apple, Inc. 1 Infinite Loop Cupertino, CA 95014 Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv, Israel | Voice: +86-755-28971787 Fax: +86-755-28972352 wuxuyong@huawei.com Voice: +86-10-82829151 Fax: +86-10-82829075 e-mail to : fengsl@hisilicon.com Voice: 650-283-4174 E-mail: harrybims@mac.com E-mail: mariana.goldhamer@alvarion.com |
| Re: | IEEE 802.16-08/019 IEEE 802.16 Working Group Letter Ballot Recirc #29b: Announcement (2008-04-07) |  |
| Abstract | Semantic consolidation on Coexistence Functionality primitives in 16h, moving CX proxy description into Annex and generalize the network address for basic connectivity between WirelessMAN-CX systems. |  |
| Purpose | To consolidate the 16h draft. |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

WirelessMAN coexistence function primitives consolidation

Wu Xuyong, Shulan Feng, Harry Bims, Mariana Goldhamer

Huawei, HiSilicon, Apple, Alvarion

Overview

In letter ballot recirculation 29a, we have notice that some content in P802.16h-D5 is describing the IP level activity and beyond the scope of 802.16, we should do cleaning up to:

1) Remove IP activity description or put it into informative annex.

2) Define the primitives to enable the coexistence function in WirelessMAN-CX systems.

Reference:

[1]  IEEE 80216-08_009r4：Letter Ballot Recirc #29a Comment Database (2008-04-07)

[2] IEEE P802.16h/D5: 802.16h draft 5(2008-03-22)

[3] IEEE 802.16-08/019: IEEE 802.16 Working Group Letter Ballot #29b: Announcement (2008-04-07)

[4] IEEE 802.16-2004: IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for

Fixed Broadband Wireless Access Systems (2004-10-01)

[5] IEEE 802.16e-2005: IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems Amendment 2: Physical and Medium Access

Control Layers for Combined Fixed and Mobile Operation in Licensed Bands and Corrigendum 1

(2006-02-28)

[6] P802.16Rev2/D4: (April 2008) DRAFT Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems

Proposed Text Changes:

Modified the figure 3 in 1.4.4 accordingly:

Replace according content in 15.6 with following:

The CX primitives are a set of primitives for supporting CX procedures between BS and NCMS.

Figure xx1. primitive flow example for C-CX-REQ/RSP

Figure xx2. primitive flow example for M-CX-REQ/RSP

15.6.1 C-CX-REQ

This primitive is used by an 802.16 entity or NCMS to request a coexistence action. The Action_Type included in this primitive defines the type of coexistence procedure to be performed. The possible Action_Types for this primitive are listed in the Table below:

| Action Type | Description |
| --- | --- |
| Get Parameter for Radio Signature | BS parameters for radio signature transmission |
| Evaluate Interference | Aggregated interference evaluation |
| Work as Slave | Conditions for slave operation |
| Reduce Power or Quit Sub-frame | Power control |
| CMI Interference Resolution | CMI interference reporting |
| Channel Switch | Ask the neighbor system to switch to an alternative channel. |
| Token Advertise | Token advertisement to neighbor BSs |
| Token Negotiation | Token negotiation |
| Token Resource Allocation | Token resource allocation |
| Frequency Channel Avoidance | Frequency channels to be avoided |
| Master Sub-Frame Switch | Request neighbor systems to switch to an alternative master sub-frame allocation |
| OCSI Back-off | Request neighbor systems to cease or restart the OCSI signaling |

The following sub-sections define the primitive when its Action Type is set to a specific action.

15.6.1.1 C-CX-REQ (Action_Type = Get parameter for radio signature)

[Take the text from the relevant contribution.]

15.6.1.2 C-CX-REQ (Action_Type = Evaluate interference)

[Take the text from the relevant contribution.]

15.6.1.3 C-CX-REQ (Action_Type = Work as Slave)

[Take the text from the relevant contribution.]

15.6.1.4 C-CX-REQ (Action_Type = Reduce Power or Quit Sub-frame)

[Take the text from the relevant contribution.]

15.6.1.5 C-CX-REQ (Action_Type = CMI Interference Resolution)

[Take the text from the relevant contribution.]

15.6.1.6 C-CX-REQ (Action_Type = Channel Switch)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to request the requested BS (in the neighborhood contact list of the requesting BS) to switch to an alternative channel.

Semantics of the service primitive:

The parameters of the primitive are as follow:

C-CX-REQ

(

Operation_Type: Action,

Action_Type: Channel Switch,

Destination: BS, NCMS,

Attribute_List:

BSID,

Requested BSID,

Channel Center Frequency,

Channel Width,

Rolling back indication,

FSN

)

| Attribute | Contents |
| --- | --- |
| BSID | The requesting BS identifier |
| Requested BSID | BS identifier of the requested BS |
| Channel Center Frequency | Channel Center Frequency of the requested BS |
| Channel Width | Channel Width of the requested BS |
| Rolling back indication | 0: to switch to one of the alternative channels 1: to switch back to the channel before the last channel switching request |
| FSN | Frame sequence number to switch channel |

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is used by the 802.16 entity (BS) to request the NCMS to inform the requested BS to switch to an alternative channel.

•NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to request the requested neighbor to switch to an alternative channel, while NCMS is requested by the requesting BS.

Effect of receipt:

•NCMS:

The NCMS perform the action to inform the requested BS to switch to an alternative channel.

•802.16 entity (BS):

The requested BS performs the coexistence procedure and response to the requesting BS accordingly.

15.6.1.7 C-CX-REQ (Action_Type = Token Advertise)

[Take the text from the relevant contribution.]

15.6.1.8 C-CX-REQ (Action_Type = Token Negotiation)

[Take the text from the relevant contribution.]

15.6.1.9 C-CX-REQ (Action_Type = Token Resource Allocation)

[Take the text from the relevant contribution.]

15.6.1.10 C-CX-REQ (Action_Type = Frequency Channel Avoidance)

[Take the text from the relevant contribution.]

15.6.1.11 C-CX-REQ (Action_Type = Master Sub-frame Switch)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to request the requested BS (in the neighborhood contact list of the requesting BS) to switch the master sub-frame allocation to an ALTSF.

Semantics of the service primitive:

The parameters of the primitive are as follow:

C-CX-REQ

(

Operation_Type: Action,

Action_Type: Delete neighbor,

Destination: BS, NCMS,

Attribute_List:

BSID,

Requested BSID,

Channel Center Frequency,

Channel Width

sub-frame ID,

Rolling back indication,

FSN

)

| Attribute | Contents |
| --- | --- |
| BSID | The requesting BS identifier |
| Requested BSID | BS identifier of the requested BS |
| Channel Center Frequency | working ChannelCenterFrequency of the requested BS |
| Channel Width | Channel width of the requested BS |
| sub-frame ID | The current master sub-frame ID of the requested BS |
| Rolling back indication | 0: to switch master sub-frame to one of the ALTSF 1: to switch back to the sub-frame before the last master sub-frame switching request |
| FSN | Frame sequence number to switch master sub-frame |

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is used by the 802.16 entity (BS) to request the NCMS to inform the requested BS to switch the master sub-frame allocation to an ALTSF.

•NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to request the requested neighbor to switch the master sub-frame allocation to an ALTSF, while NCMS is requested by the requesting BS.

Effect of receipt:

•NCMS:

The NCMS perform the action to inform the requested BS to switch the master sub-frame allocation to an ALTSF.

•802.16 entity (BS):

The requested BS performs the coexistence procedure and response to the requesting BS accordingly.

15.6.1.12 C-CX-REQ (Action_Type = OCSI Backoff)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to request the requested BS (in the neighborhood contact list of the requesting BS) to hold on the OCSI signaling or recover. See  15.3.4.4

Semantics of the service primitive:

The parameters of the primitive are as follow:

C-CX-REQ

(

Operation_Type: Action,

Action_Type: Delete neighbor,

Destination: BS, NCMS,

Attribute_List:

BSID,

Requested BSID,

OCSN,

Backoff request

)

| Attribute Contents | Contents |
| --- | --- |
| BSID | The requesting BS identifier |
| Requested BSID | identifier of the requested BS |
| OCSN | OCSN of the occupying OCSI |
| Backoff request | 1- start backoff request 0- end of backoff request |

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is used by the 802.16 entity (BS) to request the NCMS to inform the requested BS to hold on the OCSI signaling or recover, when the requesting BS is reported by its SS that collision has been detected in the OCSI which is occupying by this neighbor.

•NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to request the requested BS to hold on the OCSI signaling or recover, when NCMS is requested by the requesting BS.

Effect of receipt:

•NCMS:

The NCMS perform the action to inform the requested BS to hold on the OCSI signaling or recover.

•802.16 entity (BS):

The requested BS performs the coexistence procedure and response to the requesting BS accordingly.

15.6.2 C-CX-RSP

This primitive is used by an 802.16 entity or NCMS to response to a request in coexistence procedure. The Action_Type included in this primitive defines the type of coexistence procedure to be performed. The possible Action_Types for this primitive are listed in the table below:

| Action Type | Description |
| --- | --- |
| Get Parameter for Radio Signature | BS parameters for radio signature transmission |
| Evaluate Interference | Aggregated interference evaluation |
| Work as Slave | Conditions for slave operation |
| Reduce Power or Quit Sub-frame | Power control |
| CMI Interference Resolution | CMI interference reporting |
| Channel Switch | Respond the neighbor’s request of switching to an alternative channel. |
| Token Advertise | Token advertisement to neighbor BSs |
| Token Negotiation | Token negotiation |
| Token Resource Allocation | Token resource allocation |
| Frequency Channel Avoidance | Frequency channels to be avoided |
| Master Sub-Frame Switch | Respond the neighbor’s request of switching to an alternative master sub-frame allocation. |
| OCSI Back-off | Respond the neighbor’s request of holdinf on the OCSI signaling or recover. |

The following sub-sections define the primitive when its action type is set to a specific action.

15.6.2.1 C-CX-RSP (Action_Type = Get parameter for radio signature)

[Take the text from the relevant contribution.]

15.6.2.2 C-CX-RSP (Action_Type = Evaluate interference)

[Take the text from the relevant contribution.]

15.6.2.3 C-CX-RSP (Action_Type = Work as slave)

[Take the text from the relevant contribution.]

15.6.2.4 C-CX-RSP (Action_Type = Reduce power or quit sub-frame)

[Take the text from the relevant contribution.]

15.6.2.5 C-CX-RSP (Action_Type = CMI interference resolution)

[Take the text from the relevant contribution.]

15.6.2.6 C-CX-RSP (Action_Type = Channel Switch)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to respond the request of channel switching to the requesting BS.

Semantics of the service primitive:

The parameters of the primitive are as follow:

C-CX-RSP

(

Operation_Type: Action,

Action_Type: Delete neighbor,

Destination: BS, NCMS,

Attribute_List:

BSID,

Requested BSID,

Acknowledge,

Channel Center Frequency,

Channel Width,

FSN

)

| Attribute | Contents |
| --- | --- |
| BSID | The requesting BS identifier |
| Requested BSID | BS identifier of the requested BS |
| Acknowledge | 0: rejection for fail in switching 1: succeeded in switching |
| Channel Center Frequency | Channel Center Frequency of the requested BS will switch to |
| Channel Width | Channel Width of the requested BS |
| FSN | Frame sequence number of the channel switching |

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is used by the 802.16 entity (BS) to respond the request of channel switching to the requesting BS.

•NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to inform the requesting BS of the response from the requested BS.

Effect of receipt:

•NCMS:

The NCMS perform the action to inform the requesting BS of the response from the requested BS.

•802.16 entity (BS):

The requesting BS updates the information of the responding BS.

15.6.2.7 C-CX-RSP (Action_Type = Token advertise)

[Take the text from the relevant contribution.]

15.6.2.8 C-CX-RSP (Action_Type = Token negotiation)

[Take the text from the relevant contribution.]

15.6.2.9 C-CX-RSP (Action_Type = Token resource allocation)

[Take the text from the relevant contribution.]

15.6.2.10 C-CX-RSP (Action_Type = Frequency Channel Avoidance)

[Take the text from the relevant contribution.]

15.6.2.11 C-CX-RSP (Action_Type = Master Sub-frame Switch)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to respond the request of master sub-frame allocation switching to the requesting BS.

Semantics of the service primitive:

The parameters of the primitive are as follow:

C-CX-RSP

(

Operation_Type: Action,

Action_Type: Delete neighbor,

Destination: BS, NCMS,

Attribute_List:

BSID,

Requested BSID,

Acknowledge,

Target Channel Center Frequency,

Target sub-frame ID,

FSN

)

| Attribute | Contents |
| --- | --- |
| BSID | The requesting BS identifier |
| Requested BSID | BS identifier of the requested BS |
| Acknowledge | 0: rejection for fail in switching 1: succeeded in switching |
| Target Channel Center Frequency (for new master sub-frame) | target ChannelCenterFrequency of the requested BS |
| Target sub-frame ID (for new master sub-frame) | The sub-frame ID of the requested BS will switch its master sub-frame to |
| FSN | Frame sequence number of the master sub-frame switching |

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is used by the 802.16 entity (BS) to respond the request of master sub-frame allocation switching to the requesting BS.

•NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to inform the requesting BS of the response from the requested BS.

Effect of receipt:

•NCMS:

The NCMS perform the action to inform the requesting BS of the response from the requested BS.

•802.16 entity (BS):

The requesting BS updates the information of the responding BS.

15.6.2.12 C-CX-RSP (Action_Type = OCSI backoff)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to respond the request of holding on OCSI signaling or request of recover to the requesting BS. See  15.3.4.4

Semantics of the service primitive:

The parameters of the primitive are as follow:

C-CX-RSP

(

Operation_Type: Action,

Action_Type: Delete neighbor,

Destination: BS, NCMS,

Attribute_List:

BSID,

Requested BSID,

OCSN,

Response indication

)

| Attribute Contents | Contents |
| --- | --- |
| BSID | The requesting BS identifier |
| Requested BSID | identifier of the requested BS |
| OCSN | OCSN of the occupying OCSI |
| Response indication | 01- refuse to backoff 00- refuse to end the backoff 11- notification of acceptance and backoff begin 10- notification of acceptance and backoff end because of timer and counter having run out |

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is used by the 802.16 entity (BS) to respond the request of holding on OCSI signaling or request of recover to the requesting BS.

•NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to inform the requesting BS of the response from the requested BS.

Effect of receipt:

•NCMS:

The NCMS perform the action to inform the requesting BS of the response from the requested BS.

•802.16 entity (BS):

The requesting BS updates the information of the responding BS.

15.6.3 C-CX-IND

This primitive is used by an 802.16 entity or NCMS to notify the other entity about the coexistence event. The possible Event_Types for this primitive are listed in the table below:

| Event Type | Description |
| --- | --- |
| Token Frame Status Update | Token based Frame Status Update |

The following sub-sections define the primitive when its Event_Type is set to a specific type.

15.6.3.1 C-CX-IND (Event_Type = Token Frame Status Update)

[Take the text from the relevant contribution.]

15.6.4 C-CX-ACK

Function:

This primitive acknowledges the reception of an C-CX-IND or C-CX-RSP primitive.

Semantics of the service primitive:

The parameters of the primitive are as follows:

C-CX-ACK

(

Destination: BS or NCMS,

Attribute_List:

No Attribute

)

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is generated when BS receives a C-CX-IND or C-CX-RSP primitive.

• NCMS to 802.16 entity (BS):

When the NCMS receives the C-CX-ACK primitive, the NCMS will generate this primitive to the BS to acknowledge that the neighbor BS has received its C-CX-IND or C-CX-RSP primitive.

Effect of Receipt:

• 802.16 entity (BS) to NCMS:

The NCMS informs the BS that the NCMS has received the corresponding primitive.

• NCMS to 802.16 entity (BS):

Upon receiving this primitive, the BS should update the corresponding internal record.

15.6.5 M-CX-REQ

This primitive is used by an 802.16 entity or NCMS to request a coexistence reaction. The Action_Type included in this primitive defines the type of coexistence procedure to be performed. The possible Action_Types for this primitive are listed in the table below:

| Action Type | Description |
| --- | --- |
| Search Neighbors | Searching neighbor systems by inquiring the BSIS |
| Add neighbor | Add coexistence neighbor procedure in the WirelessMAN-CX system. |
| Delete neighbor | Delete coexistence neighbor procedure in the WirelessMAN-CX system. |

The following sub-sections define the primitive when its action type is set to a specific action.

15.6.5.1 M-CX-REQ (Action_Type = Search Neighbors)

[Take the text from the relevant contribution.]

15.6.5.2 M-CX-REQ (Action_Type = Add neighbor)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to request adding the requesting BS into the neighborhood contact list of the requested BS.

Semantics of the service primitive:

The parameters of the primitive are as follow:

M-CX-REQ

(

Operation_Type: Action,

Action_Type: Add neighbor,

Destination: BS, NCMS,

Attribute_List:

BSID,

Contact Network address,

Channel Center Frequency,

Channel Width,

Channel information

RTK

)

| Attribute | Contents |
| --- | --- |
| BSID | The BSID of the requested BS. |
| Contact Network address | The IP address of the requested BS or the agent of the requested BS. |
| Channel Center Frequency | in10kHz |
| Channel Width | in10kHz |
| Channel information | The channel information of the requesting BS. Containing: Modulation mode (0-reserved; 1-OFDM; 2-OFDMA; 3-15 reserved) alternative Channel Flag (0- no ALTCH; 1- have ALTCH) |
| RTK [Instruct the editor to put it into attribute list as well.] | Real time key for basic connectivity creation using BS_NURBC |

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is used by the 802.16 entity (BS) to request the NCMS to inform the requested BS to add the requesting BS into the neighborhood contact list, while the WirelessMAN-CX system of the requesting BS have discover the requested BS’s system’s neighboring.

•NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to request the requested neighbor to adding the requesting BS into the neighborhood contact list, while NCMS is requested by the requesting BS.

Effect of receipt:

•NCMS:

The NCMS perform the action to inform the requested BS to add the requesting BS into the neighborhood contact list.

•802.16 entity (BS):

The requested BS performs the coexistence procedure and response to the requesting BS accordingly.

15.6.5.3 M-CX-REQ (Action_Type = Delete Neighbor)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to request deleting the requesting BS from the neighborhood contact list of the requested BS.

Semantics of the service primitive:

The parameters of the primitive are as follow:

M-CX-REQ

(

Operation_Type: Action,

Action_Type: Delete neighbor,

Destination: BS, NCMS,

Attribute_List:

BSID,

Contact Network address

)

| Attribute | Contents |
| --- | --- |
| BSID | The BSID of the requested BS. |
| Contact Network address | The IP address of the requested BS or the agent of the requested BS. |

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is used by the 802.16 entity (BS) to request the NCMS to inform the requested BS to delete the requesting BS from the neighborhood contact list, while the WirelessMAN-CX system of the requesting BS is stop neighboring the requested BS’s system.

•NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to request the requested neighbor to delete the requesting BS from the neighborhood contact list, while NCMS is requested by the requesting BS.

Effect of receipt:

•NCMS:

The NCMS perform the action to inform the requested BS to delete the requesting BS from the neighborhood contact list.

•802.16 entity (BS):

The requested BS performs the coexistence procedure and response to the requesting BS accordingly.

15.6.6 M-CX-RSP

This primitive is used by an 802.16 entity or NCMS to response to a request in coexistence procedure. The Action_Type included in this primitive defines the type of coexistence procedure to be performed. The possible Action_Types for this primitive are listed in Table below:

| Action Type | Description |
| --- | --- |
| Search Neighbors | Responding Searching neighbor systems request |
| Add neighbor | Add coexistence neighbor procedure in the WirelessMAN-CX system. |
| Delete neighbor | Delete coexistence neighbor procedure in the WirelessMAN-CX system. |

The following sub-sections define the primitive when its action type is set to a specific action.

15.6.6.1 M-CX-RSP (Action_Type = Search Neighbors)

[Take the text from the relevant contribution.]

15.6.6.2 M-CX-RSP (Action_Type = Add neighbor)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to respond the request of adding the requesting BS into the neighborhood contact list of the responding BS.

Semantics of the service primitive:

The parameters of the primitive are as follow:

C-CX-RSP

(

Operation_Type: Action,

Action_Type: Add neighbor,

Destination: BS, NCMS,

Attribute_List:

No attribute.

)

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is used by the 802.16 entity (BS) to respond the request of adding the requesting BS into the neighborhood contact list of the responding BS.

•NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to inform the requesting BS of the response from the requested BS.

Effect of receipt:

•NCMS:

The NCMS perform the action to inform the requesting BS of the response from the requested BS.

•802.16 entity (BS):

The requesting BS updates the neighborhood contact list.

15.6.2.3 M-CX-RSP (Action_Type = Delete neighbor)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to respond the request of deleting the requesting BS from the neighborhood contact list of the responding BS.

Semantics of the service primitive:

The parameters of the primitive are as follow:

C-CX-RSP

(

Operation_Type: Action,

Action_Type: Delete neighbor,

Destination: BS, NCMS,

Attribute_List:

No attribute.

)

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is used by the 802.16 entity (BS) to respond the request of deleting the requesting BS from the neighborhood contact list of the responding BS.

•NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to inform the requesting BS of the response from the requested BS.

Effect of receipt:

•NCMS:

The NCMS perform the action to inform the requesting BS of the response from the requested BS.

•802.16 entity (BS):

The requesting BS updates the neighborhood contact list.

15.6.2.4 M-CX-RSP (Action_Type = Regulatory Authority)

TBD

15.6.7 M-CX-IND

This primitive is used by an 802.16 entity or NCMS to notify the other entity the coexistence event. The possible Event_Types for this primitive are listed in table below:

| Event Type | Description |
| --- | --- |
| Leaving Neighborhood | Indicate the system is leaving the neighborhood. |

The following sub-sections define the primitive when its event type is set to a specific type.

15.6.7.1 M-CX-IND (Event_Type = Leaving Neighborhood)

Function:

This primitive is sent by an 802.16 entity (BS) or NCMS to announce that the requesting BS is leaving the neighborhood.

Semantics of the service primitive:

The parameters of the primitive are as follows:

M-CX-IND

(

Operation Type : Action,

Action Type : Leaving Neighborhood,

Destination: BS or NCMS,

Attribute_List:

BSID,

Network address,

)

| Attribute | Contents |
| --- | --- |
| BSID | The BSID of the requesting BS. |
| Network address | The IP address of the requesting BS or the agent of the requesting BS |

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is generated when BS is leaving the neighborhood.

• NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to inform the neighbor BS that the requesting BS is leaving the neighborhood.

Effect of Receipt:

• 802.16 entity (BS) to NCMS:

The NCMS perform the action to inform the neighbor BS that the requesting BS is leaving the neighborhood.

• NCMS to 802.16 entity (BS):

The BS receives this primitive shall update their neighbor list and generate the acknowledge primitive to the leaving BS.

15.6.8 M-CX-ACK

Function:

This primitive acknowledges the reception of an M-CX-IND or M-CX-RSP primitive.

Semantics of the service primitive:

The parameters of the primitive are as follows:

M-CX-ACK

(

Destination: BS or NCMS,

Attribute_List:

No Attribute

)

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is generated when BS receives an M-CX-IND or M-CX-RSP primitive.

• NCMS to 802.16 entity (BS):

When the NCMS receives the M-CX-ACK primitive, the NCMS will generate this primitive to the BS to acknowledge that the neighbor BS has received its M-CX-IND or M-CX-RSP primitive.

Effect of Receipt:

• 802.16 entity (BS) to NCMS:

The NCMS informs the BS that the NCMS has received the corresponding primitive.

• NCMS to 802.16 entity (BS):

Upon receiving this primitive, the BS should update the corresponding internal record.

Proposed text changes for the proxy issue.

- Coexistence Proxy (CXPRX): Coexistence proxy acts as an agent to forward and receive the Coexistence Protocol (CXP) messages overover the IP based backhaul.

- ProfX_CC3: Coordinated coexistence profile 3 features

……

| coexistence proxy | Error: Reference source not found | Yes |  |
| --- | --- | --- | --- |

……

- Architecture for WirelessMAN-CX

The architecture for Radio Resource Management in the context of this clause is a distributed one and allows communication and exchange of parameters between different systems. This architecture includes a coexistence proxy and is used in support of the coordinated coexistence protocol for interference identification, prevention and resolution. Every system includes a data base (DB) to store the shared information regarding the actual usage and the intended usage of the radio resource.

To avoid direct exposure to security risks, a BS may use an agency network address for inter-systems communications during coexistence activity. The detail of such higher layer protocol is out of the scope of this standard.