| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Primitives Related to the Neighborhood Update procedure |  |
| Date Submitted | 2008-05-14 |  |
| Source(s) | Shulan Feng Hisilicon Tech. Co., LTD Bld.17, No.8, Dongbeiwang West Road, Hai-Dian District, Beijing, P. R. China Harry Bims Apple, Inc. 1 Infinite Loop Cupertino, CA 95014 Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv, Israel | Voice: +86-10-82829151 Fax: +86-10-82829075 e-mail to : fengsl@hisilicon.com Voice: 650-283-4174 E-mail: harrybims@mac.com E-mail: mariana.goldhamer@alvarion.com |
| Re: | IEEE 802.16-08/019 IEEE 802.16 Working Group Letter Ballot Recirc #29b: Announcement (2008-04-07) |  |
| Abstract | In response to comments addressing the existence of the IP transmission in the 802.16h Draft P802.16h/D4, a number of changes are proposed to enforce the primitives approach. This contribution defines the primitives related to the neighborhood update procedure. |  |
| Purpose | Accept. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Primitives Related to the Neighborhood Update Procedure

Shulan Feng, Harry Bims, Mariana Goldhamer

HiSilicon, Apple, Alvarion

# Introduction

In response to comments addressing the existence of the IP transmission in the 802.16h Draft

P802.16h/D4, a number of changes are proposed to enforce the primitives approach. This contribution defines the Leaving Neighborhood Indication/Acknowledge primitives.

## Proposed changes

- M-CX-IND(Event type = Leaving Neighborhood)Leaving Neighborhood Indication message

Function:

This primitive message is sent by the a 802.16 entity (BS) or NCMS BS to the BSIS and/or its neighbor BSs in order to announce that it the requesting BS is leaving the neighborhood.

Semantics of the service primitive:

The parameters of the primitive are as follows:

M-CX-IND

(

Operation Type : Action,

Action Type : Leaving Neighborhood,

Destination: BS or NCMS,

Attribute_List:

BSID,

CX Network address,

)

Attributes are shown in Leaving.

- Leaving Neighborhood Indication message attributes

| Attribute | Contents |
| --- | --- |
| BSID | The BSID of the requesting BS. |
| CX Proxy IP Network address | The IP address of the requesting BS or the agent of the requesting BS Either one of CX Proxy IP address IPv4 or IPv6 |

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is generated when BS is leaving the neighborhood.

• NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to inform the neighbor BS that the requesting BS is leaving the neighborhood.

Effect of Receipt:

• 802.16 entity (BS) to NCMS:

The NCMS perform the action to inform the neighbor BS that the requesting BS is leaving the neighborhood.

• NCMS to 802.16 entity (BS):

The BS receives this primitive shall update their neighbor list and generate the acknowledge primitive to the leaving BS.

Protocol:

I.	When a new system enters the scene it listens to determine its neighbors. If it successfully hears one or more BSD or SSURF messages (or BS_NURBC in CSI), it creates a compilation of what is necessary for further coexistence mechanism between the new neighbor and itself and send to the new neighbor, and the necessary updating information, if any, will be sent to each of it's neighbors in an Identify Coexistence Request message over the backhaul. Each neighbor responds with an Identify Coexistence Reply message containing their necessary information (which may be more complete than the new BS got over the air) and information on all of their neighbors whether they are neighbors of the new BS or not.

II.	After the neighbors respond with the Identify Coexistence Reply message, they then send the information about the new BS to each of their neighbors using the Add Coexistence Neighbor Request message to inform their neighbors of the new BS. The neighbors respond with an Add Coexistence Neighbor Response message confirming receipt of the information.

III.	Similarly, if a new system enters the scene, and there is another system which the new system cannot hear but the existing system can hear the new system, after a certain timeout without the new system initiating an Identify Coexistence Request/Response, the existing system initiates it as stated in I & II above.

IV.	If a system goes a period of time without hearing a neighbor or otherwise determines a neighbor has disappeared (see V, below), it will send a Delete Coexistence Neighbor Request message to its neighbors indicating that it is removing the system from its list of neighbors. This does not affect whether those other systems still think it exists; only that it is no longer considered a neighbor of the first system.

V.	If a system knowingly leaves the scene (different channel, intentional shutdown, etc.) it should courteously send a Leaving Coexistence Neighborhood Indication message to its neighbors who will respond with a Leaving Coexistence Neighborhood Reply message and should also send Delete Coexistence Neighbor Request message to its neighbors as in IV above.

These same messages could also be sent to known DB servers or BSIS if they exist. Obviously, the DB servers and BSIS wouldn't initiate the protocol since they can't hear BSD, SSURF, or BS_NURBC messages.

- M-CX-ACK(Action type = Leaving Neighborhood)Leaving Neighborhood Reply message

Function:

This primitive acknowledges that an M-CX-IND primitive for leaving neighborhood is received. The BSIS responds to the BS’s Leaving Neighborhood Indication with a Leaving Neighborhood Reply message.

Semantics of the service primitive:

The parameters of the primitive are as follows:

M-CX-RSP

(

Operation Type : Action,

Action Type : Leaving Neighborhood,

Destination: BS or NCMS,

Attribute_List:

No Attribute

)

No Attributes.

When generated:

• 802.16 entity (BS) to NCMS:

This primitive is generated when BS receives an M-CX-IND primitive for leaving neighborhood.

• NCMS to 802.16 entity (BS):

Wen the NCMS receives the M-CX-ACK primitive for leaving neighborhood, the NCMS will generate this primitive to the BS who leaving the neighbor to acknowledge that the neighbor BS has received its M-CX-IND primitive for leaving neighborhood.

Effect of Receipt:

• 802.16 entity (BS) to NCMS:

The NCMS perform the action to inform the BS who leaving the neighbor that the neighbor BS has received its M-CX-IND primitive for leaving neighborhood..

• NCMS to 802.16 entity (BS):

Upon receiving this primitive, the BS should confirm that the neighbor has received its indication primitive of leaving neighborhood and update the list of its neighbor if need.