| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Primitives concerning interference control and network-based DFS |  |
| Date Submitted | 2008-05-13 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv, Israel | E-mail: mariana.goldhamer@alvarion.com |
| Re: | LB 29 |  |
| Abstract | The document is a proposal for profiles based on the poll taken in November 2007 meeting, in relation with comment 092 in database IEEE 802.16/07-53r2 |  |
| Purpose | [Description of what specific action is requested of the 802.16 Working Group or subgroup.] |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Primitives concerning interference control and network-based DFS

Mariana Goldhamer

Alvarion Ltd.

# Introduction

This contribution provides primitives instead of the CXP messages 11-18 and 35-38 (33-36 according to the messages in-text numbering).

- CX primitives

| Code | CX Primitive Name |  |  | Direction |  | SAP type | Precedence | Reliability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Reserved |  |  | — |  |  |  |  |
| 1 | Search Neighbors Request (M-CX-SRC-REQ) |  |  | BS->BSIS |  | M-SAP | Low | High |
| 2 | Search Neighbors Response (M-CX-SRC-RSP) |  |  | BSIS->BS |  | M-SAP | Low | High |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 | Leaving Neighborhood Indication (M-CX-LV-NBR-IND) |  |  | BS-> BSIS |  | M-SAP | Low | High |
| 6 | Leaving Neighborhood Response (M-CX-LV-NBR-RSP) |  |  | BSIS->BS |  | M-SAP | Low | High |
| 7 | Add Coexistence Neighbor Request (M-CX-ADD-NBR-REQ) |  |  | BS->BS |  | M-SAP | Low | High |
| 8 | Add Coexistence Neighbor Response (M-CX-ADD-NBR-RSP) |  |  | BS->BS |  | M-SAP | Low | High |
| 9 | Delete Coexistence Neighbor Request (M-CX-DEL-NBR-REQ) |  |  | BS->BS |  | M-SAP | Low | High |
| 10 | Delete Coexistence Neighbor Response (M-CX-DEL-NBR-RSP) |  |  | BS->BS |  | M-SAP | Low | High |
| 11 | Get_Param_For_Radio_Signature_Request (C-CX-R-SIG-REQ) |  |  | BS->BS |  | C-SAP | High | Low |
| 12 | Get_Param_For_Radio_Signature_Response (C-CX-R-SIG-RSP) |  |  | BS->BS |  | C-SAP | High | Low |
| 13 | Evaluate_Interference_Request (C-CX-EV-INTRF-REQ) |  |  | BS->BS |  | C-SAP | High | Low |
| 14 | Evaluate_Interference_Response (C-CX-EV-INTRF-RSP) |  |  | BS->BS |  | C-SAP | High | Low |
| 15 | Work_as_Slave_Request (C-CX-SLAVE-REQ) |  |  | BS->BS |  | C-SAP | High | Low |
| 16 | Work_ as_Slave_Response (C-CX-SLAVE-RSP) |  |  | BS->BS |  | C-SAP | High | Low |
| 17 | Reduce_Power_or_Quit_Sub_Frame_Request (C-CX-RED-PWR-REQ) |  |  | BS->BS |  | C-SAP | High | Low |
| 18 | Reduce_Power_or_Quit_Sub_Frame_Response (C-CX-RED-PWR-RSP) |  |  | BS->BS |  | C-SAP | High | Low |
| 19 | CMI_Interference_Resolution_Indication (C-CX-INTRF—RES-IND) |  |  | BS->BS |  | C-SAP | High | Low |
| 20 | CMI_Interference_ Resolution_Response (C-CX-INTRF-RES-RSP) |  |  | BS->BS |  | C-SAP | High | Low |
| 21 |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  |
| 23 | Channel Switch Request (C-CX-CH-SWTCH-REQ) |  |  | BS->BS |  | C-SAP | Low | High |
| 24 | Channel Switch Response (C-CX-CH-SWTCH-RSP) |  |  | BS->BS |  | C-SAP | Low | High |
| 25 | Token Advertise Request (C-CX-CT- ADV-REQ) |  |  | BS->BS |  | C-SAP | Low | High |
| 26 | Token Advertise Response (C-CX-CT- ADV-RSP) |  |  | BS->BS |  | C-SAP | Low | High |
| 27 | Token Negotiation Request (C-CX-CT- NEG-REQ) |  |  | BS->BS |  | C-SAP | Low | High |
| 28 | Token Negotiation Response (C-CX-CT- NEG-RSP) |  |  | BS->BS |  | C-SAP | Low | High |
| 29 | Token Resource Allocation Request (C-CX-CT-RA-REQ) |  |  | BS->BS |  | C-SAP | Low | High |
| 30 | Token Resource Allocation Response (C-CX-CT-RA-RSP) |  |  | BS->BS |  | C-SAP | Low | High |
| 31 |  |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  |
| 34 | Token Frame Status Update Indication (M-CX-CT- FRSU-IND) |  |  | BS->BS |  | M-SAP | Low | High |
| 35 | Regulatory Authority Request (M-CX-REG-AUTH-REQ) |  |  | RAIS ->BS |  | M-SAP | Low | High |
| 36 | Regulatory Authority Response (M-CX-REG-AUTH-RSP) |  |  | BS->RAIS |  | M-SAP | Low | High |
| 37 | FREQ_AVOIDANCE Request (C-CX-FRQ-AV-REQ) |  |  | BSIS-BS |  | C-SAP | Low | High |
| 38 | FREQ_AVOIDANCE Response (C-CX-FRQ-AV-RSP) |  |  | BS-BSIS |  | C-SAP | Low | High |
| 39 | Master sub-frame Switch Request (C-CX-M-SWTCH-REQ) |  |  | BS->BS |  | C-SAP | Low | High |
| 40 | Master sub-frame Switch Response (C-CX-M-SWTCH-RSP) |  |  | BS->BS |  | C-SAP | Low | High |
| 41 | OCSI back-off request message (C-CX-OCSI-BOFF-REQ) |  |  | BS->BS |  | C-SAP | Low | High |
| 42 | OCSI back-off response message (C-CX-OCSI-BOFF-RSP) |  |  | BS->BS |  | C-SAP | Low | High |
| 43-255 | reserved |  |  |  |  |  |  |  |

# Text changes

- Introduce the following parameters in Table h9:

Master sub-frame ID				The ID on the Master sub-frame in the CX-Frame: 0,1,2.

2.  Modify the text between the pages 155, L33 and 159, L26 according to the changes below

- C-CX-REQ (Action Type= Get_Param_For_Radio_Signature)

Function:

This primitive is used by a Base Station or by a BS to request another BS to clarify the parameters used for the transmission of its Radio Signatures.

Semantics:

C-CX-REQ

(

Operation Type: Action,

Action Type: Get_Param_For_Radio_Signature,

Destination: BS,

Attribute_List:

(

Null

)

)

When generated:

This primitive is generated before the interpretation of the measured Radio Signatures.

Effect of Receipt

BS should transmit the C-CX-RSP primitive, with ActionType= Get_Param_For_Radio_Signature.

- C-CX-RSP (Action Type= Get_Param_For_Radio_Signature)

Function:

This primitive is used by a BS to indicate which are powers, antenna gains, etc used for the transmission of its Radio Signatures.

Semantics:

C-CX-RSP

(

Operation Type: Action,

Action Type: Get_Param_For_Radio_Signature,

Destination: BS,

Attribute List:

(

GPS coordinates

Channel Center Frequency(ChannelCenterFrequency)

Channel Width (ChW)

Maximum Power to be used

Transmit antenna type (1 – omni, 2 – directional)

Transmit antenna gain (dBi)

Number of distinct BS configurations

Number of SSs

MAC Frame number

)

)

| Attribute | Contents |
| --- | --- |
| GPS coordinates | The GPS coordinates of the BS |
| Channel Center Frequency(ChannelCenterFrequency) | in 10kHz |
| Channel Width (ChW) | in 10kHz |
| Maximum Power to be used | Electrical transmitted power in dBm |
| Transmit antenna type (1 – omni, 2 – directional) |  |
| Transmit antenna gain (dBi) |  |
| Number of distinct BS configurations | Number of distinct BS configurations, for different combinations of powers and beam directions, to be used for radio signatures (<20) |
| Number of SSs | Total number of SSs which will be used for radio signatures (<200) |
|  | Error: Reference source not found |
| MAC Frame number | The first MAC Frame number which contains the Master sub-frame |

When generated:

This primitive shall be generated as response to the message based on the C-CX-REQ (Action Type= Get_Param_For_Radio_Signature).

Effect of Receipt

BS can use the information for parametric calculations of the interference from the BS.

- C-CX-REQ (Action Type=Evaluate_Interference)

Function:

This primitive is used by a BS for requesting other BSs in the community to either transmit their Radio Signatures or to be silent, in order to coordinate their operation during this measurement. This measurement allows to evaluate the maximum aggregated co-channel and adjacent channel interference. The message is sent to every Base Station in the Community, indicating either the request to send the Radio Signature or to insert Gap DIUC/UIUC interval for the duration of the Radio Signature. The Radio signature may be transmitted by the BS or by the associated SSs. In case of the Radio Signature transmission by the BS, which is denoted as RBS, 20 different radio signatures are defined, which may differ due to Beam Forming or power concentration on specific sub-channels. The various RBS signatures are identified by the configuration number. The various SS signatures are identified by the RSS number, which runs from 1 to 199. The message is indicating the MAC Frame number, the offset from its start and the duration of the requested Radio Signature time-slot. A number of radio signatures may be concatenated, under the constraint that their total duration, including the propagation delays, does not exceed the total time-slot duration. It is recommended that this duration will be lower than 1ms. The time-slot shall use the DL sub-frame for BS signatures and UL sub-frame for SS signatures. The Radio Signature should be transmitted using operational characteristics.

Semantics:

C-CX-REQ

(

Operation Type: Action,

Action Type: Evaluate_Interference,

Destination: BS,

Attribute List:

(

GPS coordinates

Channel Center Frequency(ChannelCenterFrequency)

Channel width

Number of concatenated signatures

Radio signature type

Gap DIUC/UIUC insertion

MAC Frame number

Time shift from the MAC Frame start

Duration of the time-slot

)

)

| Attribute | Contents |
| --- | --- |
| GPS coordinates | The GPS coordinates of the -BS |
| Channel Center Frequency(ChannelCenterFrequency) | Channel Center Frequency of the requesting BS, for sending the Radio signature (see 8.5.1) |
| Channel width | Channel width of the Requesting BS, in units of 10kHz |
| Number of concatenated signatures | The number of concatenated radio signatures in a time-slot = N |
| Radio signature type | List with N values |
| Gap DIUC/UIUC insertion | Gap DUIC/UIUC insertion for this system or systems which do not comply with |
| MAC Frame number | MAC Frame number for sending the radio signature |
| Time shift from the MAC Frame start | Start of the signature slot or Gap interval, as offset from the MAC Frame start, in microseconds |
| Duration of the time-slot | Duration of the signature slot or Gap interval, in microseconds; the duration of the Gap interval may be higher than the duration of the Radio signature slot, in order to cover the propagation delays. The duration of the time-slot shall be less than 1ms. |

When generated:

This primitive is generated when a Radio Signature measurement is requested

Effect of Receipt

BS shall send C-CX-RSP for the same Action Type.

- C-CX-RSP (Action type= Evaluate_Interference)

Function:

This primitive is used by the BS to reply to the Evaluate_Interference_Request. It provides the timing and the identification of the scheduled radio signatures.

Semantics:

C-CX-RSP

(

Operation Type: Action,

Action Type: Evaluate_Interference,

Destination: BS,

Attribute List:

(

Acceptance indication

MAC Frame number

Time shift from the MAC Frame start

Duration of the time-slot

Number of concatenated signatures

Two field structure

)

)

| Attribute | Contents |
| --- | --- |
| Acceptance indication | Acceptance code: 1, if the request is fully accepted 2, if the request is rejected 3, if the requested will be executed at a different time; in this case the message will supplementary include the following three fields indicating when will be transmitted the radio signatures. |
| MAC Frame number | MAC Frame number for sending the radio signature |
| Time shift from the MAC Frame start | Start of the signature slot or Gap interval, as offset from the MAC Frame start, in microseconds |
| Duration of the time-slot | Duration of the signature slot or Gap interval, in microseconds; the duration of the Gap interval may be higher than the duration of the Radio signature slot, in order to cover the propagation delays. |
| Number of concatenated signatures | The number of structures in the list |
| Two field structure | { - Offset from the start of the signature slot, in microseconds (8 bits) - Radio Signature type (8 bits) } |

When generated:

This primitive is generated when a Radio Signature measurement was requested.

Effect of Receipt

The receiving BS shall send C-CX-ACK.

- C-CX-REQ (Action Type= Work_As_Slave)

Function:

This primitive is sent by a new BS to request the use an existing Master sub-frame as Slave. In general, the Master BS will evaluate the interference created by the requesting BS before answering this request.

If more than one Base Station is using the sub-frame as Master, the IBS will send this message to every BS separately.

The Radio Signature will be transmitted during the claimed Master sub-frame and will consist of a DL sub-frame, starting with the first zone using the mandatory permutation (if applicable) and continuing with other used zones and their permutations. Each zone will be transmitted using the operational power.

Semantics:

C-CX-REQ

(

Operation Type: Action,

Action Type: Work_As_Slave

Destination: BS

Attribute List:

(

GPS coordinates

Channel Center Frequency

Channel width

Master Frame Index

Power to be used in dBm

Transmit antenna type

Transmit antenna gain

MAC Frame number

Time shift from the MAC Frame start

Duration of the time-slot

)

)

| Attribute | Contents |
| --- | --- |
| GPS coordinates | The GPS coordinates of the IBS |
| Channel Center Frequency | Channel Center Frequency of the requesting BS, for sending the Radio signature (see 8.5.1) |
| Channel width | Channel width of the requesting BS for sending the Radio Signature, in 10kHz unit |
| Master Frame Index |  |
| Power to be used in dBm |  |
| Transmit antenna type (1 – omni, 2 – directional) |  |
| Transmit antenna gain (dBi) |  |
| MAC Frame number | Requested MAC Frame number to place the BS Radio Signatures (see the MAC Frame numbering starting from the absolute time) |
| Time shift from the MAC Frame start | Start of the claimed Master sub-frame, relative to start of the mentioned MAC frame, in μs |
| Duration of the time-slot | Duration of the Radio signature (shall be shorter than the claimed sub-frame duration), in μs |

When generated:

This primitive is generated when an IBS enters a system or when a BS decides to increase the time resource.

Effect of Receipt

The receiving BS shall respond with C-CX-RSP for this Action Type.

- C-CX-RSP (Action Type= Work_As_Slave)

Function:

This primitive is sent by a existing Master BS in response to the Work_As_Slave..

Semantics:

C-CX--RSP

(

Operation Type: Action,

Action Type: Work_As_Slave,

Destination: BS,

Attribute List:

(

Channel Center Frequency

Channel width

Receive antenna type (1 – omni, 2 – directional)

Receive Antenna Gain

Acceptance indication

Request for reduction of the transmit power, in dB

)

)

| Attribute | Contents |
| --- | --- |
| Channel Center Frequency (ChannelCenterFrequency) | Channel identifier of the responding OBS |
| Channel width | Channel width of the responding Master BS |
| Receive antenna type | For the Master BS |
| Receive Antenna Gain | For the Master BS |
| Acceptance indication | 1:=Acceptance 2 = Rejection 3 – Accepted if the IBS transmit power is reduced |
| Request for reduction of the transmit power, in dB | indicates the request for transmit power reduction, in dB |

When generated:

This primitive is generated when C-CX-REQ for this Action Type was received.

Effect of Receipt

The receiving BS shall respond with C-CX-ACK.

- C-CX-REQ(Action Type= Reduce_Power_or_Quit_Sub_Frame)

Function:

This primitive is sent by an operating Base Station, using the sub-frame as Master, in order to request anotherBase Station to cease the operation as Master or Slave in the current sub-frame. Supplementary, the primitive may indicate the power reduction for accepting the Base Station to use the sub-frame as Slave.

Semantics:

C-CX-REQ

(

Operation Type: Action,

Action Type: Reduce_Power_or_Quit_Sub_Frame,

Destination: BS,

Attribute List:

(

Channel Center Frequency

Channel width

Master sub-frame ID

Number of concatenated structures

Two field structure

)

)

Reduce_Power_or_Quit_Sub_Frame_Request Parameter set.

| Attribute | Contents |
| --- | --- |
| Channel Center Frequency (ChannelCenterFrequency) | Channel Center Frequency of the requesting BS, in 10kHz unit |
| Channel width | Channel width of the requesting BS, in 10kHz unit |
| Master sub-frame ID | The Master sub-frame used by the requesting BS |
| Number of concatenated structures |  |
| Two field structure | { - Radio Signature identifier (8bits) - Reduction of transmit power for operating in parallel; 255 dB has the significance of a request to STOP using the sub-frame (8 bits) } |

When generated:

This primitive is generated when a Master BS perceives a too high interference

Effect of Receipt

The receiving BS shall respond with C-CX-RSP for this Action Type .

- C-CX-RSP (Action type= Reduce_Power_or_Quit_Sub_Frame)

Function:

- This primitive is sent by a Base Station, in response to the Master Base Station’s C-CX-REQ primitive with Action type= Reduce_Power_or_Quit_Sub_Frame)

. This primitive indicates the actual power reduction for the requested signatures.

Semantics:

C-CX-RSP

(

Operation Type: Action,

Action Type: Reduce_Power_or_Quit_Sub_Frame,

Destination: BS,

Attribute List:

(

Channel Center Frequency

Channel width

Number of concatenated structures

Two field structure

)

)

| Attribute | Contents |
| --- | --- |
| Channel Center Frequency | Channel Center Frequency of the replying BS |
| Channel width | Channel width of the replying BS, in units of 10kHz |
| Number of concatenated structures |  |
| Two field structure | { - Signature identifier (8bits) - Reduction of transmit power for operating in parallel; 255 dB has the significance of a request to STOP using the sub-frame (8 bits) } |

When generated:

This primitive is generated following a C-CX-REQ with the same Action Type.

Effect of Receipt

The receiving BS shall respond with C-CX-ACK.

3. Modify the text between page 170, L50 and page 173, L18 as shown below:

- C-CX-REQ (Action Type=FREQ_AVOIDANCE)

Function:

The RAIS communicates to BS which are the operating parameters of the application to be protected, the date and time requested for protection, the area of protection, etc. The primitive is sent separately for each application to be protected.

Semantics:

C-CX-REQ

(

Operation Type: Action,

Action Type: FREQ_AVOIDANCE,

Destination: BS,

Attribute List:

(

Radio Application identifier

Tx power

Antenna type

Antenna gain

Antenna direction

Latitude

Longitude

Altitude

Maximum coverage

Number of structures

Number of TLVs in a structure

Channel Center Frequency of the transmit channel

Channel width of the transmit channel

Channel Center Frequency of the receive channel

Tx power

ACLR

ACLR

Channel width of the receive channel

Date

Absolute time

Long duration

)

)

)

| Attribute | Contents |
| --- | --- |
| Radio Application identifier | Radio Application identifier |
| Tx power | Tx power (EIRP) of the application to be protected |
| Antenna type | Of the application to be protected, same for Tx and Rx |
| Antenna gain | Of the application to be protected, same for Tx and Rx |
| Antenna direction | Of the application to be protected, same for Tx and Rx |
| Latitude | The latitude information of the center of the area to be protected. |
| Longitude | The longitude information of the center of the area to be protected. |
| Altitude | The altitude information of the center of the area to be protected. |
| Maximum coverage | Required radius of the protection area (optional) for a transmitter power of 1W EIRP |
| Number of structures | Number of elements of the following seven fields structures |
| Number of TLVs in a structure | 7 |
| ChannelCenterFrequency of the transmit channel | Center frequency of Tx operation of the protected application |
| Channel width of the transmit channel | Channel width of operation of the protected application |
| ChannelCenterFrequency of the receive channel | Center frequency of Rx operation of the protected application |
| Tx power | Maximum transmitted power in the channel |
| ACLR | ACLR of the first adjacent channel, for the protected application |
| ACLR | ACLR of the second adjacent channel, for the protected application |
| Channel width of the receive channel | Channel width of operation of the protected application |
| Date | Date of the requested operation start |
| Absolute time | Hour/min/sec/millisecond of the requested operation start |
| Long duration | Duration of the requested operation |

When generated:

This primitive is generated following a change in the SSU operation or when is needed to up-date the requests of the RAIS.

Effect of Receipt

- The receiving BS shall respond with C-CX-RSP for this Action Type.

- C-CX-RSP (Action Type= FREQ_AVOIDANCE)

Function:

The BS responds to the RAIS request for FREQ_AVOIDANCE. .

The messages specify, as confirmation, the date and time at which the requested protection will take place.

Semantics:

C-CX-FRQ-AV-RSP

(

Operation Type: Action,

Action Type: FREQ_AVOIDANCE,

Destination: RAIS

Attribute List:

(

Radio Application identifier

Date

Absolute time

Long duration

)

)

FREQ_AVOIDANCE Response attributes

| Attribute | Contents |
| --- | --- |
| Radio Application identifier | Radio Application identifier |
| Date | Date of the requested operation start |
| Absolute time | Hour/min/sec/millisecond of the requested operation start |
| Long duration | Duration of the requested operation |

When generated:

This primitive is generated following the RAIS request.

Effect of Receipt

The receiving BS shall respond with C-CX-ACK.

4. elete from Table h9 the attribute “type of sub-frame allocation”.

5. Delete from Table h8 the primitives

- Regulatory Authority Request (M-CX-REG-AUTH-REQ)

- Regulatory Authority Response (M-CX-REG-AUTH-RSP).