| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Changes to clause “Network - based identification of specific spectrum services” |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv, Israel | E-mail: mariana.goldhamer@alvarion.com |
| Re: | LB 29 |  |
| Abstract | The document is a proposal for profiles based on the poll taken in November 2007 meeting, in relation with comment 092 in database IEEE 802.16/07-53r2 |  |
| Purpose | [Description of what specific action is requested of the 802.16 Working Group or subgroup.] |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Changes to clause “Network - based identification of specific spectrum services”

Mariana Goldhamer

Alvarion Ltd.

# Introduction

In response to comments addressing the existence of the IP messages in the 802.16h Draft P802.16h/D4, are needed also a number of changes to the Networking approach in clause 15.8. The full concept shall include always a 802.16 entity, such that the RAIS shall communicate directly to a BS, and not using the BSIS as intermediate. Any text related to the BSIS was deleted (including some Token-related text).

# Text changes

- Network - based identification of specific spectrum services or applications

Overview

The method described below allows identifying specific (protected) spectrum users based on their GPS coordinates and the geographic protection area, as defined by the national regulations. Such spectrum users, during their activity period at a specific location, register themselves with a database of the National Radio Administration and indicated the range of used frequencies and the requested time intervals for protection. The registration can use a WEB application, above the scope of this document.

BWA Base Stations register themselves with the same data-base. The list of the provided attributes should include the following elements:

- GPS location

- Network address

- Tx power

- List including N structures for N channels as described below:

- Center frequency of the transmit channel i (both FDD and TDD);

- Channel width of channel i;

- ACLR (Adjacent Channel Leakage Ratio)

- ACLR in the 2nd adjacent channel

- Center frequency of the receive channel j, if FDD is used

- Bandwidth of channel j

- Antenna type and direction

- Noise power

The list of the parameters to be provided by the SSU should include:

- Radio application identification

- GPS location of the center of the protection area (not necessarily the GPS location of its operation)

- Required radius of the protection area (optional)

- Tx power

- List including N structures for N channels as described below:

- Center frequency of the transmit channel i (both FDD and TDD);

- Channel width of channel i;

- ACLR of channel i

- ACLR in the 2nd adjacent channel to channel i

- Center frequency of the receive channel j, if FDD is used

- Bandwidth of channel j;

- Antenna type and direction

- Noise power

- Date and absolute time of requested operation start

- Duration of operation; for backhaul-less systems the duration will be max. [4] hours. In case of longer operation, the spectrum request shall be renewed.

A special application, running on the Radio Administration Server, should identify the limitations imposed to the BWA applications, regarding the:

- operating frequency channels

- transmit power

- time constraints.

Architecture

The proposed architecture uses the RAIS (Radio Application Identification Server) for the devices providing specific radio applications. Generally the RAIS will be provided by the National Radio Administration. The communication of the BS with the RAIS is secured by VPN or other procedures.

gives an example of the proposed architecture. The RAIS has connections with the BWA Base Stations, but also with Base Stations providing TV applications, Satellite applications, Military applications, Health applications, Security applications, etc..

A special class are the Ad-Hoc protected applications, as wireless microphones or TV vehicle transmissions, which are mobile ad-hoc protected applications. In this situation a secure communication will be established between a Laptop with cellular interface or a hand-held device and the RAIS.

- Architecture for coexistence of BWA and preferred radio applications

Operation

When a change of the spectrum usage or an up-date is necessary, the RAIS will send the C-CX-FREQ-AVOIDANCE-REQ primitive to the affected BSs with the request to change the operating frequency and eventually with a recommendation for the new operating frequencies. From this moment all the operators in the frequency range will share the available spectrum, following procedures to be agreed or those defined by WirelessMAN-CX.

The BSwill inform the RAIS about the availability of the requested frequency channel, by using the primitive C-CX-FREQ-AVOIDANCE-RSP.