| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Changes to clause “Special procedures for backhaul-less systems” |  |
| Date Submitted | 2008-05-06 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv, Israel | E-mail: mariana.goldhamer@alvarion.com |
| Re: | LB 29 |  |
| Abstract |  |  |
| Purpose | [Description of what specific action is requested of the 802.16 Working Group or subgroup.] |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Changes to clause “Special procedures for backhaul-less systems”

Mariana Goldhamer

Alvarion Ltd.

# Introduction

In response to comments addressing the existence of the IP messages in the 802.16h Draft P802.16h/D4, are needed also a number of changes to the Backhaul-less systems in clause 15.3.5.4. As the backhaul operation is not in the scope of this draft, the name “backhaul-less” has no meaning and needs to be changed.

# Text changes

- Special signaling procedures for CX systems

In this clause are defined the procedures using energy keying in frequency domain for synchronization and selection of the operational sub-frame. These procedures are intended for low-cost CX systems, which may not use a GPS or equivalent synchronization source.

- Operating principles

The signaling procedures described in this clause allow:

- Synchronization using CXCC

-     Operation without causing interference to systems using a sub-frame as Master.

The

Synchronization with CXCC is done by looking for the signaling on sub-channel 1 and sub-channel 3, on the intended operating channel; if no synchronization signals are detected the search will be done also on those adjacent channels on which is detected energy. If no signals are detected the system will start to operate in a stand-alone mode, but will have to look for synchronization at least every 10min.

In order to avoid creating interference, the CXCC synchronized systems shall follow the Registration procedures described below.

- Registration

Registration request

The CXCC allocations within sub-channel 3 and starting with the second CXCC cycle in the Multi-frame is the beginning of a registration interval using the Coexistence Signaling. The systems shall use the CXCC allocations for sending their cumulated radio signatures during one of the allocations corresponding to a Master sub-frame, both in DL and UL. The radio signature will be used for the evaluation of the potential interference during the Master slot, to systems which use the sub-frame as Master systems. The legacy Master systems will not transmit data during CXCC allocations for sub-channel 3.

The radio signature will consist of a preamble and a MAC header, sent on the working channel and using the same power and sub-carrier allocation, as used in the regular data transmission mode;

The systems will have to use the registration procedures every Tad_reg seconds. (see Error: Reference source not found).

Registration reply

The radio units using the Master sub-frame will send a NACK (see Error: Reference source not found) signal, during the next CXCC cycle, and using the same sub-frame as used by the un-acceptable transmitter, if they appreciate that the registerin transmitter will cause interference. Typically, to a registration signal sent during a DL sub-frame, the NACK will be sent by one or more SSs, while to a registration signal sent during UL sub-frame, the NACK signal will be sent by a Base Station.

The NACK signal indicates that the requesting device cannot use the specific sub-frame, while using the requesting radio signature

Same device may try again, if using a different radio signature (for example, lower power).

Lack of response indicates that the registration is accepted for transmission during the specific sub-frame.

- Selection of suitable reception sub-frames

An unit operating according to this clause will identify suitable reception sub-frames, by using the ACS and Registration processes repetitively, searching for a suitable operation frequency. The practical interference situations, with synchronized MAC frames are BS-SS and SS-BS interference. Assuming similar transmit powers, the above mentioned processed will find, as result, Master sub-frames in which the path attenuations between interfering units are maximized.