| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Improvements to clause 15.6 |  |
| Date Submitted | 2008-06-05 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv, Israel | E-mail: mariana.goldhamer@alvarion.com |
| Re: | LB 29c |  |
| Abstract | The document is a proposal for profiles based on the poll taken in November 2007 meeting, in relation with comment 092 in database IEEE 802.16/07-53r2 |  |
| Purpose | [Description of what specific action is requested of the 802.16 Working Group or subgroup.] |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Improvements to clause 15.6

Mariana Goldhamer

Alvarion Ltd.

# Introduction

The text can be improved, as recommended below.

# Text changes

The clauses proposed for modification are presented below.

- Attributes for CX primitives

| Type | Parameter Description | Length (bytes) | Comment |
| --- | --- | --- | --- |
| 01 | BSID of source BS | 6 |  |
| 02 | GPS coordinates | 3 |  |
| 03 | BS Network Address of source BS | Variable | Depends of the network addressing within a protocol. For example, in case of for IP usage: 4 bytes if IPv4 is supported 16 bytes if IPv6 is supported |
| 04 | MAC Frame duration | 1 |  |
| 05 | MAC Frame number | 1 | The 8 least-significant bits of the absolute MAC Frame number |
| 06 | Sub-frame number | 1 |  |
| 08 | Time shift from the MAC Frame start | 2 | (in microsec) |
| 09 | Duration of the time slot | 2 | (in microsec) |
| 16 | Channel information | 2 | The channel information of the requested BS. Containing Modulation mode, alternative Channel Flag |
| 17 | Maximum coverage |  | At max. power |
| 18 | Tx power | 1 | in dBm |
| 19 | Channel Center Frequency(ChannelCenterFrequency) | 4 | in 10kHz |
|  | Channel Center Frequency of the receive channel | 4 | in 10kHz |
| 20 | Antenna type | 1 | 1 – omni 2 - directional |
| 21 | Antenna gain | 1 | In dBi; signed value |
| 22 | Acceptance indication: | 1 | 1 - accepted 2 - rejected 3,... - message dependent |
| 23 | Channel width | 2 | In 0.01MHz |
| 24 | Type | 1 | message specific |
| 25 | Reduction of transmit power | 1 | in dB |
| 26 | Gap DIUC/UIUC insertion | 1 | 1 - for this system 2 - for systems which do not comply with the criteria at type |
| 27 | Numberof distinct BS configurations | 1 | Number of distinct BS configurations < 20 |
| xx | Number of SSs | 1 | Number of SSs < 200 |
| 28 | Two field structure | 2 | Each element in the structure is coded with 8 bits) |
| 29 | Radio signature type | 1 | Radio Signature type: 1 - RSS1 of this system, signature from SS1 (logical numbering) 2 - RSS2 of this system . . 199 - RSS199 of this system 200 - RBS1 of this system (BS in configuration 1, logical numbering) 201 - RBS2 of this system, configuration 2 . . 220 - RBS20 of this system, configuration 20 221 - maximum interference from all the CX_SS (compatible with WirelessMAN-CX), which operate inside the frequency block 222- maximum interference from all the CX_BS, which operate inside the frequency block 223 - maximum interference from all the CX_SS, which operate in a channel, which overlaps the operating channel of the requesting BS 224 - maximum interference from all the CX_BS, which operate in a channel, which overlaps the operating channel of the requesting BS 225 - maximum interference from all the NON-CX_SS, which operate in a channel, which overlaps the operating channel of the requesting BS 226 - maximum interference from all the NON-CX_BS, which operate in a channel, which overlaps the operating channel of the requesting BS 227 - maximum interference from all the CX_SS, which operate in the upper adjacent channel relative to the operating channel of the requesting BS 228 - maximum interference from all the CX_BS, which operate in the upper adjacent channel relative to the operating channel of the requesting BS 229 - maximum interference from all the CX_SS, which operate in the lower adjacent channel relative to the operating channel of the requesting BS 230 - maximum interference from all the CX_BS, which operate in the lower adjacent channel relative to the operating channel of the requesting BS |
| 30 | Renting_out_start_time | 2 | in millisecond |
| 31 | Renting_out_end_time | 2 | in millisecond |
| 32 | Negotiation_Mode_Bit_Flag (NMBF) | 1 | scalar |
| 33 | T_renting_sub-frame | 2 | in millisecond |
| 34 | Start_negotiation_time | 2 | in millisecond |
| 35 | End_negotiation_time | 2 | in millisecond |
| 36 | Pricing_Bit_Flag (PBF) | 1 | Scalar |
| 37 | Minimum number of Credit Token (MNCT) | 6 | in number of credit tokens |
| 38 | Requester_bid | 6 | in number of credit tokens |
| 39 | Rented_resource_amount | 1 | Scalar |
| 40 | Renting_in_start_time | 2 | in millisecond |
| 41 | Renting_in_end_time | 2 | in millisecond |
| 42 | Minimal_payoff | 6 | scalar |
| 43 | Maximal_payoff | 6 | scalar |
| 44 | Requester_bid_update | 6 | in number of credit tokens |
| 45 | Resource_Granting_Bit_Flag (RGBF) | 1 | scalar |
| 46 | Clearing_price | 6 | in number of credit tokens |
| 47 | Renting_sub-frame_start_time | 2 | in millisecond |
| 48 | Renting_sub-frame_end_time | 2 | in millisecond |
| 49 | Acceptation_Bit_Flag (ABF) | 1 | scalar |
| 50 | LC | 1 | scalar |
| 51 | Coexistence_community_BSID | 24 | List of the BSID of the systems belonging to the community of the offeror for CT-CXP |
| 52 | Radio signature identifier | 1 | Radio signature identifier |
| 52 | Radio Application Identifier (guidance from ECC Decision 25) | 8 | ASCII coding: AGA - Air Ground Air BSS - Broadcasting Satellite Service CRS - Central Radio Station DCS 1800 - Digital Communication System DECT - Digital Enhanced Cordless Telecommunication System DME - Distance Measuring Equipment DSI - Detailed Spectrum Investigation DVB-T - Terrestrial Digital Video Broadcasting EESS - Earth Exploration-Satellite Service EGSM - Extended GSM ENG - Electronic News Gathering EPIRB - Emergency Position-Indicating Radiobeacon ERMES - European Radio Messaging System FSS - Fixed Satellite Service FWA - Fixed Wireless Access GMDSS - Global Maritime Distress and Safety System GNSS - Global Navigation Satellite System GSM - Global System for Mobile Communications HAPS - High Altitude Platform Systems HDTV - High Definition Television HIPERLAN - High Performance Radio Local Area Network IBCN - Integrated Broadband Communications Network ILS - Instrument Landing System IMT-2000- International Mobile Telecommunications ISM - Industrial, Scientific and Medical applications JTIDS - Joint Tactical Information Distribution System MIDS - Multifunctional Information Distribution System MLS - Microwave Landing System MSI - Maritime Safety Information MSS - Mobile Satellite Service MWS - Multimedia Wireless Systems NGSO - Non-geostationary Satellite Orbit OB - Outside Broadcasting OR - Off-Route PAMR - Public Access Mobile Radio (PMR) PMR - Professional Mobile Radio, Private Mobile Radio R - Route RA - Radio Astronomy SAB - Services Ancillary to Broadcasting SAP - Services Ancillary to Programming S-PCS - Satellite Personal Communication System SRD - Short Range Device TETRA - Terrestrial Trunked Radio RFID - Radio Frequency Identification systems RLAN - Radio Local Area Network RTTT - Road Transport & Traffic Telematics SNG - Satellite News Gathering SRD - Short Range Devices SSR - Secondary Surveillance Radar T-DAB - Terrestrial Digital Audio Broadcasting TACAN - Tactical Air Navigation System VLBI - Very Long Baseline Interferometry (Radio Astronomy) VOR - VHF Omni-directional Range VTS - Vessel Traffic System (radar) VSAT - Very Small Aperture Terminal |
| 53 | Radius of protection area | 1 | In km |
| 54 | Date | 3 | Format: day (5 bits): month (4 bits):year(8bits) - decimal digits, day being most significant |
| 55 | Absolute time | 4 | Format: hour(5bits): minutes (6 bits): seconds (6 bits) - decimal digits, hour being most significant |
| 56 | Antenna direction | 2 | In degrees, clock-wise, reference North (180 for South) |
| 57 | Long duration | 3 | In minutes, hex number |
| 58 | ACLR first | 1 | Ratio of the transmitter power to the leakage power in the adjacent channel, in dB |
|  | ACLR second | 1 | Ratio of the transmitter power to the leakage power in the second adjacent channel, in dB |
| 59 | Number of structures | 1 | Number of structures to be listed in continuations |
| 60 | Number of TLVs in a structure | 1 | Used in conjunction with the Number of structures |
| 61 | ID of the destination forwarding SS | 6 |  |
| 62 | Notification Bit Flag (NBF) | 1 | This flag indicates whether the forwarding SS is selected to complete the CT-CXP operations or not: 1: forwarding SS is selected 0: forwarding SS is not selected |
| 63 | BSID of destination | 6 |  |
| 64 | BS IP of destination BS | variable | 4 bytes if IPv4 is supported 16 bytes if IPv6 is supported |
| 65 | BS EIRP | 1 | dBm |
| 66 | BS_GPS_LOC | 6 | 24 MSB for BS Lat 24 LSB for BS long |
| 67 | BS_HGHT | 2 | Height of BS antenna above sea level in meters. |
| 68 | BS_RF_Sector_ID | 5 | bits 0-3: ID bits 4-7: gain (dBi),2dB resolution bits 8-15: azimuth, 2 degree resolution bits 16-23: 3dB aperture, 2 degrees resolution bites 24-25: polarization, with: 00 - vertical, 01- horizontal, 11: circular bit 26: beam-forming, 1=yes bits 27-30 no.of antenna elements bits 31-34 no.of possible beams bit 35 - Interference cancellation capability bit 36 - MIMO capability (which can imply interference cancellation to the same PHY cases) bits 37-39: reserved Note: In the case of beamforming the antenna gain refers to the gain of a single beam The azimuth and 3dB aperture refers to the whole sector covered Beam directions will be considered as uniformly spaced within the sector Beamwidth will be approximated as the sector beamwidth divided by the number of antennas A "1" in either MIMO capability or Interference cancellation capability would mean that the BS/SS is able to devote antenna resources to cancel external interference. |
| 69 | SSID | 6 |  |
| 70 | SS EIRP | 1 | dBm |
| 71 | SS_RF_Sector_ID | 5 | bits 0-3: ID bits 4-7: gain (dBi), 2dB resolution bits 8-15: azimuth, 2 degree resolution bits 16-23: 3dB aperture, 2 degrees resolution bits 24-25: polarization, with: 00- vertical, 01- horizontal, 11- circular bit 26: beam-forming, 1=yes bits 27-30 no. of antenna elements bits 31-34 no. of possible beams bit 35 - Interference cancellation capability bit 36 - MIMO capability (which can imply interference cancellation to the same PHY cases) bits 37-39: reserved Note: In the case of beamforming the antenna gain refers to the gain of a single beam The azimuth and 3dB aperture refers to the whole sector covered Beam directions will be considered as uniformly spaced within the sector Beamwidth will be approximated as the sector beamwidth divided by the number of antennas A "1" in either MIMO capability or Interference cancellation capability would mean that the BS/SS is able to devote antenna resources to cancel external interference. |
| 72 | DFS_LE_PWR_FRQ | 4 | Bit 0: Set to 1 for non-WirelessMAN-CX Bits1-5: Device Type Bits 6-15: Device detection specific Bits 15-31: CCID |
| 73 | CX contacting network address IPv4 | 4 | to support for example IPv4 |
| 74 | CX contacting network address IPv6 | 16 | to support for example IPv6 |
| 75 | SS_GPS_Loc | 6 | 24 MSB for SS Lat 24 LSB for SS Log |
| 76 | SS_HGHT | 2 | Height of SS above sea level in meters |
| 77 | DL Master sub-frame | 1 | Indicator of the MAC Frame in CX-Frame |
| 78 | Coordinated Coexistence type | 1 | 1: CX-Frame 2. CX-Frame and CX-CBP |
| 79 | Master sub-frame ID | 1 | The ID on the Master sub-frame in the CX-Frame: 0,1,2 |
|  | Latitude | 5 | See clause 11.21.1 Absolute Position TLV (Long Format) |
|  | Longitude | 5 | See clause 11.21.1 Absolute Position TLV (Long Format) |
|  | Altitude | 5 | See clause 11.21.1 Absolute Position TLV (Long Format) |
|  | Number of TLVs | 1 |  |
|  | Channel width of the receive channel | 2 | In 0.01MHz |
|  | Offset from the start of the signature slot | 1 | In microseconds |

- C-CX-REQ

This primitive is used by an 802.16 entity or NCMS to request a coexistence action. The Action_Type included in this primitive defines the type of coexistence procedure to be performed. The possible Action_Types for this primitive are listed in the table below:

| Action Type | Description |
| --- | --- |
| Get Parameter for Radio Signature | BS parameters for radio signature transmission |
| Evaluate Interference | Aggregated interference evaluation |
| Work as Slave | Conditions for slave operation |
| Reduce Power or Quit Sub-frame | Power control |
| CMI Interference Resolution | CMI interference reporting |
| Channel Switch | Ask the neighbor system to switch to an alternative channel. |
| Token Advertise | Token advertisement to neighbor BSs |
| Token Negotiation | Token negotiation |
| Token Resource Allocation | Token resource allocation |
| Frequency Channel Avoidance | Frequency channels to be avoided |
| Master Sub-Frame Switch | Request neighbor systems to switch to an alternative master sub-frame allocation |
| OCSI Back-off | Request neighbor systems to cease or restart the OCSI signaling |

The following sub-sections define the primitive when its Action_Type is set to a specific action.

- C-CX-REQ (Action_Type = Get Parameter for Radio Signature)

Function:

This primitive is used by a Base Station or by a BS to request another BS to clarify the parameters used for the transmission of its Radio Signatures.

Semantics:

C-CX-REQ

(

Operation_Type: Action,

Action Type: Get Parameter for Radio Signature,

Destination: BS,

Attribute_List:

No attribute.

)

When generated:

This primitive is generated before the interpretation of the measured Radio Signatures.

Effect of receipt:

BS should transmit the C-CX-RSP primitive with Action_Type= Get Parameter for Radio Signature.

- C-CX-REQ (Action_Type = Evaluate Interference)

Function:

This primitive is used by a BS for requesting other BSs in the community to either transmit their Radio Signatures or to be silent, in order to coordinate their operation during this measurement. This measurement allows evaluating the maximum aggregated co-channel and adjacent channel interference. The message is sent to every Base Station in the Community, indicating either the request to send the Radio Signature or to insert Gap DIUC/UIUC interval for the duration of the Radio Signature. The Radio signature may be transmitted by the BS or by the associated SSs. In case of the Radio Signature transmission by the BS, which is denoted as RBS, 20 different radio signatures are defined, which may differ due to Beam Forming or power concentration on specific sub-channels. The various RBS signatures are identified by the configuration number. The various SS signatures are identified by the RSS number, which runs from 1 to 199. The message is indicating the MAC Frame number, the offset from its start and the duration of the requested Radio Signature time-slot. A number of radio signatures may be concatenated, under the constraint that their total duration, including the propagation delays, does not exceed the total time-slot duration. It is recommended that this duration will be lower than 1ms. The time-slot shall use the DL sub-frame for BS signatures and UL sub-frame for SS signatures. The Radio Signature should be transmitted using the operational characteristics of the radio behaviour.

In case that a specific BS is requested to insert a GAP/DIUC the parameters specifying the transmission of the Radio Signature shall not be sent.

Semantics:

C-CX-REQ

(

Operation_Type: Action,

Action Type: Evaluate Interference,

Destination: BS,

Attribute_List:

GPS coordinates,

Channel Center Frequency (ChannelCenterFrequency),

Channel width,

Number of concatenated signatures,

Radio signature type,

Gap DIUC/UIUC insertion,

MAC Frame number,

Time shift from the MAC Frame start,

Duration of the time-slot

)

| Attribute | Contents |
| --- | --- |
| GPS coordinates | The GPS coordinates of the BS |
| Channel Center Frequency(ChannelCenterFrequency) | Channel Center Frequency of the requesting BS, for sending the Radio signature (see 8.5.1) |
| Channel width | Channel width of the Requesting BS, in units of 10kHz |
| Number of concatenated signatures | The number of concatenated radio signatures in a time-slot = N |
| Radio signature type | List with N values |
| Gap DIUC/UIUC insertion | Gap DUIC/UIUC insertion for this system or systems which do not comply with |
| MAC Frame number | MAC Frame number for sending the radio signature |
| Time shift from the MAC Frame start | Start of the signature slot or Gap interval, as offset from the MAC Frame start, in microseconds |
| Duration of the time-slot | Duration of the signature slot or Gap interval, in microseconds; the duration of the Gap interval may be higher than the duration of the Radio signature slot, in order to cover the propagation delays. The duration of the time-slot shall be less than 1ms. |

When generated:

This primitive is generated when a Radio Signature measurement is requested

Effect of receipt:

BS shall send C-CX-RSP for the same Action Type.

- C-CX-REQ (Action_Type = Work as Slave)

Function:

This primitive is used by a new BS to request the use of an existing Master sub-frame as Slave. In general, the Master BS will evaluate the interference created by the requesting BS before answering this request. If more than one Base Station is using the sub-frame as Master, the IBS will send this message to every BS separately.

The Radio Signature will be transmitted during the claimed Master sub-frame and will consist of a DL sub-frame, starting with the first zone using the mandatory permutation (if applicable) and continuing with other used zones and their permutations. Each zone will be transmitted using the operational power.

Semantics:

C-CX-REQ

(

Operation_Type: Action,

Action Type: Work as Slave

Destination: BS

Attribute_List:

GPS coordinates,

Channel Center Frequency,

Channel width,

Sub-Frame ID,

Tx Power,

Transmit antenna type,

Transmit antenna gain,

MAC Frame number,

Time shift from the MAC Frame start,

Duration of the time-slot

)

| Attribute | Contents |
| --- | --- |
| GPS coordinates | The GPS coordinates of the IBS |
| Channel Center Frequency | Channel Center Frequency of the requesting BS, for sending the Radio signature (see 8.5.1) |
| Channel width | Channel widthof the requesting BS for sending the Radio Signature, in 10kHz unit |
| Sub-Frame ID | Master sub-frame ID. |
| Tx Power | Power to be used (in dBm) |
| Antenna type | Transmit antenna type: 1 – omni 2 - directional |
| Antenna gain | Transmit antenna gain in dBi. |
| MAC Frame number | Requested MAC Frame number to place the BS Radio Signatures (see the MAC Frame numbering starting from the absolute time) |
| Time shift from the MAC Frame start | Start of the claimed Master sub-frame, relative to start of the mentioned MAC frame, in μs |
| Duration of the time-slot | Duration of the Radio signature (shall be shorter than the claimed sub-frame duration), in μs |

When generated:

This primitive is generated when an IBS enters a system or when a BS decides to increase the time resource.

Effect of receipt:

The receiving BS shall respond with C-CX-RSP for this Action Type.

- C-CX-REQ (Action_Type = Reduce Power or Quit Sub-frame)

Function:

This primitive is used by an operating Base Station, using the sub-frame as Master, in order to request another Base Station to cease the operation as Master or Slave in the current sub-frame. Supplementary, the primitive may indicate the power reduction for accepting the Base Station to use the sub-frame as Slave.

Semantics:

C-CX-REQ

(

Operation_Type: Action,

Action Type: Reduce Power or Quit Sub-frame,

Destination: BS,

Attribute_List:

Channel Center Frequency,

Channel width,

Master sub-frame ID,

Number of concatenated structures,

Two field structure

)

| Attribute | Contents |
| --- | --- |
| Channel Center Frequency | Channel Center Frequency of the requesting BS, in 10kHz unit |
| Channel width | Channel width of the requesting BS, in 10kHz unit |
| Master sub-frame ID | The Master sub-frame used by the requesting BS |
| Duration of the time-slot | Duration of the sub-frame |
| Number of concatenated structures | The number of the two field structures, each structure having two parameters indicated in the two field structure. |
| Two field structure | { - Radio Signature identifier (8bits) - Reduction of transmit power l; 255 dB has the significance of a request to STOP using the sub-frame (8 bits) } |

When generated:

This primitive is generated when a Master BS perceives a too high interference

Effect of receipt:

The receiving BS shall respond with C-CX-RSP for this Action Type.

- C-CX-REQ (Action_Type = Frequency Channel Avoidance)

Function:

The RAIS communicates to BS which are the operating parameters of the application to be protected, the date and time requested for protection, the area of protection, etc. In case that the protected application use paired spectrum, shall be provided the channel parameters also for the receive operation. The primitive is used separately for each application to be protected.

Semantics:

C-CX-REQ

(

Operation_Type: Action,

Action Type: Frequency Channel Avoidance,

Destination: BS,

Attribute_List:

Radio Application identifier,

Tx power,

Antenna type,

Antenna gain,

Antenna direction,

Latitude,

Longitude,

Altitude,

Maximum coverage,

Number of structures,

Number of TLVs

Channel Center Frequency

Channel width,

Channel Center Frequency of the receive channel,

Tx power,

ACLR first,

ACLR second,

Channel width of the receive channel,

Date,

Absolute time,

Long duration

)

| Attribute | Contents |
| --- | --- |
| Radio Application identifier | Radio Application identifier |
| Tx power | Tx power (EIRP) of the application to be protected |
| Antenna type | Of the application to be protected, same for Tx and Rx |
| Antenna gain | Of the application to be protected, same for Tx and Rx |
| Antenna direction | Of the application to be protected, same for Tx and Rx |
| Latitude | The latitude information of the center of the area to be protected. |
| Longitude | The longitude information of the center of the area to be protected. |
| Altitude | The altitude information of the center of the area to be protected. |
| Maximum coverage | Required radius of the protection area (optional) for a transmitter power of 1W EIRP |
| Number of structures | Number of elements of the following seven fields structures |
| Number of TLVs | Number of TLVs in a structure = 7 |
| Channel Center Frequency | Center frequency of Tx operation of the protected application |
| Channel width | Channel width of operation of the protected application |
| Channel Center Frequency of the receive channel | Center frequency of Rx operation of the protected application |
| Tx power | Maximum transmitted power in the channel |
| ACLR first | ACLR of the first adjacent channel, for the protected application |
| ACLR second | ACLR of the second adjacent channel, for the protected application |
| Channel width of the receive channel | Channel width of the receive channel of the protected application |
| Date | Date of the requested operation start |
| Absolute time | Hour/min/sec/millisecond of the requested operation start |
| Long duration | Duration of the requested operation |

When generated:

This primitive is generated following a change in the SSU operation or when is needed to up-date the requests of the RAIS.

Effect of receipt:

The receiving BS shall respond with C-CX-RSP for this Action Type.

- C-CX-RSP

This primitive is used by an 802.16 entity or NCMS to response to a request in coexistence procedure. The Action_Type included in this primitive defines the type of coexistence procedure to be performed. The possible Action_Types for this primitive are listed in the table below:

| Action Type | Description |
| --- | --- |
| Get Parameter for Radio Signature | BS parameters for radio signature transmission |
| Evaluate Interference | Aggregated interference evaluation |
| Work as Slave | Conditions for slave operation |
| Reduce Power or Quit Sub-frame | Power control |
| CMI Interference Resolution | CMI interference reporting |
| Channel Switch | Respond the neighbor’s request of switching to an alternative channel. |
| Token Advertise | Token advertisement to neighbor BSs |
| Token Negotiation | Token negotiation |
| Token Resource Allocation | Token resource allocation |
| Frequency Channel Avoidance | Frequency channels to be avoided |
| Master Sub-Frame Switch | Respond the neighbor’s request of switching to an alternative master sub-frame allocation. |
| OCSI Back-off | Respond the neighbor’s request of holdinf on the OCSI signaling or recover. |

The following sub-sections define the primitive when its action type is set to a specific action.

- C-CX-RSP (Action_Type = Get Parameter for Radio Signature)

Function:

This primitive is used by a BS to indicate which are powers, antenna gains, etc used for the transmission of its Radio Signatures.

Semantics:

C-CX-RSP

(

Operation_Type: Action,

Action Type: Get Parameter for Radio Signature,

Destination: BS,

Attribute_List:

GPS coordinates,

Channel Center Frequency

Channel Width

Tx power

Transmit antenna type

Transmit antenna gain

Number of distinct BS configurations,

Number of SSs,

MAC Frame number

)

| Attribute | Contents |
| --- | --- |
| GPS coordinates | The GPS coordinates of the IBS |
| Channel Center Frequency | in 10kHz |
| Channel Width | in 10kHz |
| Tx power | Electrical transmitted power in dBm |
| Antenna type | Transmit antenna type : 1 – omni, 2 – directional |
| Antenna gain | Transmit antenna gain in dBi |
| Number of distinct BS configurations | Number of distinct BS configurations, for different combinations of powers and beam directions, to be used for radio signatures (<20) |
| Number of SSs | Total number of SSs to be used for radio signatures (<200) |
| Master sub-frame ID |  |

When generated:

This primitive shall be generated as response to the message based on the C-CX-REQ (Action Type = Get Parameter for Radio Signature).

Effect of receipt:

BS can use the information for parametric calculations of the interference from the BS.

- C-CX-RSP (Action_Type = Evaluate Interference)

Function:

This primitive is used by the BS to reply to the C-CX-REQ with Action_Type=Evaluate Interference. It provides the timing and the identification of the scheduled radio signatures.

Semantics:

C-CX-RSP

(

Operation_Type: Action,

Action Type: Evaluate Interference,

Destination: BS,

Attribute_List:

Acceptance indication,

MAC Frame number,

Time shift from the MAC Frame start,

Duration of the time-slot,

Number of concatenated signatures,

Two field structure

)

| Attribute | Contents |
| --- | --- |
| Acceptance indication | Acceptance code: 1- if the request is fully accepted 2- if the request is rejected 3- if the requested will be executed at a different time; in this case the message will supplementary include the following three fields indicating when will be transmitted the radio signatures. |
| MAC Frame number | MAC Frame number for sending the radio signature |
| Time shift from the MAC Frame start | Start of the signature slot or Gap interval, as offset from the MAC Frame start, in microseconds |
| Duration of the time-slot | Duration of the signature slot or Gap interval, in microseconds; the duration of the Gap interval may be higher than the duration of the Radio signature slot, in order to cover the propagation delays. |
| Number of concatenated signatures | The number of structures in the list |
| Two field structure | { - Offset from the start of the signature slot, in microseconds (8 bits) - Radio Signature type (8 bits) } |

When generated:

This primitive is generated when a Radio Signature measurement was requested.

Effect of receipt:

The receiving BS shall send C-CX-ACK.

- C-CX-RSP (Action_Type = Work as Slave)

Function:

This primitive is used by an existing Master BS (OBS) in response to the Work as Slave.

Semantics:

C-CX--RSP

(

Operation_Type: Action,

Action Type: Work as Slave,

Destination: BS,

Attribute_List:

Channel Center Frequency,

Channel width,

Antenna type (1 - omni, 2 - directional),

Antenna Gain,

Acceptance indication,

Request for reduction of the transmit power, in dB

)

| Attribute | Contents |
| --- | --- |
| Channel Center Frequency | Channel identifier of the responding OBS |
| Channel width | Channel width of the responding OBS |
| Receive antenna type | Receive antenna type for the OBS |
| Receive Antenna Gain | Receive antenna gain for the OBS |
| Acceptance indication | 1-Acceptance 2-Rejection 3-Accepted if the IBS transmit power is reduced |
| Reduction of transmit power | indicates the request for transmit power reduction, in dB |

When generated:

This primitive is generated when C-CX-REQ for this Action Type was received.

Effect of receipt:

The receiving BS shall respond with C-CX-ACK.

- C-CX-RSP (Action_Type = Reduce Power or Quit Sub-frame)

Function:

This primitive is used by a Base Station, in response to the Master Base Station's C-CX-REQ primitive with Action type= Reduce Power or Quit Sub-frame.

This primitive indicates the actual power reduction for the requested signatures.

Semantics:

C-CX-RSP

(

Operation_Type: Action,

Action Type: Reduce Power or Quit Sub-frame,

Destination: BS,

Attribute_List:

Channel Center Frequency,

Channel width,

Number of concatenated structures,

Two field structure

)

| Attribute | Contents |
| --- | --- |
| Channel Center Frequency | Channel Center Frequency of the replying BS |
| Channel width | Channel width of the replying BS, in units of 10kHz |
| Number of concatenated structures |  |
| Two field structure | { - Signature identifier (8bits) - Reduction of transmit power; 255 dB has the significance of a request to STOP using the sub-frame (8 bits) } |

When generated:

This primitive is generated following a C-CX-REQ with the same Action Type.

Effect of receipt:

The receiving BS shall respond with C-CX-ACK.

- C-CX-RSP (Action_Type = Frequency Channel Avoidance)

Function:

The BS responds to the RAIS request for Frequency Channel Avoidance.

The messages specify, as confirmation, the date and time at which the requested protection will take place.

Semantics:

C-CX-RSP

(

Operation_Type: Action,

Action Type: Frequency Channel Avoidance,

Destination: RAIS

Attribute_List:

Radio Application identifier,

Date,

Absolute time,

Long duration

)

| Attribute | Contents |
| --- | --- |
| Radio Application identifier | Radio Application identifier |
| Date | Date of the requested operation start |
| Absolute time | Hour/min/sec/millisecond of the requested operation start |
| Long duration | Duration of the requested operation |

When generated:

This primitive is generated following the RAIS request.

Effect of receipt

The receiving BS shall respond with C-CX-ACK.