| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Changes to Section 6.3.2.3.62 |  |
| Date Submitted | 2008-06-06 |  |
| Source(s) | John Sydor Communications Research Centre Ottawa, Canada | Voice: 1-613-998-2388 Fax: e-mail to : john.sydor@crc.ca |
| Re: | IEEE 802.16 Working Group Letter Ballot #29 |  |
| Abstract | Changes to Draft D6 to include additional information re:CCID_IND Message |  |
| Purpose | Discussion and accept. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Changes to Section 6.3.2.3.62

John Sydor

Communications Research Centre

Ottawa, Canada

Introduction: Additional information  regarding  the interfering BS’s  GPS location and height  is required for the CCID_IND message detailed in Section 6.3.2.3.62 of Ref 1. Additionally, the BS_RF_Sector_ID field of the message needs greater detail regarding the antenna system being use by the interfering BS.

REF 1: IEEE P802.16h/D6, May 2008

Proposed Changes:

Italic font is instructions to the editor

Blue Font underlined  are text additions

Red Font strikethrough are text deletions.

The following changes are to the text found on Page 14 and 15 of Ref 1. The changes are text additions to existing paragraphs and changes to Table 108aa. The Changes are shown below.

Page 14 Starting line7

DFS_LE_PWR_FRQ: This parameter is used to identify the types of interfering non-Wireless-

MAN-CX devices and provide information that may be specific to those particular devices. Up to 32 classes of devices are possible to identify. Radar information such as PPS and Pulse duration will be given in the Device Detection Specific field. The CCID is the identification number of this specific CCID_IND message and is used to tag this specific interference event. The number will be a rolling number from 1 to 65536.

RSSI report: The mean and standard deviation of the interfering signal reported by this message,

measured only for interfering signals that are valid Interference detection events and exceed the RSSI

threshold in the CCID_RES message. See Section 8.2.2, 8.3.9, 8.4.11 for details.

GPS_Loc: The GPS location of the BS

HGHT: The height of the BS above sea level in meters.

Make the following changes to the Table 108aa located on Page 1, Starting at Line 4.

Table 108aa---CCID_IND message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CCID_IND_Message_Foprmat(){ |  |  |
| Management Message Type=67 | 8 Bits |  |
| DFS_LE_PWR_FRQ | 32 Bits | Bit 0:Set to 1 for non-WirelessMAN-CX Bits1-5:Device Type Bits 6-15:Device detection Specific Bits 15-31:CCID |
| INT_BSD_Frq | 16 bits | Bits 0-7:The number of interference detection events Bits 8-15: CXCC repetition cycles. |
| BSID | 48 bits | Foreign Base Station ID; Set to 0 for Non- WirelessMAN-CX Interference |
| BS_RF_Sector_ID | 32 40 bits | bits 0-3: ID bits 4-7: gain (dBi),2dB resolution bits 8-15: azimuth, 2 degree resolution bits 16-23: 3dB aperture, 2 degrees resolution bites 24-25: polarization, with: 00 - vertical, 01- horizontal, 11: circular bit 26: beam-forming, 1=yes bits 27-31 reserved bits 27-30 no.of antenna elements bits 31-34 no.of possible beams bit 35 - Interference cancellation capability bit 36 - MIMO capability (which can imply interference cancellation to the same PHY cases) bits 37-39: reserved Set all to 0 for Non-WirelessMAN-CX Interference |
| BS EIRP | 8 bits | Nominal EIRP of interfering BS; Set to 0 for Non-WirelessMAN-CX Interference |
| CX_CMI_D(n) | 8 bits | Coexistence Messaging Interval ID In which WirelessMAN-CX interference detected. Set to 0 if interference event detected during silent period of sub-channel 1. Set to 1-3 if detected during CX_CMI_D(1-3); Set to 4 if detected during CX_CMI_D(4). |
| GPS Loc | 48 bits | The GPS location of the interfering Base Station 24 MSB for BS Lat 24 LSB for BS long |
| HGHT | 16 bits | The height of the interfering BS above sea level in meters |
| RSSI report | 16 bits | bit 0-7: RSSI Mean value in dBm bit 8-15: RSSI standard deviation in dB |
| TLV Encoded Information | Variable | TLV-specific |
| } |  |  |

End of Proposed Changes