| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Annex h_A in ASN.1 |  |
| Date Submitted | 2008-06-11 |  |
| Source(s) | Rina Nathaniel Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv, Israel | E-mail: rina.nathaniel@alvarion.com |
| Re: | LB 29c |  |
| Abstract |  |  |
| Purpose | [Description of what specific action is requested of the 802.16 Working Group or subgroup.] |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Annex h_A in ASN.1

Rina Nathaniel

Alvarion Ltd.

# Introduction

This contribution provides the content of Annex h_A in ASN.1.

# Text changes

Replace Table h_A with the following text:

Module-InfoDB DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

DeviceType ::= ENUMERATED {

bs (0),

nbs(1),

ss (1)

}

IPVersion ::= BIT STRING (SIZE (1)){

IPv4 (0),

IPv6 (1)

}

InetAddressIPv4 ::= OCTET STRING (SIZE  (4))

InetAddressIPv6 ::= OCTET STRING (SIZE (16))

InetAddress :: = CHOICE {

IPv4, InetAddressIPv4

IPv6, InetAddressIPv6

}

negStatus ::= BIT STRING SIZE(8){

Get-communication-in-the-IP-network(0),

Be-registered-in(1),

Registered-to(2),

Done-for-resource-sharing (3)

}

csiParam  :: = SEQUENCE {

Tcsi_start    OCTET STRING (SIZE (2)),

-- Starting point of the CSI in each frame,

-- which is counted in PSs/symbols.

Tcsi_duration OCTET STRING (SIZE (1)),

-- Time duration of each CSI interval,

-- which is counted in PSs/symbols.

CSI-Cycle     OCTET STRING (SIZE (1)),

-- CSI cycle in unit of CX-Frame

CSIN          OCTET STRING (SIZE (2)),

-- frame serial number of OCSI presented within the CX-Frame

StartCSI   ::= SEQUENCE {

-- Starting of CSI measurement

LengthCSI     OCTET STRING (SIZE (2)),

-- Length of CSI measurement

}

listedDevice :: = SEQUENCE {

Index        OCTET STRING (SIZE (2)),

DeviceID     OCTET STRING (SIZE (6))

}

modulation ::= ENUMERATED {OFDM(1), OFDMA(2)}

--0, 3 to 15 – reserved

ResID OCTET STRING (SIZE(2))

--Bits 15-8: Channel ID

--Bits  7-0: sub-frame ID

BS_PHYmode  ::= SEQUENCE {

Modulation modulation,

ChannelCenterFrequency OCTET STRING (SIZE (4)),

--  Channel Center Frequency, in 10kHz.

ChannelWidth           OCTET STRING (SIZE (4)),

-- in 10kHz

MasterSubFrameID       BIT STRING (SIZE (1)),

-- Sub-frame number:

-- Bit7: sub-frame structure supported

--     0 - not supported

--     1 - supported

-- Bit6: master sub-frame allocated

--     0 - do not have a master sub-frame

--     1 - have a master sub-frame

-- Bit5-3: number of sub-frames in frame structure

-- Bit2-1: the master sub-frame index this BS

NumOfALTCH          OCTET STRING (SIZE (1)),

-- The number of alternative channels to which this BS

-- can switch without interference.

ResourceIDforALTSF ::= SEQUENCE OF ResID (SIZE (1..NumOfALTCH))

}

----- Table h-A1

Bs-DB :: = SEQUENCE {

BSID           OCTET STRING (SIZE (6)),

IP-Version-map IPVersion,

BSIPAddr       InetAddress,   -- IP address of the BS

ConIPAddr      InetAddress,   -- contacting IP address

RTK            OCTET STRING (SIZE (2)),

-- Random Temporary Key

ExChNr         OCTET STRING (SIZE (2)),

-- Extended Channel Number

-- 2 byte base reference to frequency range or deployment

-- band. This reference maps to an absolute frequency value

BaseFrequency  OCTET STRING (SIZE (4)),

-- 4 byte specific channel number reference

ChSp           OCTET STRING (SIZE (2)),

-- 2 bytes channel spacing value (10kHz increments)

OCSI_ID        OCTET STRING (SIZE (1)),

-- CSIN of OCSI allocation on working channel

NegotiationStatus   negStatus,

CSI-Parameter  csiParam,

-- Coexistence Signaling Interval regulated by region/country

NCoNBRs        OCTET STRING (SIZE (1)),

-- The number of coexistence neighbors of this BS

ListOfNeighborBS ::= SEQUENCE OF listedDevice SIZE (1.. NCoNBRs)

-- Each Index in the list referrs to a BS in the neighborhood,

-- and points to a set of information described in Information

-- table for the systems inside the neighborhood or community

Profile  ::= SEQUENCE {

phy-mode BS_PHYmode,

MaxPower    OCTET STRING (SIZE (1)),

-- Maximum power in dBm

NumOfSS     OCTET STRING (SIZE (2)),

-- Number of registered SS on working channel

ListOfregisteredSS ::=

SEQUENCE OF listedDevice SIZE (1.. NumOfSS)

-- Each Index here is referring to a SS in this system,

-- and points to a set of information described in Information

-- table for the SSs inside the system containing this database

}

}

AntennaAzimut  OCTET STRING (SIZE (1)),

-- Antenna azimuth wrt True North, 2 degree resolution

AntennaBeamWidth  OCTET STRING (SIZE (1)),

-- Antenna beam width at -3dB point, 2 degree resolution.

VictimSS  ::= SEQUENCE {

SSID   OCTET STRING (SIZE (6)),

RSSI   OCTET STRING (SIZE (2))

}

-- 1byte RSSI mean

-- 1byte standard deviation

VictimSSInfo ::= SEQUENCE {

NumOfVictimSS OCTET STRING (SIZE(2)),

-- The number of SSs in this system which have reported that their interference

-- by this BS is beyond then acceptable interference threshold.

ListOfVictimSS ::= SEQUENCE OF VictimSS SIZE(1..NumOfVictimSS)

}

listedCoexistNeighbor ::= SEQUENCE {

BSID             OCTET STRING (SIZE (6)),

ChannelCenterFrequency  OCTET STRING (SIZE (4)),

-- Channel Center Frequency, in 10kHz.

ChannelWidth     OCTET STRING (SIZE (2)),

-- Channel width, in 10kHz

Flag             BIT STRING (SIZE(1))

-- Flag indicates this neighbor has one or more alternative channels.

}

NBS_PHYmode  ::= SEQUENCE {

ChannelCenterFrequency OCTET STRING (SIZE (4)),

--  Channel Center Frequency of this neighbor, in 10kHz.

ChannelWidth      OCTET STRING (SIZE (4)), -- in 10kHz

AltChFlag           BIT STRING (SIZE(1)),

-- Flag indicates this neighbor has one or more alternative channels.

MasterSubFrameID  BIT STRING (SIZE (8)),

--Sub-frame number:

-- Bit7: sub-frame structure supported

--     0 - not supported

--     1 - supported

-- Bit6: master sub-frame allocated

--     0 - do not have a master sub-frame

--     1 - have a master sub-frame

-- Bit5-3: number of sub-frames in frame structure

-- Bit2-1: the master sub-frame index this BS is using.

AltSubFrameFlag   BIT STRING (SIZE(1))

--Flag indicates this neighbor has one or more alternative sub-frame.

}

YesNo  BIT STRING (SIZE (1)){yes (0), no (1)}

associatedSS ::= SEQUENCE {

InterfSSID  OCTET STRING (SIZE(6)),

-- SS_ID of this SS causing interference to this BS

CMI-InterfOccurrence  OCTET STRING (SIZE(2)),

-- Number of instances where interference recorded.

RSSIofSS        OCTET STRING (SIZE(2)),

-- RSSI of this interfering SS

SSinterfRes    YesNo

-- Has the interference caused by this SS

-- been resolved by use of the CXP between this

-- BS and the foreign system?

}

ForBSListed SEQUENCE {

For_BSID   OCTET STRING (SIZE (6)),

-- BSID of this foreign BS

IP-Version-map IPVersion,

ForBSIPAddr       InetAddress,

-- IP address of this foreign BS

ForBSCMI-ID  OCTET STRING  (SIZE (1)),

-- CMI_ID of this foreign BS

NumOfForSS  OCTET STRING (SIZE (2)),

-- Number of SS associated with this foreign BS causing interference to this BS

ListOfSS SEQUENCE OF associatedSS SIZE ( 1..NumOfForSS )

-- All SSs associated with this foreign BS, which cause co-channel interference

}

---- Table h-A2

NeighborBs-DB :: = SEQUENCE {

Index          OCTET STRING (SIZE (2)),

BSID           OCTET STRING (SIZE (6)),

RTK            OCTET STRING (SIZE (2)),

-- Random Temporary Key

IP-Version-map IPVersion,

ConIPAddr      InetAddress,   -- contacting IP address

SectorID    OCTET STRING (SIZE (1)),

ExChNr         OCTET STRING (SIZE (2)),

-- Extended Channel Number

-- 2 byte base reference to frequency range or deployment

-- band. This reference maps to an absolute frequency value

BaseFrequency  OCTET STRING (SIZE (4)),

-- 4 byte specific channel number reference

ChSp           OCTET STRING (SIZE (2)),

-- 2 bytes channel spacing value (10kHz increments)

OCSI_ID        OCTET STRING (SIZE (1)),

-- CSIN of OCSI allocation on working channel

NegotiationStatus   negStatus,

CoexistNeighbor YesNo,

BSLocation     OCTET STRING (SIZE (6)),

-- GPS coordinates of this Base Station

BS_RF_antenna_sector_ID OCTET STRING (SIZE (1)),

-- Identifier of antenna creating this sector

BS_nominal_EIRP   OCTET STRING (SIZE(1)),

-- Nominal EIRP of this Base Station

BS_antenna_azimuth   AntennaAzimut,

-- Azimuth orientation of this Base Station's antenna

BS_antenna_beamwidth AntennaBeamWidth,

-- Azimuth Beam width of this BS's antenna

VictimSSDB  ::= CHOICE {  CoexistNeighbor  CHOICE {

yes  VictimSSInfo,

no   NULL}  },

NumOfCoexistNeighbor  OCTET STRING (SIZE (1)),

-- The number of coexistence neighbors of this BS

ListOfCoexistNeighbor ::=

SEQUENCE OF listedCoexistNeighbor SIZE (1.. NumOfCoexistNeighbor),

Profile  ::= SEQUENCE {

phy-mode NBS_PHYmode,

MaxPower    OCTET STRING (SIZE (1)),

-- Maximum power in dBm

NumOfSS     OCTET STRING (SIZE (2))

-- Number of registered SS on working channel

},

CMIused YesNo,

ListOfForBS ::= CHOICE {  CMIused   CHOICE {

yes  ForBSInfo,

no       NULL},

-- All Coexisting neighbor BS information. This is the list of foreign BS,

--which may be causing interference to this BS and its SS

ForBSInfo ::= SEQUENCE {

-- All Coexisting neighbor BS information. This is the list of foreign BS, which may be causing interference to this BS and its SS

NOfCoNeighbor    OCTET STRING (SIZE(2)),

--Number of coexistence neighbors

ForBSDataBase ::= SEQUENCE OF ForBSListed SIZE(1..NOfCoNeighbor)

}

}

}

DetectedBSinfo ::= SEQUENCE {

IP-Version-map IPVersion,

ConIPAddr   InetAddress,             -- IP address of the contacting reported by the SS

IBS_BSID    OCTET STRING (SIZE (6)),

-- The BSID reported by SS

RTK        OCTET STRING (SIZE (2)),

--RTK in the BS_NURBC reported by SS

Sector_ID   OCTET STRING (SIZE (1)), -- Reported by SS

FrameNumber OCTET STRING (SIZE (3)), -- Reported by SS

ErrorStatus ENUMERATED (no-error(0),

not-capable-decode-the-energy-pulse-symbol(1),

not-able-to-find-the-Start-of-Frame (2),

not-able-to-find-the-End-of-Frame  (3),

not-able-to-pass-the CRC-check-for-message(5)

--5 to 255 – reserved

}

DetectedBS  ::= SEQUENCE {

BSID   OCTET STRING (SIZE (6)),

BS_NURBCdetected    YesNo,

DetectedBSDataBase  CHOICE { BS_NURBCdetected CHOICE {

yes  DetectedBSinfo,

no   NULL}

}

ListOfSourceBS ::=SEQUENCE {

NumOfSourceBS OCTET STRING (SIZE (1)), -- The number of interference source of coexistence neighbor

DetectedBSDB  ::= SEQUENCE OF DetectedBS  SIZE (1..NumOfSourceBS),

RSSI          OCTET STRING (SIZE (2))

-- 1byte RSSI mean  1byte standard deviation

}

ForBSinfo   ::= SEQUENCE {

BSID               OCTET STRING (SIZE (6)),

-- BSID of this foreign BS as determined from its BSD

BSEIRP             OCTET STRING (SIZE (1)),

-- EIRP of this foreign BS as determined from its BSD

BSantennaSectorID  OCTET STRING (SIZE (1)),

-- Antenna sector ID of this foreign BS as per BSD

IP-Version-map     IPVersion,

BSNetworkAddress   InetAddress,

-- Network contact address of this foreign BS as per BSD

BSD-occurrence-ratio  OCTET STRING (SIZE (2)),

-- Defined as the ratio of demodulated foreign BSD messages (

-- first byte) to CXCC cycles (second byte). A metric indicating severity

-- of interference caused by this foreign co-channel BS.

InterfRes   YesNo,

-- 1 bit An indication that interference from this foreign BS has been resolved by the CXP.

BS-CMI-ID   OCTET STRING  (SIZE (1))

-- CMI_ID of this foreign BS

}

AssBSinfo ::= SEQUENCE {

BSID         OCTET STRING (SIZE (6)),

-- BSID to which this SS is associated

BS-RSSI     OCTET STRING (SIZE (2)),

-- Mean RSSI of BS downlink to which this SS is associated

BS-RSSI-Var OCTET STRING (SIZE (2)),

-- Variance of RSSI of downlink

BS-BER   OCTET STRING (SIZE (2)),

-- BER of downlink

Num-of-For-BS OCTET STRING (SIZE (2)),

-- Number of foreign BS this SS has detected via BSD

DBofForBS SEQUENCE OF ForBSinfo SIZE (1..Num-of-For-BS)

}

---- Table h-A3

Ss-DB:: = SEQUENCE {

Index          OCTET STRING (SIZE (2)),

SSID           OCTET STRING (SIZE (6)),

location       OCTET STRING (SIZE (6)), -- Optional ??

antennaBW    AntennaBeamWidth,

-- Beam width of this SS antenna

UL-EIRP       OCTET STRING (SIZE (1)),

-- Nominal  EIRP of this SS

InterfStatus   YesNo,

--Interfered by coexistence neighbor?

DBInterfSource ::= CHOICE { InterfStatus CHOICE {

yes      ListOfSourceBS,

no       NULL}},

CMIused YesNo,

DBofAssBS ::= CHOICE { CMIused CHOICE {

yes      AssBSinfo,

no       NULL}},

-- Data Base of BSs associated with this SS is

}

END

DataBase DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS DeviceType, Bs-DB, NeighborBs-DB, Ss-DB

FROM Module-InfoDB;

DataBaseStructure ::= CHOICE { DeviceType  CHOICE {

bs  Bs-DB,

nbs NeighborBs-DB,

ss  Ss-DB }

END