| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Consolidation of parameters and attributes in clause 11.22 (comment 673) |  |
| Date Submitted | 2008-11-05 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv | E-mail: mariana.goldhamer@alvarion.com |
| Re: |  |  |
| Abstract |  |  |
| Purpose | Actions: 1. Modification of ToC 2. Capture of the text in the SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Consolidation of parameters and attributes in clause 11.22 (comment 673)

Mariana Goldhamer

Alvarion Ltd.

# Introduction

This contribution addresses a number of SB comments related to the consolidation of attributes in clause 15.6 with the attribute table in clause 11.22.

The attributes in 11.22, including the attributes in Table 19, are:

Text changes

- CX inter-system communication encodings

The following Type-Length-Value (TLV) types may be present in CX-FWD-REQ, CX-FWD-RSP and CX-FWD-IND messages, depending on the coexistence action identified by their Action Code.

- TLV types for inter-system coexistence messages

| Type | Name | Length (bytes) | Value |
| --- | --- | --- | --- |
| 01 | BSID of Source BS | 6 | The BSID of the source base station |
| 02 | GPS coordinates | 3 | GPS coordinates |
| 03 | Network Address of source BS | 4 or 16 | Higher-layer protocol network address of the source base station length = 4 bytes if IPv4 is supported length = 16 bytes if IPv6 is supported |
| 04 | MAC Frame number | 1 | The 8 least-significant bits of the absolute MAC Frame number |
| 05 | Time shift from the MAC Frame start | 2 | Time shift from the MAC Frame start (in microseconds) |
| 06 | Duration of the time slot | 2 | Duration of the time slot (in microseconds) |
| 07 | Channel information | 2 | The channel information of the requested BS. Contains the modulation mode and alternative Channel Flag Bit0~Bit7: Modulation mode (0- reserved; 1- OFDM; 2- OFDMA; 3-15 reserved) Bit8~Bit15: alternative Channel Flag (0- no ALTCH; 1- have ALTCH) |
| 08 | Tx power | 1 | Tx power in dBm |
| 09 | Channel Center Frequency | 4 | Channel Center Frequency in 10kHz |
| 10 | Antenna Type | 1 | Transmit antenna type or receive antenna type.(1 - omni, 2 - directional) |
| 11 | Antenna Gain | 1 | Transmit antenna gain or receive antenna gain in dBi. Signed value |
| 12 | Acceptance indication | 1 | 1 - accepted 2 - rejected 3 - message dependent |
| 13 | Channel width | 2 | In 0.01MHz |
| 14 | Reduction of transmit power | 1 | Reduction of transmit power in dB |
| 15 | Gap DIUC/UIUC insertion | 1 | Gap DIUC/UIUC insertion 1 - for this system 2 - for systems which do not comply with this criteria |
| 16 | Number of distinct BS configurations | 1 | Number of distinct BS configurations < 20 |
| 17 | Number of SSs | 1 | Number of SSs < 200 |
| 18 | Two field structure | 2 | Each element in the structure is coded with 8 bits) |
| 19 | Radio signature type | 1 | Radio Signature type: 1 - RSS1 of this system, signature from SS1 (logical numbering) 2 - RSS2 of this system ... 199 - RSS199 of this system 200 - RBS1 of this system (BS in configuration 1, logical numbering) 201 - RBS2 of this system, configuration 2 ... 220 - RBS20 of this system, configuration 20 221 - maximum interference from all the CX_SS (compatible with WirelessMAN-CX), which operate inside the frequency block 222- maximum interference from all the CX_BS, which operate inside the frequency block 223 - maximum interference from all the CX_SS, which operate in a channel, which overlaps the operating channel of the requesting BS 224 - maximum interference from all the CX_BS, which operate in a channel, which overlaps the operating channel of the requesting BS 225 - maximum interference from all the NON-CX_SS, which operate in a channel, which overlaps the operating channel of the requesting BS 226 - maximum interference from all the NON-CX_BS, which operate in a channel, which overlaps the operating channel of the requesting BS 227 - maximum interference from all the CX_SS, which operate in the upper adjacent channel relative to the operating channel of the requesting BS 228 - maximum interference from all the CX_BS, which operate in the upper adjacent channel relative to the operating channel of the requesting BS 229 - maximum interference from all the CX_SS, which operate in the lower adjacent channel relative to the operating channel of the requesting BS 230 - maximum interference from all the CX_BS, which operate in the lower adjacent channel relative to the operating channel of the requesting BS |
| 20 | Renting_out_start_time | 4 | The starting time of the renting out period proposed by the offeror on that channel in absolute time. The absolute time is based on a UTC time stamp that has the format HH:MM:SS:ms |
| 21 | Renting_out_end_time | 4 | The ending time of the renting out period proposed by the offeror on that channel in absolute time. The absolute time is based on a UTC time stamp that has the format HH:MM:SS:ms. |
| 22 | T_renting_sub-frame | 2 | Total amount of time per master sub-frame rented out by the offeror BS (in milliseconds) |
| 23 | Minimum number of Credit Token (MNCT) | 6 | Minimum number of Credit Token (MNCT) (in number of credit tokens) |
| 24 | Requester_bid | 6 | Number of credit tokens per resource unit bidded by the requester in response to the offeror advertisement (in number of credit tokens) |
| 25 | Rented_resource_amount | 1 | Fraction (scalar) of T_renting_sub-frame the requester is interested in and bidding for. |
| 26 | Renting_in_start_time | 2 | Starting time for the period the requester is interested to rent in [Renting_out_start_time, Renting_out_end_time], and for which the requester's bid applies for (in milliseconds) |
| 27 | Renting_in_end_time | 2 | Ending time for the period the requester is interested to rent in [Renting_out_start_time, Renting_out_end_time], and for which the requester's bid applies for (in milliseconds) |
| 28 | Renting_sub-frame_start_time | 2 | This field specifies the starting time for transmission by the selected requester within T_renting_sub-frame (in milliseconds) |
| 29 | Renting_sub-frame_end_time | 2 | This field specifies the ending time for transmission by the selected requester within T_renting_sub-frame (in milliseconds) |
| 30 | Acceptation_Bit_Flag (ABF) | 1 | This flag indicates that the requester accepts the grant at the proposed clearing price. |
| 31 | LC | 1 | List of other channels (frequency domain) proposed by the offeror BS for renting |
| 32 | Coexistence_community_BSID | 24 | List of BSIDs for the systems belonging to the community of the offeror for CT-CXP |
| 33 | Adjacent Channel Leakage Ratio (ACLR) | 1 | Ratio of the transmitter power to the leakage power in the adjacent channel (in dB) |
| 34 | Number of concatenated structures | 1 | Number of structures to be listed consecutively |
| 35 | ID of the forwarding SS | 6 | ID of the forwarding SS |
| 36 | Notification Bit Flag (NBF) | 1 | This flag indicates whether the forwarding SS is selected to complete the CT-CXP operations: 1: forwarding SS is selected 0: forwarding SS is not selected |
| 37 | BSID of destination BS | 6 | BSID of destination BS |
| 38 | Network address of destination BS | variable | Higher-layer protocol network address of the destination base station length = 4 bytes if IPv4 is supported length = 16 bytes if IPv6 is supported |
| 39 | EIRP | 1 | The EIRP of the BS or SS (in signed units of 1 dBm) |
| 40 | GPS_LOC | 6 | The GPS location of the Base Station or SS. 24 MSB for BS Lat 24 LSB for BS long |
| 41 | HGHT | 2 | Height of BS or SS antenna above sea level (in meters). |
| 42 | RF_Sector_ID | 5 | The RF antenna sector ID is used to identify the RF transmitting antenna at the base station or SS where multiple RF antennas may be used or a Beam Forming AAS antenna is used. bits 0-3: ID bits 4-7: gain (dBi),2dB resolution bits 8-15: azimuth, 2 degree resolution bits 16-23: 3dB aperture, 2 degrees resolution bites 24-25: polarization, with: 00 - vertical, 01- horizontal, 11: circular bit 26: beam-forming, 1=yes bits 27-30 no.of antenna elements bits 31-34 no.of possible beams bit 35 - Interference cancellation capability bit 36 - MIMO capability (which can imply interference cancellation to the same PHY cases) bits 37-39: reserved Note: In the case of beamforming the antenna gain refers to the gain of a single beam The azimuth and 3dB aperture refers to the whole sector covered Beam directions will be considered as uniformly spaced within the sector Beamwidth will be approximated as the sector beamwidth divided by the number of antennas. A "1" in either MIMO capability or Interference cancellation capability would mean that the BS/SS is able to devote antenna resources to cancel external interference. |
| 43 | Type of Interfering Device: | 1 | Type of interfering device. Bit 0: Set to 1 for a non-WirelessMAN-CX interfering source. Bits 1-5: Device Type Bits 6-15: Device Detection specific |
| 44 | Inhibition Period | 2 | Number of CXCC cycles that the interfered-with device has been inhibited prior to sending an interference report. |
| 45 | Interference Threshold | 1 | Threshold in dBm for a valid interference event for this device. |
| 46 | Interference Incidence frequency | 2 | Number of interference events per specified CXCC cycles, from indicated interference sources sustained by the device |
| 47 | RSSI Report | 2 | Mean and standard deviation of RSSI. |
| 48 | CX_CMI | 1 | CMI intervals in which interference detections occur for the device. |
| 49 | Interfering Device Identity | 4 | The SSID or BSID of the interfering device. |
| 50 | Interference Victim Device Identity | 4 | The BSID or SSID of victim device(s) |
| 51 | CCID | 2 | The Interference report number for which this message applies |
| 52 | Inhibition Period Set | 2 | Inhibits the interference report of the device by the specified number of CXCC cycles |
| 53 | Interference Threshold Set | 2 | Set the interference threshold of the device to the specified level. |
| 54 | RTK | 2 | Real time key for basic connectivity creation using BS_NURBC. |
| 55 | Interference Resolution Status | 1 | Resolution of interference to which stated CCID applies: resolved/tolerated/pending |
| 56 | Sub-Frame ID | 1 | The sub-frame ID in the CX-frame, 0,1,2,3; the sub-frames are numbered separately for DL and UL |
| 57 | Switching Acknowledge | 1 | The acknowledgement of a master sub-frame switching request. 0: rejection for fail in switching; 1: succeeded in switching |
| 58 | Rolling back indication | 1 | Rolling back indication. 0: switch to one of the alternative channels; 1: switch back to the channel before the last channel switching request |
| 59 | OCSN | 1 | OCSN of the occupying OCSI |
| 60 | Backoff request | 1 | OCSI backoff request. 1- start backoff request 0- end of backoff request |
| 61 | Response indication | 1 | OCSI backoff response. 01- refuse to backoff 00- refuse to end the backoff 11- notification of acceptance and backoff begin 10- notification of acceptance and backoff end because of timer and counter having run out |
| 62 | Radio signature identifier | 1 | Radio signature identifier |
| 63 | Offset from the start of the signature slot | 1 | Offset from the start of the signature slot (in microseconds) |
|  | MAC Frame duration | 1 | The 802.16 code for MAC frame duration |
|  | Maximum coverage | 2 | In 100m increments |
|  | Channel Center Frequency of the receive channel | 4 | in 10kHz |
|  | Type | 1 | Message specific |
|  | Negotiation_Mode_Bit_Flag (NMBF) | 1 | Scalar – needs definition! |
|  | Master sub-frame ID | 1 | The ID on the Master sub-frame in the CX-Frame: 0,1,2 |
|  | Start_negotiation_time | 2 | in millisecond |
|  | End_negotiation_time | 2 | in millisecond |
|  | Pricing_Bit_Flag (PBF) | 1 | Scalar |
|  | Minimal_payoff | 6 | scalar |
|  | Maximal_payoff | 6 | scalar |
|  | Requester_bid_update | 6 | in number of credit tokens |
|  | Resource_Granting_Bit_Flag (RGBF) | 1 | scalar |
|  | Clearing_price | 6 | in number of credit tokens |
| 48 | Radio Application Identifier (guidance from ECC Decision 25) | 8 | ASCII coding: AGA - Air Ground Air BSS - Broadcasting Satellite Service CRS - Central Radio Station DCS 1800 - Digital Communication System DECT - Digital Enhanced Cordless Telecommunication System DME - Distance Measuring Equipment DSI - Detailed Spectrum Investigation DVB-T - Terrestrial Digital Video Broadcasting EESS - Earth Exploration-Satellite Service EGSM - Extended GSM ENG - Electronic News Gathering EPIRB - Emergency Position-Indicating Radiobeacon ERMES - European Radio Messaging System FSS - Fixed Satellite Service FWA - Fixed Wireless Access GMDSS - Global Maritime Distress and Safety System GNSS - Global Navigation Satellite System GSM - Global System for Mobile Communications HAPS - High Altitude Platform Systems HDTV - High Definition Television HIPERLAN - High Performance Radio Local Area Network IBCN - Integrated Broadband Communications Network ILS - Instrument Landing System IMT-2000- International Mobile Telecommunications ISM - Industrial, Scientific and Medical applications JTIDS - Joint Tactical Information Distribution System MIDS - Multifunctional Information Distribution System MLS - Microwave Landing System MSI - Maritime Safety Information MSS - Mobile Satellite Service MWS - Multimedia Wireless Systems NGSO - Non-geostationary Satellite Orbit OB - Outside Broadcasting OR - Off-Route PAMR - Public Access Mobile Radio (PMR) PMR - Professional Mobile Radio, Private Mobile Radio R - Route RA - Radio Astronomy SAB - Services Ancillary to Broadcasting SAP - Services Ancillary to Programming S-PCS - Satellite Personal Communication System SRD - Short Range Device TETRA - Terrestrial Trunked Radio RFID - Radio Frequency Identification systems RLAN - Radio Local Area Network RTTT - Road Transport & Traffic Telematics SNG - Satellite News Gathering SRD - Short Range Devices SSR - Secondary Surveillance Radar T-DAB - Terrestrial Digital Audio Broadcasting TACAN - Tactical Air Navigation System VLBI - Very Long Baseline Interferometry (Radio Astronomy) VOR - VHF Omni-directional Range VTS - Vessel Traffic System (radar) VSAT - Very Small Aperture Terminal |
| 49 | Radius of protection area | 1 | In km |
| 50 | Date | 3 | Format: day (5 bits): month (4 bits):year(8bits) - decimal digits, day being most significant |
| 51 | Absolute time | 4 | Format: hour(5bits): minutes (6 bits): seconds (6 bits) - decimal digits, hour being most significant |
| 52 | Antenna direction | 2 | In degrees, clock-wise, reference North (180 for South) |
| 53 | Long duration | 3 | In minutes, hex number |
|  | ACLR second | 1 | Ratio of the transmitter power to the leakage power in the second adjacent channel, in dB |
| 57 | Number of TLVs in a structure | 1 | Used in conjunction with the Number of structures |
| 66 | SSID | 6 |  |
| 68 | SS_RF_Sector_ID | 5 | bits 0-3: ID bits 4-7: gain (dBi), 2dB resolution bits 8-15: azimuth, 2 degree resolution bits 16-23: 3dB aperture, 2 degrees resolution bits 24-25: polarization, with: 00- vertical, 01- horizontal, 11- circular bit 26: beam-forming, 1=yes bits 27-30 no. of antenna elements bits 31-34 no. of possible beams bit 35 - Interference cancellation capability bit 36 - MIMO capability (which can imply interference cancellation to the same PHY cases) bits 37-39: reserved Note: In the case of beamforming the antenna gain refers to the gain of a single beam The azimuth and 3dB aperture refers to the whole sector covered Beam directions will be considered as uniformly spaced within the sector Beamwidth will be approximated as the sector beamwidth divided by the number of antennas A "1" in either MIMO capability or Interference cancellation capability would mean that the BS/SS is able to devote antenna resources to cancel external interference. |
|  | DFS_LE_PWR_FRQ | 4 | Bit 0: Set to 1 for non-WirelessMAN-CX Bits1-5: Device Type Bits 6-15: Device detection specific Bits 15-31: CCID |
|  | CX contacting network address IPv4 | 4 | to support for example IPv4 |
|  | CX contacting network address IPv6 | 16 | to support for example IPv6 |
|  | Coordinated Coexistence type | 1 | 1: CX-Frame 2. CX-Frame and CX-CBP |
|  | Latitude | 5 | See clause 11.21.1 Absolute Position TLV (Long Format) |
|  | Longitude | 5 | See clause 11.21.1 Absolute Position TLV (Long Format) |
|  | Altitude | 5 | See clause 11.21.1 Absolute Position TLV (Long Format) |
| 82 | Channel width of the receive channel | 2 | In 0.01MHz |

End text changes

The attributes in clause 15.6 are listed below; if they are identical with those in clause 11.22, they are simply deleted. If they have the same name but are not identical, there is a comment added in the special comment column. Some of them were added to 11.22 without notice in the table below (starting with Radio application Identifier).

It is still needed the revision of the primitive in clause 15 to make sure that the same names are used in both 11.22 and list of attributes inside the primitives.

Text changes

The list of possible attributes is contained in clause 11.22.

Attrib.

| Type | Parameter Description | Length (bytes) | Comment | Special comment – not for insertion in the draft |
| --- | --- | --- | --- | --- |
|  |  |  |  | Added to 11.22 |
|  |  |  |  | Appears only in the Annex; needs deletion from the Annex |
|  |  |  |  | 15.6.5.2 M-CX-REQ (Action_Type = Add neighbor) is not fully consistent with definition in 11.22 |
|  |  |  |  | Added to 11.22 |
|  |  |  |  | Added to 11.22 |
|  |  |  |  | Added to 11.22 |
|  |  |  |  | Added to 11.22 |
| - |  |  |  |  |
|  |  |  |  | Appears as “Network address of the destination BS” |
|  |  |  |  | Appears as “EIRP” |
|  |  |  |  | Appears as “GPS_LOC” |
|  |  |  |  | Appears as “HGHT” |
|  |  |  |  | Appears as “EIRP” |
|  |  |  |  | Appears as “GPS_LOC” |
|  |  |  |  | Appears as “HGHT” |
|  |  |  |  | Appears as “Master Sub-frame ID” |

End text changes