| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On fixing the parameters and attributes of coexistence messages |  |
| Date Submitted | 2008-12-25 |  |
| Source(s) | Harry Bims 1 Infinite Loop Cupertino, CA 95014 | E-mail: harrybims@me.com |
| Re: |  |  |
| Abstract | Proposed changes to 11.24, 15.5.3, and 15.6 |  |
| Purpose | Cleanup of the attributes and parameter names and defintions, and harmonization with 802.16j and 802.16Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# On fixing the parameters and attributes of coexistence messages

Harry Bims

Apple, Inc.

# Overview

This contribution addresses several problems with parameter and attribute names and definitions.  It also harmonizes them with 802.16j and 802.16Rev2.

Specific editorial changes

The following is a legend of text formats that is used in the remainder of this contribution.

Blue text represents specific editorial additions.

Red strikethrough text is to be deleted.

Black text is text already in the draft.

Bold italic text is editorial instructions to the editor.

Modify the following text accordingly:

# Text proposal for clause 15.5.3.1

Modify the text:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=BSD) message.

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| Network address of Source BS | The network address of the Base Station emitting the BSD. |
| BS EIRP | The BS EIRP is signed in units of 1 dBm. The EIRP at which the BSD message was sent; usually the maximum allowable EIRP for the operation of this Base Station. |
| BSID of the source BS | Base Station Identifier of the Base Station emitting transmitting the BSD. |
| RF_Sector_ID | The RF antenna sector ID is used to identify the RF transmitting antenna at the base station where multiple RF antennas may be used or a Beam Forming AAS antenna is used. This contains information about: - the ID of RF transmitting antenna at the base station - the antenna gain (dBi) - the azimuth direction of transmission relative to the true North - the antenna aperture at 3dB attenuation - the antenna polarization (Vertical, Horizontal, Cross, Circular) |
| GPS_LOC | The GPS location of the Base Station emitting transmitting the BSD |
| HGHT | The height of the antenna emitting transmitting the BSD. |

# Text proposal for clause 15.5.3.2

Replace the text “A CX-FWD-REQ message having SSURF as its Action Code should include the following parameters:” with:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=SSURF) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| SSID of the forwarding SS | Subscriber station MAC (Media Access Control) identifier, in the context of this message, identifies the transmitting SS. |
| BSID of the source BS | Base Station Identifier of the Base Station emitting transmitting the BSD. |
| EIRP | The SS EIRP is signed in units of 1 dBm. The EIRP at which the SSURF message was sent; usually the maximum allowable EIRP for the operation of this station. |
| RF_Sector_ID | The RF antenna sector ID is used to identify the RF transmitting antenna at the subscriber station. It contains information about: - the ID of RF transmitting antenna at the subscriber station - the antenna gain (dBi) - the azimuth direction of transmission relative to the true North - the antenna aperture at 3dB attenuation - the antenna polarization (Vertical, Horizontal, Cross, Circular) |
| Network address of Source BS | The network address of the serving base station. |
| GPS_LOC | The GPS location of the Base Station emitting transmitting the SSURF |
| HGHT | The height of the SS above sea level in meters. |

# Text proposal for clause 15.5.3.3

Replace the text “A CX-FWD-REQ message having CT-CX-ADV-REQ as its Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=CT-CX-ADV-REQ) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of the source BS | BSID of the offeror |
| T_renting_sub-frame | Total amount of time per Master sub-frame rented out by the offeror BS. |
| Renting_out_start_time | The starting time of the renting out period proposed by the offeror on that channel, expressed in absolute time. The absolute time is based on a UTC time stamp that follows the HH:MM:SS:ms format. |
| Renting_out_end_time | The ending time of the renting out period proposed by the offeror on that channel, expressed in absolute time. The absolute time is based on a UTC time stamp that follows the HH:MM:SS:ms format. |
| MNCT | Minimum number of credit tokens per RRU required per requester's bid. |
| LC | List of other channels (frequency domain) proposed by the offeror BS for renting. |
| SSID of the forwarding SS | SSID of the forwarding SS |

# Text proposal for clause 15.5.3.4

Replace the text “A CX-FWD-RSP message having CT-CX-ADV-RSP as its Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-RSP (Action type Code=CT-CX-ADV-RSP) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of the source BS | BSID of the requester BS. |
| BSID of the destination BS | BSID of the offeror BS. |
| Requester_bid | Number of credit tokens per resource unit bidded by the requester in response to the offeror advertisement. |
| Rented_resource_amount | Fraction (scalar) of T_renting_sub-frame the requester is interested in and bidding for. |
| Renting_in_start_time | Starting time of the period from which the requester is interested to rent in within [Renting_out_start_time, Renting_out_end_time], and for which the requester's bid applies for. |
| Renting_in_end_time | Ending time of the period the requester is interested to rent in within [Renting_out_start_time, Renting_out_end_time], and for which the requester's bid applies for. |
| SSID of the forwarding SS | SSID of the forwarding SS |

# Text proposal for clause 15.5.3.5

Replace the text “A CX-FWD-REQ message having CT-CX-RA-REQ as its Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=CT-CX-RA-REQ) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of the source BS | BSID of the offeror BS |
| BSID of the destination BS | BSID of the requester BS associated with the forwarding SS |
| Resource_Granting_Bit_Flag (RGBF) | This flag indicates whether or not the offeror supplies the resource requested by the requester. |
| Renting_sub-frame_start_time | This field is useful only when RGBF = 1. This field specifies the starting time of transmission of the selected requester within T_renting_sub-frame. |
| Renting_sub-frame_end_time | This field is useful only when RGBF = 1. This field specifies the ending time of transmission of the selected requester within T_renting_sub-frame. |
| Coexistence_community_BSID | This field is useful only when RGBF = 1. This field specifies the BSIDs of the BS belonging to the community of the offeror. This information is transmitted to the requester so that the granted requester can inform these systems about the Master usage change within this specific time [Renting_sub-frame_start_time, Renting_sub-frame_end_time] of the frame and for a period starting at Renting_in_start_time and ending at Renting_in_end_time. |

# Text proposal for clause 15.5.3.6

Replace the text “A CX-FWD-RSP message having CT-CX-RA-RSP as its Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-RSP (Action type Code=CT-CX-RA-RSP) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of the source BS | BSID of the requester BS (associated with the forwarding SS when using over the air inter-system communications). |
| BSID of the destination BS | BSID of the offeror BS. |
| Acceptation_Bit_Flag (ABF) | When RGBF =1, this flag indicates that the requester accepts the grant at the proposed clearing price. |
| SSID of the source forwarding SS | SSID of the forwarding SS |

# Text proposal for clause 15.5.3.7

Replace the text “A CX-REQ message having CT-CX-ADPD as its Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=CT-CX-ADPD) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of the source BS | BSID of the requester BS. |
| SSID of the forwarding SS | SSID of the forwarding SS. |
| Renting_in_start_time | Starting time of the period from which the requester BS is interested to rent in some resources. For values received before this specified time, the forwarding SS is not allowed to report CT CX-ADV-REQ message content to its home BS (requester). This starting time is identified by a UTC time stamp that follows the HH:MM:SS:ms format, and occurs after the transmission of the message. |
| Renting_in_end_time | Ending time of the period the requester BS is interested to rent in some resources. For values received before this specified time, the forwarding SS is not allowed to report CT CX-ADV-REQ message content to its home BS (requester). This ending time is identified by a UTC time stamp that follows the HH:MM:SS:ms format, and occurs after the transmission of the message. |
| MNCT | Maximum admissible number of credit tokens per RRU the requester BS will provide to get the radio resources proposed by the offeror BS. Above this number of tokens, the forwarding SS is not allowed to report CT CX Advertisement Request message content to this home BS (requester). |

# Text proposal for clause 15.5.3.8

Replace the text “A CX-FWD-REQ message having CT-CX-ACK as its Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=CT-CX-ACK) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of the source BS | BSID of the requester BS. |
| SSID of the forwarding SS | SSID of the forwarding SS. |
| BSID of the destination BS | BSID of the offeror BS. |

# Text proposal for clause 15.5.3.9

Replace the text “A CX-FWD-RSP message having CT-CX-NTF as its Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-RSP (Action type Code=CT-CX-NTF) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of the source BS | BSID of the offeror BS. |
| SSID of the forwarding SS | SSID of the forwarding SS associated with the destination BS (requester). |
| Notification Bit Flag (NBF) | This flag indicates whether the forwarding SS is selected to complete the CT-CXP operations. |

# Text proposal for clause 15.5.3.10

Replace the text “A CX-FWD-REQ message having Add Neighbor Request as Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=Add Neighbor Request) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| Channel Center Frequency | The center frequency of the requesting BS in kHz. |
| Channel Width | The channel width of the requesting BS in kHz. |
| Channel information | The channel information of the requesting BS, including modulation mode, and alternative Channel Flag. |
| RTK | Real time key for basic connectivity creation using BS_NURBC. |

# Text proposal for clause 15.5.3.14

Replace the text “The message CX-FWD-REQ with Action Code = Get Parameter for Radio Signature Request shall use the following parameter:” with:

The following parameter shall be included in the CX-FWD-REQ (Action type Code=Get Parameter for Radio Signature) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of the destination BS | The BSID of the destination BS. |

# Text proposal for clause 15.5.3.15

Modify the following text:

The following parameters may shall be included in this message:

Modify the title row of the table:

| TLV Parameter Name | Notes |
| --- | --- |

# Text proposal for clause 15.5.3.16

Replace the text “When a specific BS is requested to be inserted in a GAP DIUC, the parameters specifying the transmission of the Radio Signature shall not be sent.  The following parameters may be included in this message.” with:

The following parameters shall be included in this message with one exception.  When this message requests that a BS insert a Gap DIUC/UIUC interval, the parameters specifying the transmission of the Radio Signature shall not be included in the message.

Modify the following rows of the table:

| TLV Parameter’s Name | Notes |
| --- | --- |
| Channel Center Frequency(ChannelCenterFrequency) | Channel Center Frequency of the requesting BS, for sending the Radio signature (see 8.5.1) |
| Number of concatenated signaturesstructures | The number, N, of concatenated radio signatures in a time-slot = N. Each radio signature is listed in a Two field structure TLV in this primitive. |
| Radio signature typeTwo field structure | List with N values This TLV shall follow immediately after the “Number of concatenated structures” TLV, which indicates the number, N, of radio signature types in the message { - Offset from the start of the signature slot, in microseconds (8 bits) - Radio Signature type (8 bits) } |

# Text proposal for clause 15.5.3.17

Modify the following text:

The following parameters may shall be included in this message:

Modify the following rows of the table:

| TLV Parameter’s Name | Notes |
| --- | --- |
| Number of concatenated signaturesstructures | The number, N, of two field structures in the listconcatenated radio signatures in a time-slot. Each radio signature is listed in a Two field structure TLV in this primitive. |

# Text proposal for clause 15.5.3.18

Modify the following text:

The following parameters may shall be included in this message:

Modify the following rows of the table:

| TLV Parameter’s Name | Notes |
| --- | --- |
| Channel Center Frequency(ChannelCenterFrequency) | Channel Center Frequency of the requesting BS, for sending the Radio signature (see 8.5.1) |

# Text proposal for clause 15.5.3.19

Modify the following text:

The following parameters may shall be included in this message:

Modify the following rows of the table:

| TLV Parameter’s Name | Notes |
| --- | --- |
| Channel Center Frequency(ChannelCenterFrequency) | Channel identifier of the responding OBS |

# Text proposal for clause 15.5.3.20

Modify the following text:

The following parameters may shall be included in this message:

Modify the following row of the table:

| TLV Parameter’s Name | Notes |
| --- | --- |

# Text proposal for clause 15.5.3.21

Modify the following text:

The following parameters may shall be included in this message:

Modify the following row of the table:

| TLV Parameter’s Name | Notes |
| --- | --- |

# Text proposal for clause 15.5.3.22

Replace the text “A CX-FWD-REQ message having CMI Interference Indication as Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=CMI Interference Indication) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| Interference Victim Device Identity | BSID or SSID of victim system devices. |
| Type of Interfering Device | (1) Type or identity of non-WirelessMAN-CX interfering source. Or (2) BSID or SSID of interfering WirelessMAN-CX device. |
| Inhibition Period | Number of CXCC cycles foreign device has been inhibited prior to sending an interference report. |
| Interference Threshold | Threshold (in dBm) for a valid interference event for this device. |
| RSSI Report | Mean RSSI level and standard deviation of interference of each interference source. |
| CX_CMI | CMI intervals in which interference detections occur for the device. |
| RF Sector IDRF_Sector_ID | Radiation pattern characteristics of Iinterfering systems’ BS or SS |
| EIRP | EIRP of the interfering system’s device |
| Interfering Device Identity | The SSID or BSID of the interfering device. |
| Network Address | Network address of the system to which this interfering device is associated. |
| GPS_LOC | GPS_LOC of interfering system’s BS |
| HeightHGHT | Height of BS of interfering system |
| CCID | Interference report number relating to the specific interference event. |

# Text proposal for clause 15.5.3.23

Replace the text “A CX-FWD-RSP message having CMI Interference Resolution as its Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-RSP (Action type Code=CMI Interference Resolution) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| Interference Victim Device Identity | BSID or SSID of victim device(s) |
| CCID | Interference report number to which this message applies |
| Inhibition Period Set | Inhibit interference report of device by given number of CXCC cycles |
| Interference Threshold Set | Set interference threshold of device to given level. |
| Interference Incidence count cycles | The number of CXCC cycles in which there was an interference incidence. |
| Interference Resolution Status | Resolution of interference to which stated CCID applies: resolved/tolerated/pending |

# Text proposal for clause 15.5.3.24

Replace the text “A CX-FWD-REQ message having Channel Switch Request as Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=Channel Switch Request) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of source BS | The requesting BS identifier |
| BSID of destination BS | BS identifier of the requested BS. |
| Channel center frequency | Channel Center Frequency of the requested BS. |
| Channel Width | Channel Width of the requested BS. |
| Rolling back indication | Rolling back indication. |
| MAC Frame Number | Frame sequence number to switch channel. |

# Text proposal for clause 15.5.3.25

Replace the text “A CX-FWD-RSP message having Channel Switch Response as Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-RSP (Action type Code=Channel Switch Response) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of source BS | The requesting BS identifier. |
| BSID of destination BS | BS identifier of the requested BS. |
| Switch Acknowledge | The acknowledge for the channel switch request. |
| Channel Center Frequency | Channel Center Frequency of the requested BS will switch to. |
| Channel Width | Channel Width of the requested BS. |
| MAC Frame Number | Frame sequence number of the channel switching. |

# Text proposal for clause 15.5.3.26

Replace the text “A CX-FWD-REQ message having Master Sub-Frame Switch Request as Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=Master Sub-Frame Switch Request) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of source BS | The requesting BS identifier |
| BSID of destination BS | BS identifier of the requested BS. |
| Channel Center Frequency | The requested BS should switch its channel to this frequency. |
| Channel Width | Channel Width of the requested BS. |
| Sub-Frame ID | The current Master sub-frame ID of the requested BS. |
| Rolling back indication | Rolling back indication. |
| MAC Frame Number | Frame sequence number to switch channel. |

# Text proposal for clause 15.5.3.27

Replace the text “A CX-FWD-RSP message having Master Sub-Frame Switch Response as Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-RSP (Action type Code=Master Sub-Frame Switch Response) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of source BS | The requesting BS identifier. |
| BSID of destination BS | BS identifier of the requested BS. |
| Switching Acknowledge | The acknowledge of Master sub-frame switching request. |
| Channel Center Frequency | Target Channel Center Frequency of the requested BS. |
| Channel Width | Target Channel Width of the requested BS. |
| Sub-Frame ID | The requested BS should switch its Master sub-frame to the sub-frame with this sub-frame ID value. |
| MAC Frame Number | MAC Frame sequence number of the Master sub-frame switching. |

# Text proposal for clause 15.5.3.28

Replace the text “A CX-FWD-REQ message having OCSI Backoff Request as Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-REQ (Action type Code=OCSI Backoff Request) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of source BS | The requesting BS identifier. |
| BSID of destination BS | BS identifier of the requested BS. |
| OCSN | OCSN of the occupying OCSI |
| Backoff Request | The type of backoff request. |

# Text proposal for clause 15.5.3.29

Replace the text “A CX-FWD-RSP message having OCSI Backoff Response as Action Code shall include the following parameters:” with:

The following parameters shall be included in the CX-FWD-RSP (Action type Code=OCSI Backoff Response) message:

Modify the immediately following text by placing the list of parameters for the message in a table of TLV Parameters as shown below:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of source BS | The requesting BS identifier. |
| BSID of destination BS | BS identifier of the requested BS. |
| OCSN | OCSN of the occupying OCSI |
| Response indication | OCSI backoff response indication. |

# Text proposal for clause 15.6.1.2

Modify the following text:

Channel Center Frequency (ChannelCenterFrequency),

Number of concatenated signaturesstructures,

Modify the following rows of the table:

| Attribute | Contents |
| --- | --- |
| Channel Center Frequency(ChannelCenterFrequency) | Channel Center Frequency of the requesting BS, for sending the Radio signature (see 8.5.1) |
| Number of concatenated signaturesstructures | The number, N, of concatenated radio signatures in a time-slot = N. Each radio signature is listed in a Two field structure TLV in this primitive. |
| Radio signature typeTwo field structure | List with N values This TLV shall follow immediately after the “Number of concatenated structures” TLV, which indicates the number, N, of radio signature types in the message { - Offset from the start of the signature slot, in microseconds (8 bits) - Radio Signature type (8 bits) } |

# Text proposal for clause 15.6.2.2

Modify the following text:

Number of concatenated signaturesstructures,

Modify the following rows of the table:

| Attribute | Contents |
| --- | --- |
| Channel Center Frequency(ChannelCenterFrequency) | Channel Center Frequency of the requesting BS, for sending the Radio signature (see 8.5.1) |
| Number of concatenated signaturesstructures | The number, N, of concatenated radio signatures in a time-slot = N. Each radio signature is listed in a Two field structure TLV in this primitive. |
| Radio signature typeTwo field structure | List with N values This TLV shall follow immediately after the “Number of concatenated structures” TLV, which indicates the number, N, of radio signature types in the message { - Offset from the start of the signature slot, in microseconds (8 bits) - Radio Signature type (8 bits) } |

# Text proposal for clause 15.6.2.2

Modify the following rows of the table:

| Attribute | Contents |
| --- | --- |
| Interference Incidence count cyclesInhibition Period | Specify number of CXCC cycles for interference incidence count purposeNumber of CXCC cycles that the interfered-with device has been inhibited prior to sending an interference report. |

# Text proposal for clause 11.24

Modify the following rows of the table:

Table 612a—TLV types for inter-system coexistence messages

| Type | Name | Length (bytes) | Value |
| --- | --- | --- | --- |
| 83 | FSN | 2 | Frame sequence number of the channel switching |

# Text proposal for clause 15.6.2.11

Modify the following text:

Target Channel Center Frequency,

Target sub-frame ID,

Channel Center Frequency (ChannelCenterFrequency),

Number of concatenated signaturesstructures,

Modify the following rows of the table:

| Attribute | Contents |
| --- | --- |
| Target Channel Center Frequency (for new Master sub-frame) | target Channel Center Frequency of the requested BS |
| Target sub-frame ID (for new Master sub-frame) | The requested BS should switch its Master sub-frame to the sub-frame with this sub-frame ID value. |

# Text proposal for clause 15.6.5.2

Modify the following text:

CX Contact Network address

Modify the following rows of the table:

| Attribute | Contents |
| --- | --- |
| CX Contact Network address | The IPv4/IPv6 address of the requested BS or the agent of the requested BS. |

# Text proposal for clause 15.6.5.3

Modify the following text:

CX Contact Network address

Modify the following rows of the table:

| Attribute | Contents |
| --- | --- |
| CX Contact Network address | The IPv4/IPv6 address of the requested BS or the agent of the requested BS. |

# Text proposal for clause 15.6.6.1

Modify the following text:

DL Master sub-frameMaster sub-frame ID

Modify the following rows of the table:

| Attribute | Contents |
| --- | --- |
| DL Master sub-frameMaster sub-frame ID | Indicator of the MAC Frame in CX-Frame |