| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonization of DIUC and UIUC values with IEEE P802.16Rev2/D6 |  |
| Date Submitted | 2008-12-25 |  |
| Source(s) | Harry Bims Apple, Inc. 1 Infinite Way Cupertino, CA 95014 | Voice: 1-650-283-4174 E-mail: harrybims@me.com |
| Re: | Comment 663 in IEEE 802.16-08/047r3 |  |
| Abstract | This contribution contains proposed editorial changes that harmonize DIUC and UIUC |  |
| Purpose | To instruct the editor to make the proposed changes to IEEE P802.16h/D8 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Harmonization of DIUC and UIUC values with IEEE P802.16Rev2/D6

Harry Bims
Apple, Inc.

# Overview

The resolution of Comment 663 did not solve the harmonization problem with IEEE P802.16Rev2/D6.  This contribution proposes a solution that aligns the clause numbering and the DIUC and UIUC values in IEEE P802.16h/D8 and IEEE P802.16Rev2/D6 by making changes to IEEE P802.16h/D8.

Specific editorial changes

The following is a legend of text formats that is used in the remainder of this contribution.

Blue text represents specific editorial additions.

Red strikethrough text is to be deleted.

Black text is text already in the draft.

Bold italic text is editorial instructions to the editor.

# Text proposal for clause 8.3.6.2

Modify the following text accordingly:

[Modify the following row in Table 281]

Table 281—OFDM DL-MAP Dummy IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Dummy_IE() { |  |  |
| Extended DIUC | 4 | 0x060x07...0x0F |
| Length | 4 | 0..15 |
| Unspecified data | variable | The Length field specifies the size of this field in bytes |
| } | – | – |

[Modify Clause 8.3.6.2.10]

An extended IE with an extended DIUC value of 0x070x06 is issued by the BS to request periodic channel measurement

(see 15.3.5.4).

[Center the entries in the Size column of Table 285a]

Table 285a—OFDM Periodic channel Measurement IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Periodic_Channel_Measurement_IE() { |  |  |
| Extended DIUC | 4bits | Periodic_Channel_Measurement_IE=0x07 0x06 |
| Length | 4 bits | Length=0x04 |
| Extended Channel Nr | 16 bits | Center frequency in kHz (WirelessMAN-UCP and WirelessMAN-CX only) |
| CID |  | Basic CID of the SS for which the channel measurement IE is directed. |
| Number of Measurement Request | 4 bits | n |
| For(i=0;i<n;i++) { |  |  |
| Measurement request index | 4 bits |  |
| Number of Frames for Periodic Measurement | 16 bits | the total number of frames to perform periodic measurement 0: indicates that the periodic measurement will continue all the time |
| Start Frame Number Offset | 8 bits | The number of frames after the current frame where the measurement will start. |
| Number of Frames between two Measurement Intervals | 8 bits | Number of frames between two available measurement frames |
| Start of Measurement Interval | 8 bits | OFDMA offset of the beginning of measurement interval |
| End of Measurement Interval | 8 bits | OFDMA offset of the end of measurement interval |
| } |  |  |
| } |  |  |

# Text proposal for clause 8.4.5.3

The Extended Channel Measurement IE is not described in the draft.  It should not be added to Table 327.

[modify the following rows to table 326327]

| Extended-3 DIUC (hexadecimal) | Usage |
| --- | --- |
| 010x2 | TxCNT IE |
| 02 | Extended Channel Measurement IE |
| 030x3 | Periodic Channel Measurement IE |
| 04-0F0x4–0xF | reserved |

[Modify the following rows in Table 372a]

| Syntax | Size (bit) | Usage |
| --- | --- | --- |
| Extended-2 DIUC | 4 bits | 0x0F (Extended-3 DIUC) |
| Length | 4 bits | Length in bytes |
| Extended-3 DIUC | 4 bits | TxCNT IE = 0x012 |
| Tx Count | 8 bits | Count, modulo 256, of DL sub-frames trasmitted |
| padding | 4 bits |  |

[Modify the following rows in Table 372b]

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Extended-2 DIUC | 4 bits | 0x0F (Extended-3 DIUC) |
| Length | 4 bits | Length in bytes |
| Extended-3 DIUC | 4 bits | Periodic_Channel_Measurement_IE=0x03 |
| Channel Nr | 8 bits | Center frequency in kHz (WirelessMAN-UCP and WirelessMAN-CX only) |
| CID | 16 | Basic CID of the SS for which the channel measurement IE is directed. |
| Number of Measurement Request | 4 bits | The number, n, of periodic measurement requests included in this IE. |
| Measurement request index | 4 bits |  |
| Number of Frames for Periodic Measurement | 16bits | the total number of frames to perform periodic measurement,In unit Frame 0: indicate the periodic measurement shall continue all the time. |
| Start Frame Number Offset | 4 bits | The offset of frame which start measurement to the current frame |
| Number of Frames between two Measurement Intervals | 4 bits | Number of frames between two available measurement frames |
| Start of Measurement Interval | 4 bits | OFDMA offset of the beginning of measurement interval |
| End of Measurement Interval | 4 bits | OFDMA offset of the end of measurement interval |

# Harmonization of clause numbering with 802.16j

[Modify clause 11.22 to become clause 11.27]

[Modify clause 11.23 to become clause 11.87]

[Modify clause 11.24 to become clause 11.29]
[Modify clause 8.4.5.3.31 to become clause 8.4.5.3.32]

[Modify clause 8.4.5.3.32 to become clause 8.4.5.3.33]

[start numbering of definitions in clause 3. Definitions at 3.128, instead of 3.95]

[modify the following rows in the table of clause 11.4.1]

| Name | Type (1 byte) | Length | Value (variable length) | PHY Scope |
| --- | --- | --- | --- | --- |
| CSI allocation | 6264 | 1 | Bit0-3: CSI Cycle (the number of CX-Frames between the bits in CSI sequence of one system) Default value - 2 Bit4-5: CSIN of this system (indicating the OCSI allocation of this system) 0-OCSI1 1-OCSI2 2-OCSI3 3-reserved Bit6-7: reserved | WirelessMAN-CX (OFDM/OFDMA) |
| CSI Timing | 6365 | 6 | Bit0-15: TCSITSTART: Starting PSs/symbol number of CSI signaling transmission (offset from the starting point of Frame) Bit16-31: TCSIDSTART: Starting PSs number of CSI signaling receiving (offset from the point of TCSITSTART) Bit32-48: TCSIDDUR: CSI detection duration in unit of PSs | WirelessMAN-CX (OFDM/OFDMA) |