| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Solution for comment A8 on frequency keying |  |
| Date Submitted | 2009-01-15 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv | Voice: +972 544 22 55 48 E-mail: mariana.goldhamer@alvarion.com |
| Re: |  |  |
| Abstract |  |  |
| Purpose | Actions: 1. Modification of ToC 2. Capture of the text in the SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Solution for comment A8 on frequency keying

Mariana Goldhamer

Alvarion Ltd.

# Introduction

Comment a8 indicates that signaling using the frequency keying introduces PHY changes. This contribution modifies the signaling procedures such that regular sub-channelized transmissions are used. The actual sub-channels used for regular data transmission will give an indication of the required action or synchronization.

# Proposed solution

- Mechanisms based on energy keying in frequency domain

- Signal Definition

The mechanisms based on energy keying in frequency domain are used for time synchronization or for the registration of the systems using special signaling procedures.

The frequency-keyed energy pulses use the regular data transmission on a single sub-channel or a combination of subchannels. The transmissions shall use DL_perm_id=0 and UL_perm_id=0 and shall use a preamble, located at the beginning of the zone used for transmission (8.4.5.3.3, 8.4.5.4.6). In DL, a zone will be scheduled at Tcc_ss. a

The energy on a sub-channel transmitted by the BS should be detected in the interval stating at Tcc_ss and ending at Tcc_s.

The energy on a sub-channel transmitted by the SS should be detected in the interval stating at the UL transmission and ending at Tcc_se.

The following figures show the desired spectral density for radio signaling. Independent of the actual channel width, the preambles are sent using the narrowest channel possible in the band. The center of the virtual narrowest channel shall coincide with the center of the used channel. In the following example, in which channels of 5, 10 and 20MHz may be used, the narrowest channel is 5MHz and any other system should be able to detect the preambles, because they are not attenuated by any radio filter. The narrowest channel is centered in the frequency domain around the actually used channel center.

- Desired spectral densities for different channel BWs

In Radio signal definition the radio signals SFQ1-SFQ9 are defined.

These signals are used for synchronization or time marking. The transmissions can also be used for frequency-keyed message coding, in which case the transmission defined for a sub-channel represents the binary value “1” if it is transmitted. Only sub-channels 1-

- Radio signal definition

| Sub-channel index (0..11) |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SFQ1 |  |  |  |  | H | L | L | H | H | L |  |  |  |  |  |  |  |
| SFQ2 |  |  |  |  | L | H | L | L | H | H |  |  |  |  |  |  |  |
| SFQ3 |  |  |  |  | H | L | H | L | L | H |  |  |  |  |  |  |  |
| SFQ4 |  |  |  |  | L | H | L | H | L | L |  |  |  |  |  |  |  |
| SFQ5 |  |  |  |  | L | L | H | L | H | L |  |  |  |  |  |  |  |
| SFQ6 |  |  |  |  | L | L | L | H | L | H |  |  |  |  |  |  |  |
| SFQ7 |  |  |  |  | H | L | L | L | H | L |  |  |  |  |  |  |  |
| SFQ8 |  |  |  |  | L | H | H | L | L | H |  |  |  |  |  |  |  |
| SFQ9 |  |  |  |  | L | L | H | H | L | L |  |  |  |  |  |  |  |

- Synchronization using CXCC

The synchronization signals are transmitted by the Master BS and optionnaly by the SS/MS.

A synchronization signal may be placed at the start of the CXCC time-slot, used by a specific system as Master, in the following way:

- BS synchronized by GPS, IEEE 1588, or equivalent satellite-derived time should use the 3d CXCC sub-channel, SFQ1

- BS synchronized by another BS should use the 3rd CXCC sub-channel, SFQ2

- SS associated with a BS which is synchronized by GPS, IEEE 1588, or equivalent satellite-derived time should send the SFQ3 synchronization signal in the 3dCXCC sub-channel, using an UL allocation, if instructed by the BS to do so;

- SS associated with a BS, which is synchronized by another BS or by NTP, should send the SFQ4 synchronization signal in the 3rd CXCC sub-channel, using an UL allocation, if instructed by the BS to do so;

The synchronization signal is a subset of the radio signals defined in Mechanisms based on energy keying in frequency domain, Radio signal definition. The  indicates the sequence of the radio signals to be used for marking the slots of the coexistence control channel. These signals are defined in such a way that a cognitive radio can determine the CXCC allocation corresponding to the detected synchronization signals.