| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Improvements on messages in 15.5 and 15.6 |  |
| Date Submitted | 2009-03-03 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv | Voice: +972 544 22 55 48 E-mail: mariana.goldhamer@alvarion.com |
| Re: | 16h WG Recirc. |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Improvements on messages in 15.5 and 15.6

Mariana Goldhamer

Alvarion Ltd.

# Introduction

CXP contains some more or less sophisticated messages, but there is no message to set the basic parameters for operation.

# Proposed solution

A new message has to introduced in 15.5 and 15.6.

# Text changes

Add new clause 15.5.3.30

15.5.3.30 CX-FWD-REQ (Action code: SET CX Basic Parameters Request)

The CX-FWD-REQ message with Action Code = SET CX Basic Parameters Request) is sent by a BS to a coexistence BS in the community to request the change of one or more basic operating parameters, such as: operating frequency channel, channel width, CX-Frame type, Master sub-frame for DL and UL, Additional Master sub-frame for UL. The actual parameter list can use any suitable TLV in clause 11.29.

The following parameters shall be included in the CX-FWD-REQ (Action Code = SET CX Basic Parameter Request) message:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of source BS | The requesting BS identifier |
| BSID of destination BS | BS identifier of the requested BS |
| MAC Frame Number | MAC Frame sequence number to change the parameters. |

15.5.3.31 CX-FWD-REQ (Action code: SET CX Basic Parameters Response)

The CX-FWD-RSP message with Action Code = SET CX Basic Parameters Response) is sent by a BS to replay to the CX-FWD-REQ message with Action Code = SET CX Basic Parameters Request).

The following parameters shall be included in the CX-FWD-RSP (Action Code = SET CX Basic Parameter Response) message:

| TLV Parameter Name | Notes |
| --- | --- |
| BSID of source BS | The requesting BS identifier |
| BSID of destination BS | BS identifier of the requested BS |
| MAC Frame Number | MAC Frame sequence number of the actual change. |

End change

Add new clause 15.6.1.13

16.6.1.13 C-CX-REQ (Action_Type = SET CX Basic Parameters)

Function:

This primitive is used by an 802.16 entity (BS) or NCMS to ask a BS to change one or more basic operating parameters, such as: operating frequency channel, channel width, CX-Frame type, Master sub-frame ID for DL and UL, Additional UL Master sub-frame ID. The actual parameter list can use any suitable TLV in clause 11.29.

Semantics:

The parameters of the primitive are as follows:

C-CX-REQ

(

Operation_Type: Action,

Action_Type: SET CX Basic Parameters

Destination: BS

Attribute_List:

BSID of the requester

Requested BSID

MAC Frame number

Channel Center Frequency,

Channel Width,

CX-Frame_ID

Master sub-frame ID

Additional UL Master sub-frame ID

)

| Attribute | Contents |
| --- | --- |
| BSID of source BS | The requesting BS identifier |
| BSID of destination BS | BS identifier of the requested BS |
| MAC Frame Number | MAC Frame sequence number of the requested change. |
| Channel Center Frequency | Channel center frequency |
| Channel Width | Channel width |
| CX-Frame_ID | CX-Frame ID |
| Master sub-frame ID | Master sub-frame ID (refers to both DL and UL sub-frames) |
| Additional UL Master sub-frame ID | Additional UL Master sub-frame ID, as defined in fig. 409 |

When generated:

o 802.16 entity (BS) to 802.16 entity (BS)

This primitive is used by the 802.16 entity (BS) to request another BS to set the operational parameters.

o NCMS to 802.16 entity (BS):

This primitive is used by the NCMS to ask the requested BS to set the operational parameters.

Effect of receipt:

o 802.16 entity (BS):

The requested BS performs or not the requested action and respond to the requesting BS / NCMS accordingly.

End change

Add new clause 15.6.2.13

16.6.2.13 C-CX-RSP (Action_Type = SET CX Basic Parameters)

Function:

This primitive is used by an 802.16 entity (BS) to respond to the 802.16 entity (BS) or to the NCMS which has requested the parameter change.

Semantics:

The parameters of the primitive are as follows:

C-CX-RSP

(

Operation_Type: Action,

Action_Type: SET CX Basic Parameters

Destination: BS, NCMS

Attribute_List:

BSID of the requester

Requested BSID

MAC Frame number

Acceptance indication

)

| Attribute | Contents |
| --- | --- |
| BSID of source BS | The requesting BS identifier |
| BSID of destination BS | BS identifier of the requested BS |
| MAC Frame Number | MAC Frame sequence number of the actual change. |
| Acceptance indication | Code for acceptance or refusal |

When generated:

o In response to C-CX-REQ (Action_Type = SET CX Basic Parameters)

Effect of receipt:

o According to the acceptance or refusal, up-date the operational parameters of the requested BS.

End change