| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Additional CX-Frame structures for two systems on a channel |  |
| Date Submitted | 2009-05-04 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street, Tel Aviv Shulan Feng Hisilicon Tech. Co., LTD Bld.17, No.8, Dongbeiwang West Road, Hai-Dian District, Beijing, P. R. China | Voice: +972 544 22 55 48 E-mail: mariana.goldhamer@alvarion.com Voice: +86-10-82829151 e-mail to : fengsl@hisilicon.com |
| Re: |  |  |
| Abstract |  |  |
| Purpose | Actions: 1. Modification of ToC 2. Capture of the text in the SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Additional CX-Frame structures for two systems on a channel

Mariana Goldhamer, Shulan Feng

Alvarion Ltd., Hisilicon Tech.

# Introduction

In the last recirculation were added figures with CX-Frame structure for the case when there are only two systems on a channel as response to comments A159 and A180; however the solution is incomplete.

Another issue is related to the selection of the CX-Frame from a number of variants; it is needed to define a default value. The best candidates for the default values are the CX-Frames accommodating two systems, because they have a better support for existing UL-MAP messages.

Comment A159:

Are missing figures for two systems

Comment A180

This clause needs improvement; if only 2 Master systems are on a channel it is easier to find better solutions using the existing MAP relevance

Suggested remedy:

Contribution needed.

# Proposed text changes

1. Modify starting with page 101, line 45:

Figure 407 indicates the recommended CX-Frame in case that accommodates only two Master Frames. Each Master frame is followed by a Shared frame. It is recommended that an operator will use systematically the same Master Frame. The up-link scheduling can be done using the CX-UL-MAP message or the regular UL-MAP message with shorter relevance.

Fig. 407 Coexistence Frame with two Master systems

Another possibility is shown in fig. 407’. This CX-Frame is better suited for up-link scheduling using the regular UL-MAP message.

Fig. 407’ Coexistence Frame with two Master systems

2. Replace the Table 648 with the following table, including the added CX-Frame and also including some editorial changes; modify the text before the table as shown:

The identification of the different CX-Frame types is done as shown in Table 648. In case of channel sharing between 802.16 systems only, the default CX-Frame ID = 4. Other CX-Frame IDs can be used based on operator coordination.

| No. of 802.16 Master systems on a channel | Common sub-frame (Yes/No) | Coexistence Frame details | CX-Frame ID |
| --- | --- | --- | --- |
| 3 | No | Figure 405 | 0 |
| 3 | Yes | Figure 406 | 1 |
| 2 | No | One UL Master sub-frame per system, Figure 407 | 2 |
| 2 | Yes | One UL Master sub-frame per system | 3 |
| 2 | No | Two UL Master sub-frames per system, Figure 407’ | 4 (Default) |
| 2 | Yes | Two UL Master sub-frames per system | 5 |
| 2 | No | CX-Frame for a macro and micro cell sharing a channel, Figure 408 | 6 |
| 2 | Yes | CX-Frame for a macro and micro cell sharing a channel | 7 |

3. Align the table content with the actual figure numbering

4. Add support for CX-Frame structure for CX-CBP for only two systems on a channel by modifying the text below:

The CX-Frame structure when there are three systems on a channel is shown below:

Figure 410—CX-Frame structure for CX-CBP

In case of only two systems on a channel, from which one system is bursty, the 802.16 system can use the CXSBI as Master.

The CX-Frame ID is supplemented with the fields in Table xx. In case of channel sharing between 802.16 systems and Bursty systems, the default CX-Frame ID = 10. Other CX-Frame IDs can be used based on operator coordination.

Table xx: Additional CX-Frame IDs for CX-CBP

| No. of 802.16 Master systems during CXSBI | Common sub-frame (Yes/No) | Coexistence Frame details | CX-Frame ID |
| --- | --- | --- | --- |
| 2 | No | CX-CBP, one UL Master sub-frame per system, Figure 410 | 8 |
| 2 | Yes | CX-CBP, one UL Master sub-frame per system | 9 |
| 1 | No | CX-CBP, CXSBI interval used as Master | 10 (default) |
| 1 | Yes | CX-CBP, CXSBI interval used as Master | 11 |

5. Modify the CX-Frame ID TLV in clause 11:

101 / CX-Frame ID / 2 / Bits 0-11: seeTable 648 and Table 648’

Bits 12-15: Reserved, shall be set to 0.