| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text changes related to CSI |  |
| Date Submitted | 2009-09-22 |  |
| Source(s) | Dan Gal, Alcatel-Lucent Amir Ghasemi, CRC Mariana Goldhamer, Alvarion Ron Murias, InterDigital Shawn Taylor, Huawei | dgal@alcatel-lucent.com aghasemi@crc.ca mariana.goldhamer@alvarion.com ron@murias.ca staylor@esm.cx |
| Re: |  |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Text changes related to CSI

Dan Gal, Amir Ghasemi, Mariana Goldhamer, Ron Murias, Shawn Taylor

Alcatel-Lucent, CRC, Alvarion, InterDigital, Huawei

Implement the following text changes:

15.3.4.1 Coexistence Signaling Interval

The CSI (Coexistence Signaling Interval) is a predefined reserved short time slot at the end of a DL subframe, in which only oneBS may transmit data.

15.3.4.1.1 DL timing adjustment for Coexistence Signaling

If a BS transmits normal data during the CSI slot, it shall mean a logical “1” to the SSs in other systems. If the BS does not transmit data during the CSI slot, it shall mean a logical “0” to the SSs in other systems. The timing of the CSI slot is described in clause 10.5.2. The timing is shown in Figure 396.

10.5

TCSIEREF / CSI end reference time is defined as an offset from the starting point of the synchronized frame. CSI end reference time is the time offset to the end of the last symbol in down-link.

. /  /  /  //

TCSISREF / CSI start reference time is defined as an offset value from the starting point of the synchronized

frame. CSI shall start within the last symbol or earlier. (see 11.4.1) /   /  TCSIEREF - 100 μs /

11.4.1

CSI Timing/ 66 /4 /Bit0-15: TCSITSTART: Time offset to the start of the first symbol of the CSI interval.

Bit16-31: TCSIDDUR:

CSI detection duration, which is an integer multiple of symbol durations.

Figure 396 – Instruction to the Editor

Check and remake if needed the “Figure 396—Timing of CSI bit unit”, to be consistent with the definitions in 10.5. Delete all the time references which are not used. Make the end of the CSI identical with the end of the DL transmission. Delete “TCSIDSTART” from the figure.