To:	Roger Marks

Chair, IEEE 802.16 Working Group

r.b.marks@ieee.org

cc: 	Roger Marks

Vice President - Technology Standards, WiMAX Forum

roger.marks@wimaxforum.org

Tim Hewitt

Chair, Regulatory Working Group, WiMAX Forum

tim.hewitt@wimaxforum.org

Reference: IEEE L802.16-09/0085r1

August 28, 2009

Subject: WiMAX Forum TWG Contribution to development of Candidate IMT-Advanced RIT based on IEEE 802.16

Dear Dr. Marks,

Thank you for the liaison statement of 16 July (IEEE L802.16-09/0085r1) informing the WiMAX Forum of IEEE 802.16’s activities regarding IMT-Advanced submission and preparations, calling our attention to the Call for Comments and Contributions on IMT-Advanced Submission (IEEE L802.16-09/0079), and referencing our previous statement (L802.16-09/0045) of 29 April.

Following up that prior contribution, the WiMAX Forum Technical Working Group (TWG) would like to submit the attached draft contribution (Annex A) in response to IEEE L802.16-09/0079. This contribution, in particular, proposes responses to open items in [DRAFT] SUBMISSION OF A CANDIDATE IMT-ADVANCED RIT BASED ON IEEE 802.16 (PART I) (L802.16-09/0050d1).

Depending on the outcome of IEEE 802.16 Session #63, TWG may provide other contributions for IEEE 802.16 Session #63.5 in advance of the ITU-R WP 5D meeting of October 2009.

- Thank you very much for your attention to this matter of mutual importance.

Sincerely,

Wonil Roh, wonil.roh@samsung.com and

Vladimir Yanover vladimir.yanover@alvarion.com

Chairs, WiMAX Forum Technical Working Group (TWG)

annex A: Band Classes, Power classes and Emission Mask Table of Contents

1.	Introduction	4

2.	References	5

3.	Definitions, Symbols and Acronyms	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Acronyms	6

4.	Frequency bands	7

5.	Power Classes	8

5.1	Mobile Station	8

6.	Spectrum Mask	9

6.1	Mobile Station	9

6.1.1	Band Class 3	9

6.1.1.1	TDD	9

6.1.1.2	FDD	11

6.1.2	Band Class 5	12

6.1.2.1	TDD	12

6.1.2.2	FDD	13

6.1.3	Band Class 6	15

6.1.3.1	Frequency Sub-band 1710-1770/2110-2170 MHz	15

6.1.3.2	Frequency Sub-band other than 1710-1770/2110-2170 MHz	16

6.1.4	Band Class 7	17

6.1.4.1	TDD	17

6.1.4.2	FDD	19

6.1.5	Band Class 8	21

6.2	Base Station	22

6.2.1	Band Class 3	23

6.2.1.1	TDD	23

6.2.1.2	FDD	24

6.2.2	Band Class 5	26

6.2.2.1	TDD	26

6.2.3	Band Class 6	27

6.2.3.1	Frequency Sub-band 1710-1755/2110-2155 MHz	27

6.2.4	Band Class 7	29

- LIST OF TABLES

Table 1. List of Band Classes	7

Table 2. Mobile Station Power Classes	8

Table 3. Spectrum Emission Mask for 5 MHz Bandwidth	9

Table 4. Spectrum Emission Mask for 10 MHz Bandwidth	9

Table 5. Spectrum Emission Mask for 20 MHz Bandwidth	9

Table 6. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3	10

Table 7. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3	10

Table 8. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3	10

Table 9. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3	11

Table 10. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3	11

Table 11. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3	11

Table 12. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5	12

Table 13. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5	13

Table 14. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5 (3402.5 fc  3497.5)	14

Table 15. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5 (3405 fc  3495)	15

Table 16. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz)	15

Table 17. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz)	15

Table 18. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz)	16

Table 19. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 6	16

Table 20. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 6	16

Table 21. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 6	16

Table 22. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (700.5 fc  795.5)	17

Table 23. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (799.5 fc  859.5)	17

Table 24. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (703 fc  793)	17

Table 25. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (802 fc  857)	18

Table 26. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (703 fc  793)	18

Table 27. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (807 fc  852)	18

Table 28. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (700.5 fc  834.5)	19

Table 29. Spectrum Emission for 5 MHz Bandwidth Band Class 7 (834.5 fc  859.5)	19

Table 30. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (703 fc < 837)	19

Table 31. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (837 fc  857)	20

Table 32. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (708 fc  842)	20

Table 33. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (842 fc  852)	20

Table 34. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 8	21

Table 35. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 8	21

Table 36. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 8	21

Table 37. Spectrum Emission Mask for 5 MHz Bandwidth	22

Table 38. Spectrum Emission Mask for 10 MHz Bandwidth	22

Table 39. Spectrum Emission Mask for 20 MHz Bandwidth	22

Table 40. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3	23

Table 41. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3	23

Table 42. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3	24

Table 43. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3-Japan	24

Table 44. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3-Japan	24

Table 45. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3-US	24

Table 46. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3-US	24

Table 47. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3-US	24

Table 48. Relative Transmit Spectral Power Density Mask	26

Table 49. Absolute Spectral Emission Mask	26

Table 50. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz)	27

Table 51. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz)	27

Table 52. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz)	27

Table 53. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7-US	29

Table 54. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7-US	29

Table 55. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7-US	29

Table 56. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7-Europe	32

Table 57. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7-Europe	32

Table 58. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7-Europe	33

# Introduction

The purpose of this contribution is to propose the response text to the following questions in IMT-Advanced Technology Description Template – Characteristics Template (TDT) in support of Candidate IMT-Advanced RIT based on IEEE 802.16.

Item 1: 4.2.3.2.8.3	What are the frequency bands supported by the RIT?  Please list.

- Item 2: 4.2.3.2.11 Power classes

- Item 3: 4.2.3.2.23.5 Does the proposal satisfy a specific spectrum mask? Provide details.

As a baseline, this contribution uses the WiMAX Forum Mobile Release 1.0 and 1.5 specifications relevant to Items 1 and 3 above as provided in WiMAX Forum Mobile Certification Profile ([1] and [2]) and Radio Specification ([3]) documents and proposes additional text to properly cover all aspects of the three items.

On Item 3, spectrum emission masks for the following four Test Scenarios as specified in Report ITU-R M.2135 ([5]) are provided.

- Indoor Hotspot: 3.4 GHz

- Urban Micro-cell: 2.5 GHz

- Urban Macro-cell: 2.0 GHz

- Rural Macro-cell: 0.8 GHz

# References

- WiMAX Forum Mobile Certification Profile R1.0, Description 1

- WiMAX Forum Mobile Certification Profile R1.5, Description 2

- DRAFT-T23-005-R015v04-B, WiMAX Forum Mobile Radio Specification document

- Working document towards a preliminary draft revision of Recommendation ITU-R M.1036-3 - Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations (RR)

- Report ITU-R M.2135, GUIDELINES FOR EVALUATION OF RADIO INTERFACE TECHNOLOGIES FOR IMT-ADVANCED

# Definitions, Symbols and Acronyms

# Definitions

This section provides the list of definitions used through out the document.

Integration Bandwidth: Integration Bandwidth refers to the frequency range over which the emission power is integrated.

# Symbols

This section provides the list of symbols used through out the document along with their descriptions.

f	frequency in MHz unless specified otherwise.

fc	center frequency in MHz unless specified otherwise.

Δf	absolute value of frequency offset of frequency f from the center frequency of the channel (|fc-f|). The offset is in MHz unless specified otherwise.

PTx		measured transmit power into antenna in dBm

PTx,max	measured maximum transmit power into antenna in dBm

Pnom 		declared maximum conducted transmit power

# Acronyms

This section provides the list of acronyms and abbreviations used through out the document.

BS		base station

BW		channel bandwidth size

MS		mobile station

PSD		power spectral density

# Frequency bands

The purpose of this section is to address the IMT-Advance TDT item 4.2.3.2.8.3	(What are the frequency bands supported by the RIT?  Please list.) for IEEE 802.16 based candidate.

Table 1 provides the list of band classes.

- Table 1. List of Band Classes

| Band Class | Uplink MS Transmit Frequency (MHz) | Downlink MS Receive Frequency (MHz) | Duplex Mode |
| --- | --- | --- | --- |
| 1 | 2300-2400 | 2300-2400 | TDD |
| 2 | 2305-2320, 2345-2360 | 2305-2320, 2345-2360 | TDD |
|  | 2345-2360 | 2305-2320 | FDD |
| 3 | 2496-2690 | 2496-2690 | TDD |
|  | 2496-2572 | 2614-2690 | FDD |
| 4 | 3300-3400 | 3300-3400 | TDD |
| 5L | 3400-3600 | 3400-3600 | TDD |
|  | 3400-3500 | 3500-3600 | FDD |
| 5H | 3600-3800 | 3600-3800 | TDD |
| 6 | 1710-1770 | 2110-2170 | FDD |
|  | 1920-1980 | 2110-2170 | FDD |
|  | 1710-1755 | 2110-2155 | FDD |
|  | 1710-1785 | 1805-1880 | FDD |
|  | 1850-1910 | 1930-1990 | FDD |
|  | 1710-1785, 1920-1980 | 1805-1880, 2110-2170 | FDD |
|  | 1850-1910, 1710-1770 | 1930-1990, 2110-2170 | FDD |
| 7 | 698-862 | 698-862 | TDD |
|  | 776-787 | 746-757 | FDD |
|  | 788-793, 793-798 | 758-763, 763-768 | FDD |
|  | 788-798 | 758-768 | FDD |
|  | 698-862 | 698-862 | TDD/FDD |
|  | 824-849 | 869-894 | FDD |
|  | 880-915 | 925-960 | FDD |
|  | 698-716, 776-793 | 728-746, 746-763 | FDD |
| 8 | 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920 | 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920 | TDD |
| 9 | 450-470 | 450-470 | TDD |
|  | 450.0-457.5 | 462.5-470.0 | FDD |

# Power Classes

The purpose of this section is to address the IMT-Advance TDT item 2: 4.2.3.2.11 (Power classes) for IEEE 802.16 based candidate.

# Mobile Station

Table 2 provides the list of Mobile Station Power Classes.

- Table 2. Mobile Station Power Classes

| Class Identifier | Pnom (Nominal Max Output Power) (dBm) |
| --- | --- |
| Power Class 1 | 20 |
| Power Class 2 | 23 |
| Power Class 3 | 26 |

- In Table 2, Pnom is the declared maximum conducted transmit power.

In power classes of Table 2, tolerances are not included. In general, tolerances associated to Pnom depends on various parameters including MCS level, spectrum band span (associated to band edge) etc.  Generic and band specific tolerances associated to these parameters may be considered.

# Spectrum Mask

The purpose of this section is to address the IMT-Advance TDT item 4.2.3.2.23.5 (Does the proposal satisfy a specific spectrum mask? Provide details) for IEEE 802.16 based candidate.

Note that, in this section, spectrum emission masks for the following four Test Scenarios as specified in Report ITU-R M.[IMT.EVAL] ([5]) is provided.

- Indoor Hotspot: 3.4 GHz (3.4-3.6 GHz)

- Urban Micro-cell: 2.5 GHz (2.5-2.690 GHz)

- Urban Macro-cell: 2.0 GHz (1.710-2.170 GHz)

- Rural Macro-cell: 0.8 GHz (698-862 MHz)

- The information in section is applicable to all power classes of Section 5.

  - Mobile Station

Unless otherwise specified in sub sections of Section 6.1 for specific bands, the spectrum masks of Table 3, Table 4 and Table 5 are applicable.

- Table 3. Spectrum Emission Mask for 5 MHz Bandwidth

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | -13 |
| 2 | 3.5 to  12.5 | 1000 | -13 |

- Table 4. Spectrum Emission Mask for 10 MHz Bandwidth

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to < 6 | 100 | -13 |
| 2 | 6 to  25 | 1000 | -13 |

- Table 5. Spectrum Emission Mask for 20 MHz Bandwidth

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to  50 | 1000 | -13 |

# Band Class 3

# TDD

The Spectrum Emission Mask for 5 MHz bandwidth is specified in Table 6.

- Table 6. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port. |
| --- | --- | --- | --- |
| 1 | 2.5 to <3.5 | 50 | -13 |
| 2 | 3.5 to <7.5 | 1000 | -13 |
| 3 | 7.5 to <8 | 500 | If PTx  +23 and (2547.5  fc  2622.5) then -23-2.28(∆f -7.5) else -16 |
| 4 | 8 to <10.4 | 1000 | -25 |
| 5 | 10.4 to  12.5 | 1000 | If PTx  +23 and (2547.5  fc  2622.5) then -21-1.68(∆f - 8) else -25 |

The Spectrum Emission Mask for 10 MHz bandwidth is specified in Table 7.

- Table 7. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <6 | 100 | -13 |
| 2 | 6 to <10 | 1000 | -13 |
| 3 | 10 to <11 | 1000 | -13-12(f -10) |
| 4 | 11 to <15 | 1000 | -25 |
| 5 | 15 to <20 | 1000 | If PTx +23 dBm and (2550  fc  2620) then -21 - 32(f –10.5)/19 else -25 |
| 6 | 20 to  25 | 1000 | If PTx  +23 dBm and (2550  fc  2620) then -37 else -25 |

The Spectrum Emission Mask for 20 MHz bandwidth is specified in Table 8.

- Table 8. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to <15 | 1000 | -13 |
| 3 | 15 to <16 | 1000 | -13-12(f -15) |
| 4 | 16 to  50 | 1000 | -25 |

# FDD

The Spectrum Emission Mask for 5 MHz bandwidth is specified in Table 9.

- Table 9. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port. |
| --- | --- | --- | --- |
| 1 | 2.5 to <3.5 | 50 | -13 |
| 2 | 3.5 to <7.5 | 1000 | -13 |
| 3 | 7.5 to <8 | 500 | -16 |
| 4 | 8 to <10.4 | 1000 | -25 |
| 5 | 10.4 to  12.5 | 1000 | -25 |

The Spectrum Emission Mask for 10 MHz bandwidth is specified in Table 10.

- Table 10. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <6 | 100 | -13 |
| 2 | 6 to <10 | 1000 | -13 |
| 3 | 10 to <11 | 1000 | -13-12(f -10) |
| 4 | 11 to <15 | 1000 | -25 |
| 5 | 15 to <20 | 1000 | -25 |
| 6 | 20 to  25 | 1000 | -25 |

The Spectrum Emission Mask for 20 MHz bandwidth is specified in Table 11.

- Table 11. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to <15 | 1000 | -13 |
| 3 | 15 to <16 | 1000 | -13-12(f -15) |
| 4 | 16 to  50 | 1000 | -25 |

# Band Class 5

# TDD

The Spectrum Emission Mask for 5 MHz bandwidth is specified in Table 12.

- Table 12. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5

| Frequency offset f (MHz) | Maximum Emission Level (dBc) | Integration Bandwidth |
| --- | --- | --- |
| 2.5 to < 3.5 | -33.5-15(∆f-2.5) | 30 kHz |
| 3.5 to < 7.5 | -33.5-1(∆f-3.5) | 1 MHz |
| 7.5 to < 8.5 | -37.5-10(∆f-7.5) | 1 MHz |
| 8.5 to  12.5 | -47.5 | 1 MHz |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 3.485 MHz.

- The first measurement position with a 1 MHz filter is at f equals to 4 MHz; the last is at f equals to 12 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth.

- Note that equivalent PSD type mask can be derived by applying 10*log ((5 MHz)/(30  kHz))= 22.2 dB and 10*log((5 MHz)/(1 MHz))= 7 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively.

The Spectrum Emission Mask for 10 MHz bandwidth is specified in Table 13.

- Table 13. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5

| Frequency offset f (MHz) | Maximum Emission Level (dBc) | Integration Bandwidth |
| --- | --- | --- |
| 5.0 to <7.0 | -33.5-9(∆f-5.0) | 30 kHz |
| 7.0 to <15.0 | -36.5-0.5(∆f-7.0) | 1 MHz |
| 15.0 to <17.0 | -40.5-5(∆f-15.0) | 1 MHz |
| 17.0 to 25.0 | -50.5 | 1 MHz |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 510.015 MHz; the last is at f equals to 6.985 MHz.

- The first measurement position with a 1 MHz filter is at f equals to 7.5 MHz; the last is at f equals to 24.5 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth.

- Equivalent PSD type mask can be derived by applying 10*log ((10 MHz)/(30 kHz))= 25.2 dB and 10*log((10 MHz)/(1 MHz))= 10 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively.

# FDD

The Spectrum Emission Mask for 5 MHz bandwidth is specified in Table 14.

- Table 14. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5 (3402.5 fc  3497.5)

| Frequency offset f (MHz) | Maximum Emission Level (dBc) | Integration Bandwidth |
| --- | --- | --- |
| 2.5 to <3.5 | -33.5-15(∆f-2.5) | 30 kHz |
| 3.5 to <7.5 | -33.5-1(∆f-3.5) | 1 MHz |
| 7.5 to <8.5 | -37.5-10(∆f-7.5) | 1 MHz |
| 8.5 to 12.5 | -47.5 | 1 MHz |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 3.485 MHz.

- The first measurement position with a 1 MHz filter is at f equals to 4 MHz; the last is at f equals to 12 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth.

- Note that equivalent PSD type mask can be derived by applying 10*log ((5 MHz)/(30  kHz))= 22.2 dB and 10*log((5 MHz)/(1 MHz))= 7 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively.

The Spectrum Emission Mask for 10 MHz bandwidth is specified in Table 15.

- Table 15. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5 (3405 fc  3495)

| Frequency offset f (MHz) | Maximum Emission Level (dBc) | Integration Bandwidth |
| --- | --- | --- |
| 5.0 to <7.0 | -33.5-9(∆f-5.0) | 30 kHz |
| 7.0 to <15.0 | -36.5-0.5(∆f-7.0) | 1 MHz |
| 15.0 to <17.0 | -40.5-5(∆f-15.0) | 1 MHz |
| 17.0 to 25.0 | -50.5 | 1 MHz |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 510.015 MHz; the last is at f equals to 6.985 MHz.

- The first measurement position with a 1 MHz filter is at f equals to 7.5 MHz; the last is at f equals to 24.5 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth.

- Equivalent PSD type mask can be derived by applying 10*log ((10 MHz)/(30 kHz))= 25.2 dB and 10*log((10 MHz)/(1 MHz))= 10 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively.

# Band Class 6

# Frequency Sub-band 1710-1770/2110-2170 MHz

The Spectrum Emission Mask for 5 MHz bandwidth is specified in Table 16.

- Table 16. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz)

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | -13 |
| 2 | 3.5 to  12.5 | 1000 | -13 |

The Spectrum Emission Mask for 10 MHz bandwidth is specified in Table 17.

- Table 17. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz)

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to < 6 | 100 | -13 |
| 2 | 6 to  25 | 1000 | -13 |

- The Spectrum Emission Mask for 20 MHz bandwidth is specified in Table 18.

- Table 18. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz)

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to 50 | 1000 | -13 |

# Frequency Sub-band other than 1710-1770/2110-2170 MHz

- The Spectrum Emission Mask for 5 MHz bandwidth is specified in Table 19.

- Table 19. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 6

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <3.5 | 50 | -13 |
| 2 | 3.5 to < 7.5 | 1000 | -10 |
| 3 | 7.5 to <8.5 | 1000 | -13 |
| 4 | 8.5 to 12.5 | 1000 | -25 |

- The Spectrum Emission Mask for 10 MHz bandwidth is specified in Table 20.

- Table 20. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 6

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <6 | 100 | -13 |
| 2 | 6 to <10 | 1000 | -10 |
| 3 | 10 to <15 | 1000 | -13 |
| 4 | 15 to 25 | 1000 | -25 |

- The Spectrum Emission Mask for 20 MHz bandwidth is specified in Table 21.

- Table 21. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 6

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to <15 | 1000 | -10 |
| 3 | 15 to <30 | 1000 | -13 |
| 4 | 30 to 50 | 1000 | -25 |

# Band Class 7

# TDD

- Table 22 and Table 23 specify the spectrum emission mask with 5 MHz channel bandwidth.

- Table 22. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (700.5 fc  795.5)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <2.6 | 30 | -13 |
| 2 | 2.6 to 12.5 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 2.585 MHz. The first measurement position with a 100 kHz filter is at f equals to 2.650 MHz; the last is at f equals to 12.450 MHz.

- Table 23. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (799.5 fc  859.5)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <7.5 | 5 | 1.6 |
| 2 | 7.5 to 12.5 | 2 | -10 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 5 MHz. The first measurement position with a 2 MHz filter is at f equals to 8.5 MHz; the last is at f equals to 11.5 MHz.

- Table 24 and Table 25 specify the spectrum emission mask with 10 MHz channel bandwidth.

- Table 24. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (703 fc  793)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5.0 to <5.1 | 30 | -13 |
| 2 | 5.1 to 25.0 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 5.015 MHz; the last is at f equals to 5.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 5.150 MHz; the last is at f equals to 24.950 MHz.

- Table 25. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (802 fc  857)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <10 | 5 | 1.6 |
| 2 | 10 to 25 | 2 | -10 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 7.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 11 MHz; the last is at f equals to 24 MHz.

Table 26 and Table 27 specify the spectrum emission mask with 20 MHz channel bandwidth.

- Table 26. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (703 fc  793)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10.0 to <10.1 | 30 | -13 |
| 2 | 10.1 to 50.0 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 10.015 MHz; the last is at f equals to 10.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 10.150 MHz; the last is at f equals to 50.950 MHz.

- Table 27. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (807 fc  852)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <15 | 5 | 1.6 |
| 2 | 15 to 50 | 2 | -10 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 12.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 16 MHz; the last is at f equals to 49 MHz.

# FDD

- Table 28 and Table 29 specify the spectrum emission mask with 5 MHz channel bandwidth.

- Table 28. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (700.5 fc  834.5)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <2.6 | 30 | -13 |
| 2 | 2.6 to 12.5 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 2.585 MHz. The first measurement position with a 100 kHz filter is at f equals to 2.650 MHz; the last is at f equals to 12.450 MHz.

- Table 29. Spectrum Emission for 5 MHz Bandwidth Band Class 7 (834.5 fc  859.5)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <7.5 | 5 | 1.6 |
| 2 | 7.5 to 12.5 | 2 | -10 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 5 MHz. The first measurement position with a 2 MHz filter is at f equals to 8.5 MHz; the last is at f equals to 11.5 MHz.

- Table 30 and Table 31 specify the spectrum emission mask with 10 MHz channel bandwidth.

- Table 30. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (703 fc < 837)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5.0 to <5.1 | 30 | -13 |
| 2 | 5.1 to 25.0 | 100 | -13 |

Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 5.015 MHz; the last is at f equals to 5.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 5.150 MHz; the last is at f equals to 24.950 MHz.

- Table 31. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (837 fc  857)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <10 | 5 | 1.6 |
| 2 | 10 to <15 | 2 | -10 |
| 3 | 15 to 25 | 1 | -25 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 7.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 11 MHz; the last is at f equals to 14 MHz. The first measurement position with a 1 MHz filter is at f equals to 15.5 MHz; the last is at f equals to 24.5 MHz.

- Table 32 and Table 33 specify the spectrum emission mask with 20 MHz channel bandwidth.

Table 32. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (708 fc  842)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10.0 to <10.1 | 30 | -13 |
| 2 | 10.1 to 50.0 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 10.015 MHz; the last is at f equals to 10.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 10.150 MHz; the last is at f equals to 49.950 MHz.

- Table 33. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (842 fc  852)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <15 | 5 | 1.6 |
| 2 | 15 to <20 | 2 | -10 |
| 3 | 20 to 50 | 1 | -25 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 12.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 16 MHz; the last is at f equals to 19 MHz. The first measurement position with a 1 MHz filter is at f equals to 20.5 MHz; the last is at f equals to 49.5 MHz.

# Band Class 8

- The Spectrum Emission Mask for 5 MHz bandwidth is specified in Table 34.

- Table 34. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 8

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | -13 |
| 2 | 3.5 to  12.5 | 1000 | -13 |

- The Spectrum Emission Mask for 10 MHz bandwidth is specified in Table 35.

- Table 35. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 8

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to < 6 | 100 | -13 |
| 2 | 6 to  25 | 1000 | -13 |

- The Spectrum Emission Mask for 20 MHz bandwidth is specified in Table 36.

- Table 36. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 8

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to <15 | 1000 | -10 |
| 3 | 15 to <30 | 1000 | -13 |
| 4 | 30 to 50 | 1000 | -25 |

# Base Station

The spectrum masks of Table 37, Table 38 anTBD are applicable to all bands and all region u.nless specific mask for a band or a region is specified in a relevant sub section of Section 6.2.

Table 37. Spectrum Emission Mask for 5 MHz Bandwidth

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <7.5 | 100 | -7-7(∆f-2.55)/5 |
| 2 | 7.5 to 12.5 | 100 | -14 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 2.550 MHz; the last is at f equals to 12.450 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 38. Spectrum Emission Mask for 10 MHz Bandwidth

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <10 | 100 | -7-7(∆f-5.05)/5 |
| 2 | 10 to <15 | 100 | -14 |
| 3 | 15 to 25 | 1000 | -13 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 5.05 MHz; the last is at f equals to 14.95 MHz. The first measurement position with a 1 MHz filter is at f equals to 15.5 MHz; the last is at f equals to 24.5 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 39. Spectrum Emission Mask for 20 MHz Bandwidth

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <15 | 100 | -7-7(∆f-10.05)/5 |
| 2 | 15 to <20 | 100 | -14 |
| 3 | 20 to 50 | 1000 | -13 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 10.05 MHz; the last is at f equals to 19.95 MHz. The first measurement position with a 1 MHz filter is at f equals to 20.5 MHz; the last is at f equals to 49.5 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

    - Band Class 3

# TDD

The Spectrum Emission Mask of Table 40, Table 42 and Table 44 apply to US region.

Table 40. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 2.5  f  3.5 MHz | 13 dBm | 50 kHz |
| 3.5  f  12.5 MHz | 13 dBm | 1 MHz |

– Japan

Table 42. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 5  f  6 MHz | 13 dBm | 100 kHz |
| 6  f  25 MHz | 13 dBm | 1 MHz |

– Japan

.

Table 44. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 10  f  11 MHz | 13 dBm | 200 kHz |
| 11  f  50 MHz | 13 dBm | 1 MHz |

The Spectrum Emission Mask of Table 45 and Table 46 apply to Japan region.

Table 45. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3-Japan

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 7.5 MHz  f < 12.25 | 151.4 × (f 7.5) dBm | 1 MHz |
| 12.25  f  22.5 MHz | 22 dBm | 1 MHz |

Table 46. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3-Japan

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 15  f  25 MHz | 22 dBm | 1 MHz |

      - FDD

The Spectrum Emission Mask of Table 47, Table 48 and Table 49 apply to US region.

Table 47. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3-US

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 2.5  f  3.5 MHz | 13 dBm | 50 kHz |
| 3.5  f  12.5 MHz | 13 dBm | 1 MHz |

Table 48. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3-US

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 5  f  6 MHz | 13 dBm | 100 kHz |
| 6  f  25 MHz | 13 dBm | 1 MHz |

Table 49. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3-US

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 10  f  11 MHz | 13 dBm | 200 kHz |
| 11  f  50 MHz | 13 dBm | 1 MHz |

3

# Band Class 5

# TDD

The Spectrum Emission Mask for 5, 10 and 20 MHz bandwidth sizes are specified in Table 53 and Table 54. Table 53 specifies breakpoints of the underlying piecewise linear power spectral density mask.  This mask is a relative mask and conditionally applicable depending on the base station Pnom power levee 46 specifies the emission levels of an underlying piecewise step function appliable conditionally only to some of Pnom power levls.

Table 53. Relative Transmit Spectral Power Density Mask

| Power | Frequency Offset |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0.5*BW | 0.71*BW | 1.06*BW | 2.0*BW | 2.5*BW |
| 39 dBm  Pnom | -20 dB | -27 dB | -32 dB | -50dB | -50dB |
| 33 dBm  Pnom ≤39 dBm | -20 dB | -27 dB | -32 dB | -50 dB + (39 dBm - Pnom) | Refer to Table 54 |

Table 54. Absolute Spectral Emission Mask

| Power | Frequency Offset |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0.50 BW  f  0.71 BW | 0.71 BW  f  1.06 BW | 1.06 BW  f  2.00 BW | 2.00 BW  f  2.50 BW |
| 33 dBm  Pnom ≤ 39 dBm | Refer to Table 53 | Refer to Table 53 | Refer to Table 53 | -21 + x dBm/MHz |
| Pnom ≤ 33 dBm | -5.5 dBm/MHz | -5.5 dBm/MHz | -23.5 dBm/MHz | -23.5 dBm/MHz |

Notes:

- In 6, x = -10 log(BW/10)

# Band Class 6

# Frequency Sub-band 1710-1755/2110-2155 MHz

The Spectrum Emission Mask of Table 55 and Table 56 and Table 52 apply to US regin Table 55.

Table 55. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz)

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port. |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | -13 |
| 2 | 3.5 to  12.5 | 1000 | -13 |

Table 56. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz)

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to < 6 | 100 | -13 |
| 2 | 6 to  25 | 1000 | -13 |

Table 57. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz)

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 10  f  11 MHz | 13 dBm | 200 kHz |
| 11  f  50 MHz | 13 dBm | 1 MHz |

# . Spectrum Emission Mask for 5 MHz Bandwidth Band Class 6

. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 6

0 MHz Bandwidth Band Class 6

|  | Frequency offset f from channel center (MHz) |  |  |
| --- | --- | --- | --- |

    - Band Class 7

The Spectrum Emission Mask of Table 61, Table 62 and Table 63 apply to US region.

- Table 61. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7-US

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to < 2.6 | 30 | -13 |
| 2 | 2.6 to 12.5 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 2.585 MHz. The first measurement position with a 100 kHz filter is at f equals to 2.650 MHz; the last is at f equals to 12.450 MHz.

- Table 62. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7-US

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5.0 to < 5.1 | 30 | -13 |
| 2 | 5.1 to  25.0 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 5.015 MHz; the last is at f equals to 5.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 5.150 MHz; the last is at f equals to 24.950 MHz.

- Table 63. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7-US

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10.0 to <10.1 | 30 | -13 |
| 2 | 10.1 to  50.0 | 100 | -13 |

The Spectrum Emission Mask of Table 70, Table 71 and Table 72 apply to Europe region.

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 10.015 MHz; the last is at f equals to 10.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 10.150 MHz; the last is at f equals to 50.950 MHz.

- 7

- 7

- 7

- 8

- Notes:

- 8

- Notes:

- 8

|  | - Frequency offset f from channel center (MHz) |  |  |
| --- | --- | --- | --- |

Table 70. Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7-Europe

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <7.5 | 100 | -7-7(∆f-2.55)/5 |
| 2 | 7.5 to 12.5 | 100 | -14 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 2.550 MHz; the last is at f equals to 12.450 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 71. Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7-Europe

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <10 | 100 | -7-7(∆f-5.05)/5 |
| 2 | 10 to <15 | 100 | -14 |
| 3 | 15 to 25 | 100 | -13 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 5.05 MHz; the last is at f equals to 24.95 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 72. Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7-Europe

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <15 | 100 | -7-7(∆f-10.05)/5 |
| 2 | 15 to <20 | 100 | -14 |
| 3 | 20 to 50 | 100 | -13 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 10.05 MHz; the last is at f equals to 49.95 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Annex B: Proposed Responses to some Open Items in “[DRAFT] SUBMISSION OF A CANDIDATE IMT-ADVANCED RIT BASED ON IEEE 802.16 (PART I)”

| 4.2.3.2.10.2 | Provide details of any power control scheme included in the proposal, for example: [In downlink, power levels can be set in a per-allocation basis and detail of power setting is implementation specific.] For uplink power control, details are provided below. {Response to this question is not within scope of WiMAX Forum. It is suggested to refer to the relevant sections in AWD-D1.} - Power control step size (dB) 0.5 dB - Power control cycles per second Maximum one power control command per subframe that translates into 1600 Hz. - Power control dynamic range (dB) The power adjustment range is -0.5 dB to 1.0 dB. The minimum power control dynamic range required is 45 dB. - Minimum transmit power level with power control The minimum transmit power of an MS with power control is -22 dBm assuming 23 dBm declared maximum transmit power and minimum 45 dB dynamic range. The power levels indicated here are at the antenna port and do not reflect the effect of antenna gain. - Associated signaling and control messages. To maintain at the BS a power density consistent with the modulation and coding rate used by each MS, the BS may change the MS’s TX power through direct power adjustment/control signaling. The uplink power control is signaled as part of downlink control structure. See the illustrations in item 4.2.3.2.7.1 and more details in section 15.3.6.2.2.4 of [1]. |
| --- | --- |

| 4.2.3.2.11 | Power classes |
| --- | --- |
| 4.2.3.2.11.1 | Mobile station emitted power |
| 4.2.3.2.11.1.1 | What is the radiated antenna power measured at the antenna (dBm)? The typical conducted output transmit power, measured as the mean power delivered to the antenna port, according to Power Class 2 of Section YYY is less than or equal to 23 dBm. The total radiated antenna power, assuming 0 dBi antenna gain is less than or equal to 23 dBm. |
| 4.2.3.2.11.1.2 | What is the maximum peak power transmitted while in active or busy state? The maximum peak power transmitted while in active or busy state is 23 dBm. |
| 4.2.3.2.11.1.3 | What is the time averaged power transmitted while in active or busy state? Provide a detailed explanation used to calculate this time average power. [Editor Note: TBD] {A complete response to this TBD possibly requires SLS simulation within the framework of ITU specified Test Environments that is currently done in IEEE.} |
| 4.2.3.2.11.2 | Base station emitted power |
| 4.2.3.2.11.2.1 | What is the base station transmit power per RF carrier? Base Stations transmit power, measured as the mean power delivered to the antenna port, depends on various factors including those related to the deployment scenario and possible local Regulatory requirements. |
| 4.2.3.2.11.2.2 | What is the maximum peak transmitted power per RF carrier radiated from antenna? The maximum peak transmitted power per RF carrier radiated from antenna depends on the Base Station peak transmit power at the antenna port and the antenna gain. The peak transmitted power per RF carrier radiated from antenna is within a range of values properly considered to cover target deployment scenarios. This value may be subject to local Regulatory limitations. |
| 4.2.3.2.11.2.3 | What is the average transmitted power per RF carrier radiated from antenna? [Editor Note: TBD] {A complete response to this TBD possibly requires SLS simulation within the framework of ITU specified Test Environments that is currently done in IEEE.} |