| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Summary of Simulator Calibrations for IMT-Advanced |  |
| Date Submitted | 2009-09-24 |  |
| Source(s) | Jeongho Park, Jason Junsung Lim, Taeyoung Kim, Sudhir Ramakrishna, Jaeweon Cho, Hokyu Choi Samsung Electronics Apostolos Papathanassiou, Alexei Davydov, Chunming Han, Roshni Srinivasan,Sassan Ahmadi, Takashi Shono Intel Wookbong Lee, Jinsam Kwak, Min-seok Oh, Sunam Kim, Sungho Park LG Mark Cudak, Frederick Vook Motorola I-Kang Fu, Kelvin Chou, Yu-Hao Chang, Pei-Kai Liao, Paul Cheng MediaTek Sunil Vadgama, Michiharu Nakamura, Hua Zhou Fujitsu Laboratories Ltd Mitsuo Nohara, Satoshi Imata KDDI R&D Laboratories Masoud Olfat Clearwire Kwangjae Lim, Wooram Shin ETRI Yan-Xiu Zheng, Frank Ren, PA Ting, Chang-Lan Tsai, Yutao Hsieh ITRI Hongwei Yang Alcatel-Lucent Shanghai Bell Tetsu Ikeda, Takashi Mochizuki NEC Kenji Saito UQ Communications | jeongho.jh.park@samsung.com apostolos.papathanassiou@intel.com wbong@lge.com Mark.Cudak@motorola.com IK.Fu@mediatek.com michi.nakama@jp.fujitsu.com mi-nohara@kddilabs.jp masoud.olfat@clearwire.com kjlim@etri.re.kr zhengyanxiu@itri.org.tw Hongwei.yang@alcatel-sbell.com.cn t-ikeda@ap.jp.nec.com kenji@uqc.jp |
| Re: | IEEE802.16 Session #63.5 in Hawaii, USA |  |
| Abstract | The purpose of this contribution is to provide calibration report for IEEE 802.16m performance self-evaluation results. |  |
| Purpose | To provide details of calibration works in IEEE 802.16m performance self-evaluation results |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Summary of Simulator Calibrations for IMT-Advanced

Jeongho Park, Jason Junsung Lim, Taeyoung Kim, Sudhir Ramakrishna, Jaeweon Cho, Hokyu Choi

Samsung Electronics

Apostolos Papathanassiou,Alexei Davydov, Chunming Han, Roshni Srinivasan, Sassan Ahmadi, Takashi Shono

Intel

Wookbong Lee, Jinsam Kwak, Min-seok Oh, Sunam Kim, Sungho Park

LG

Mark Cudak, Frederick Vook

Motorola

I-Kang Fu, Kelvin Chou, Yu-Hao Chang, Pei-Kai Liao, Paul Cheng

MediaTek

Sunil Vadgama, Michiharu Nakamura, Hua Zhou

Fujitsu

Mitsuo Nohara, Satoshi Imata

KDDI R&D Laboratories

Masoud Olfat

Clearwire

Kwangjae Lim, Wooram Shin

ETRI

Yan-Xiu Zheng, Frank Ren, PA Ting, Chang-Lan Tsai, Yutao Hsieh

ITRI

Kenji Saito

UQ Communications

Hongwei Yang

Alcatel-Lucent Shanghai Bell

Tetsu Ikeda, Takashi Mochizuki

NEC Corporation

Kenji Saito

UQ Communications

# 1. Introduction

This contribution summarizes the joint calibration activities for evaluating the IEEE 802.16m air interface using the IMT-Advanced evaluation methodology. The system level simulation study where the performance is examined by system level simulations is an essential part for the purpose of self-evaluation of the technical performance as a proponent for IMT-advanced. The system-level performance has been evaluated through multiple tests considering various assumptions and environments in order to increase the reliability of the calibration work. Therefore, the evaluation methodologies, assumptions and configurations to perform system level simulations have been discussed.

In the tables of Sections 2 to 4, the different sources of the evaluation results correspond to contributors from the different affiliations according to the following mapping:

| Source 1 | Intel Corporation |
| --- | --- |
| Source 2 | Clearwire |
| Source 3 | ETRI |
| Source 4 | Fujitsu Laboratories Ltd. |
| Source 5 | KDDI R&D Laboratories |
| Source 6 | LG Electronics |
| Source 7 | MediaTek Inc. |
| Source 8 | Motorola Inc. |
| Source 9 | Samsung Electronics |
| Source 10 | Alcatel-Lucent Shanghai Bell |
| Source 11 | UQ Communications |
| Source 12 | NEC Corporation |
| Source 13 | ITRI |

# 2. Test environments and evaluation configurations

The four mandatory test environments, i.e., Indoor (InH), Urban micro-cell (UMi), Urban macro-cell (UMa), and Rural macro-cell (RMa), are considered. The baseline evaluation configuration parameters and additional parameters are aligned with [1] and the summary is described as Table 1.

Table 1 Evaluation Parameters

| Deployment scenario for the evaluation process | Indoor hotspot | Urban micro-cell | Urban macro-cell | Rural macro-cell |
| --- | --- | --- | --- | --- |
| Layout | Indoor floor | Hexagonal grid | Hexagonal grid | Hexagonal grid |
| Inter-site distance | 60 m | 200 m | 500 m | 1 732 m |
| Base station (BS) antenna height | 6 m, mounted on ceiling | 10 m, below rooftop | 25 m, above rooftop | 35 m, above rooftop |
| Number of BS antenna elements | Up to 4 rx Up to 4 tx | Up to 4 rx Up to 4 tx | Up to 4 rx Up to 4 tx | Up to 4 rx Up to 4 tx |
| Total BS transmit power | 24 dBm for 40 MHz, 21 dBm for 20 MHz | 41 dBm for 10 MHz, 44 dBm for 20 MHz | 46 dBm for 10 MHz, 49 dBm for 20 MHz | 46 dBm for 10 MHz, 49 dBm for 20 MHz |
| User terminal (UT) power class | 21 dBm | 24 dBm | 24 dBm | 24 dBm |
| UT antenna system | Up to 2 tx Up to 2 rx | Up to 2 tx Up to 2 rx | Up to 2 tx Up to 2 rx | Up to 2 tx Up to 2 rx |
| Minimum distance between UT and serving cell(2) | >= 3 m | >= 10 m | >= 25 m | >= 35 m |
| Carrier frequency (CF) for evaluation (representative of IMT bands) | 3.4 GHz | 2.5 GHz | 2 GHz | 800 MHz |
| Outdoor to indoor building penetration loss | N.A. | Table A1-2 in [1] | N.A. | N.A. |
| Outdoor to in-car penetration loss | N.A. | N.A. | 9 dB (LN, σ = 5 dB) | 9 dB (LN, σ = 5 dB) |
| User distribution | Randomly and uniformly distributed over area | Randomly and uniformly distributed over area. 50% users outdoor (pedestrian users) and 50% of users indoors | Randomly and uniformly distributed over area. 100% of users outdoors in vehicles | Randomly and uniformly distributed over area. 100% of users outdoors in high speed vehicles |
| User mobility model | Fixed and identical speed ¦v¦ of all UTs, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UTs, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UTs, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UTs, randomly and uniformly distributed direction |
| Channel model | InH Indoor hotspot (LoS, NLoS) | UMi Urban micro (LoS, NLoS, Outdoor-to-indoor) | UMa Urban macro (LoS, NLoS) | RMa Rural macro (LoS, NLoS) |
| UT speeds of interest | 3 km/h | 3 km/h | 30 km/h | 120 km/h |
| BS noise figure | 5 dB | 5 dB | 5 dB | 5 dB |
| UT noise figure | 7 dB | 7 dB | 7 dB | 7 dB |
| BS antenna gain (boresight) | 0 dBi | 17 dBi | 17 dBi | 17 dBi |
| UT antenna gain | 0 dBi | 0 dBi | 0 dBi | 0 dBi |
| Thermal noise level | –174 dBm/Hz | –174 dBm/Hz | –174 dBm/Hz | –174 dBm/Hz |
| Cable loss (or feeder loss) | 2 dB | 2 dB | 2 dB | 2 dB |
| Evaluated service profiles | Full buffer best effort | Full buffer best effort | Full buffer best effort | Full buffer best effort |
| Simulation bandwidth | 20 + 20 MHz (FDD), or 40 MHz (TDD) | 10 + 10 MHz (FDD), or 20 MHz (TDD) | 10 + 10 MHz (FDD), or 20 MHz (TDD) | 10 + 10 MHz (FDD), or 20 MHz (TDD) |
| Number of users per cell | 10 | 10 | 10 | 10 |

Other configurations and assumptions not described in Table 1 are specified in [2]. The antenna characteristics, channel model approach, and drop concept are aligned with [1]. Additional configuration information is provided in this document when the description in [1] can be interpreted in multiple ways.

- The user drop concept

For user dropping, it is stated in [1] that users are dropped independently with uniform distribution over predefined area of the network layout throughout the system. However, this statement may have two different interpretations. Case 1: Users can be dropped so that the number of users per sector equals 10, i.e., 570 users are dropped over the area covered by 57 sectors; Case 2: 570 users are dropped uniformly over the whole area and the serving sector for each user is determined after all users are dropped. We consider user dropping so that equal number of users is dropped in each sector, i.e., Case 1. This implies that a user is re-dropped when the number of users connected to the serving sector exceeds the target number.

- Cell selection

A user is connected to the sector which has highest geometry. It is implemented by finding a sector transmitting the strongest signal to the user among the neighboring sectors. Once a user is connected to a serving sector at the dropping stage, the connection is consistent over whole simulation time.

- Path loss model

Additional clarification to calculate path loss for outdoor-to-indoor (O-to-I) case in UMi environment is necessary, since it is ambiguous on generating path loss and channel model parameters of indoor users whether it should be based on LoS or NLoS. For this case, LoS or NLoS is determined by using the LoS probability to calculate path loss, while the NLoS condition for O-to-I channel model parameter is applied to generate small-scale parameters.

- Antenna pattern & Configuration

Two kinds of BS antenna patterns are supported. One is horizontal antenna pattern and the other is vertical antenna pattern. The horizontal antenna pattern used for each BS sector is specified as:

where A() is the relative antenna gain (dB) in the direction , 180º    180º, and min [.] denotes the minimum function, 3dB is the 3 dB beamwidth (corresponding to 3dB 70º), and Am = 20 dB is the maximum attenuation.

A similar antenna pattern is for elevation in simulations that need it. In this case the antenna pattern is given by:

where Ae() is the relative antenna gain (dB) in the elevation direction, =atan(hBS/d) , −90º    90º,  3dB is the elevation 3 dB value (3dB=15º),  tilt is the electrical tilt angle, which is specified according to deployment scenario. Note that antenna tilting is applied in both downlink and uplink. Table 1 shows the antenna tilting angle for each test environment used in the evaluation.

Table 2 Antenna Tilting Angle

| Deployment scenario for the evaluation process | Indoor hotspot | Urban micro-cell | Urban macro-cell | Rural macro-cell |
| --- | --- | --- | --- | --- |
| Tilting Angle (deg) | 0 | 12 | 12 | 6 |

Two kinds of uniform linear array antenna (ULA) configuration are supported i.e., co-polarized ULA and cross-polarized ULA. For the purpose of evaluation, we use the co-polarized ULA as the basic antenna configuration.

- Cluster beam gain

The antenna attenuation pattern needs to be applied when there are multiple clusters arrived or departed with different angles. For the purpose of calibration the field pattern based on LoS direction is applied for all clusters so that each cluster experiences the same field pattern.

- Large-scale parameters

Spatial large-scale parameters (LSP) are used as control parameters to generate small-scale fading. It is noted that different MSs located at the same spatial position experience the same LSP parameters. One of the following three methods can be used for generating LSP parameters. The difference of the methods is minor with respect to the system performance.

- Method 1

- Make lattices with dcorr over the entire area.

- Generate 4X1 (or 5X1) normal distributed and independent random vector for LSP parameters at each lattice point.

- When a user is dropped, 4X1 (or 5X1) random vector are obtained by linear interpolation with random vectors at the four closest lattice points from the user position.

- Obtain 4X1 (or 5X1) cross-correlated random vector by multiplying the random vectors with square root of 4X4 (or 5X5) correlation matrix derived from Table A1-7 in [1].

- Modify each random variable of the vector so that the mean and variance becomes the mean and variance for each LSP.

- Transform dB scale to linear scale.

- Method 2

- Make fine lattices over the entire area.

- Generate 4X1 (or 5X1) normal distributed and independent random vector at each lattice point.

- Convert random vector so that each element be auto-correlated using FIR filtering with impulse response

H(d) = exp(-d/dcorr) where d is the distance between two lattice points and dcorr is the correlation distance.

- When a user is dropped, the random vector at the closest lattice point is selected.

- Obtain 4X1 (or 5X1) cross-correlated random vector by multiplying the random vectors with square root of 4X4 (or 5X5) correlation matrix derived from Table A1-7 in [1].

- Modify each random variable of the vector so that the mean and variance becomes the mean and variance for each LSP.

- Transform dB scale to linear scale.

- Method 3

- When a user is dropped, a random vector 4X1 (or 5X1) is generated in which each is independent.

- Modify each random variable of the vector so that the mean and variance becomes the mean and variance for each LSP.

- Transform dB scale to linear scale.

# 3. Geometry Calibration

In the following figures, the downlink geometry (SINR distributions) graphs are presented for the mandatory IMT-Advanced test environments. The results show that the distributions derived from each source are well matched with each other for all test environments.

# 4. Link Adaptation and SINR-PER Mapping

When an HARQ burst is sent with a certain MCS level, the effective SINR of the received signal is calculated in the system level simulations. The decision of packet error is made randomly by considering the packet error probability for a given effective SINR. The packet error probability per SINR for a given MCS is predetermined where the average error probability is derived from link level simulations based on the AWGN assumption. The effective SINR at the receiver side is obtained according to the methodology in [2].

The IEEE 802.16m coding rate is determined by the number of data tones of assigned resources and MIMO rank to send a particular HARQ burst due to the used rate matching scheme. Realistic modeling requires considering different set of MCS levels per resource allocation, per pilot pattern, and per MIMO rank which is extremely laborious for system level simulation. Some approximations are unavoidable to model link adaptation with rate matching scheme for the sake of simplicity. The difference due to this approximation does not affect significantly the performance.

For the sake of calibration, 16 MCS levels out of total 32 MCS levels are agreed to be used for system level simulations. Table 3 shows the selected 16 MCS levels and corresponding data burst size as color-coded shadow which the total 8 LRUs and 4 subframes can accomodate with rank 1 at once. Note that, in this case, 12 pilot tones are assumed to be included in a PRU and consequently total 3072 tones can be used for data burst transmission.

Table 3 Used 16 MCS levels (color-coded rows) and corresponding data burst sizes

| I_SizeOffset | I_minimal_size | Modulation order | Data burst size idx | Data burst size (byte) | Number of coded block | Coding rate |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 20 | 2 | 20 | 64 | 1 | 0.083333333 |
| 1 | 20 | 2 | 21 | 71 | 1 | 0.092447917 |
| 2 | 20 | 2 | 22 | 80 | 1 | 0.104166667 |
| 3 | 20 | 2 | 23 | 90 | 1 | 0.1171875 |
| 4 | 20 | 2 | 24 | 100 | 1 | 0.130208333 |
| 5 | 20 | 2 | 25 | 114 | 1 | 0.1484375 |
| 6 | 20 | 2 | 26 | 128 | 1 | 0.166666667 |
| 7 | 20 | 2 | 27 | 145 | 1 | 0.188802083 |
| 8 | 20 | 2 | 28 | 164 | 1 | 0.213541667 |
| 9 | 20 | 2 | 29 | 181 | 1 | 0.235677083 |
| 10 | 20 | 2 | 30 | 205 | 1 | 0.266927083 |
| 11 | 20 | 2 | 31 | 233 | 1 | 0.303385417 |
| 12 | 20 | 2 | 32 | 262 | 1 | 0.341145833 |
| 13 | 20 | 2 | 33 | 291 | 1 | 0.37890625 |
| 14 | 20 | 2 | 34 | 328 | 1 | 0.427083333 |
| 15 | 20 | 2 | 35 | 368 | 1 | 0.479166667 |
| 16 | 20 | 2 | 36 | 416 | 1 | 0.541666667 |
| 17 | 20 | 2 | 37 | 472 | 1 | 0.614583333 |
| 18 | 20 | 2 | 38 | 528 | 1 | 0.6875 |
| 19 | 20 | 4 | 39 | 600 | 1 | 0.390625 |
| 20 | 20 | 4 | 40 | 656 | 2 | 0.427083333 |
| 21 | 20 | 4 | 41 | 736 | 2 | 0.479166667 |
| 22 | 20 | 4 | 42 | 832 | 2 | 0.541666667 |
| 23 | 20 | 4 | 43 | 944 | 2 | 0.614583333 |
| 24 | 20 | 6 | 44 | 1056 | 2 | 0.458333333 |
| 25 | 20 | 6 | 45 | 1200 | 3 | 0.520833333 |
| 26 | 20 | 6 | 46 | 1416 | 3 | 0.614583333 |
| 27 | 20 | 6 | 47 | 1584 | 3 | 0.6875 |
| 28 | 20 | 6 | 48 | 1800 | 3 | 0.78125 |
| 29 | 20 | 6 | 49 | 1888 | 4 | 0.819444444 |
| 30 | 20 | 6 | 50 | 2112 | 4 | 0.916666667 |
| 31 | 20 | 6 | 51 | 2400 | 4 | 1.041666667 |

The following figure shows the link level performance of total 16 MCS levels in AWGN channel.

5. SU-MIMO results for calibration

Table 4 is used for the downlink SU-MIMO calibration work.

Table 4 Evaluation Parameters for Single User MIMO Calibration

| Parameters | Value |
| --- | --- |
| Duplex | FDD |
| Bandwidth | DL:10MHz |
| Number of cells | 57 cells |
| Scheduler | PF User selection happens for every resource allocation |
| Transmission scheme | SU-MIMO, Codebook based |
| Antenna configuration | 4x2 a. For BS side Uncorrelated co-polarized: Co-polarized antennas separated 4 wavelengths (illustration for 4 Tx: ¦ ¦ ¦ ¦) b. For MS side Vertical polarized and 0.5 wavelength separation |
| Receiver Type | MMSE |
| HARQ | DL: CC, Max 4 retransmission |
| Link adaptation and feedback | CQI measurement : perfect CQI feedback delay : 5ms No feedback transmission error Every MS reports to BS information which includes CQI, PMI of subbands. One subband represents four contiguous PRUs. Since there are total 48 PRUs in 10MHz BW, total 12 subbands exists. |
| Channel estimation | Ideal |
| Control overhead | Approximately 28.6% DL control overhead is assumed. |

Table 5 and Table 6 show the system level simulation results for cell (sector) spectral efficiency and cell-edge user spectral efficiency. Various results from different sources show similar values both for cell and edge-edge user spectral efficiency. As shown in Tables 5 and 6, the differences from the various sources are well within tolerable levels for system level simulation studies.

Table 5 Downlink cell spectral efficiency comparison

| Contributor | Source 6 | Source 9 | Source 1 | Source 7 | Source 3 | Average |
| --- | --- | --- | --- | --- | --- | --- |
| Sector S.E in UMi | 1.97 | 1.96 | 2.0 | 1.82 | 2.06 | 1.96 |

Table 6. Downlink cell-edge user spectral efficiency comparison

| Contributor | Source 6 | Source 9 | Source 1 | Source 7 | Source 3 | Average |
| --- | --- | --- | --- | --- | --- | --- |
| Edge S.E in UMi | 0.0827 | 0.0814 | 0.062 | 0.058 | 0.078 | 0.0724 |

# 6. Conclusion

Based on the detailed information provided in this contribution, the system level simulator calibration results for the purpose of self-evaluation of IEEE 802.16m were shown to be well aligned across all contributing sources.

# References

[1] Report ITU-R M.2135, “Guidelines for evaluation of radio interface technologies for IMT-Advanced”, 2008.

[2] IEEE 802.16m Evaluation Methodology Document (EMD), IEEE 802.16m-08/004r6, January 2009.