| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for additional requirements for IEEE 802.16p for low power consumption |  |
| Date Submitted | 2010-11-05 |  |
| Source(s) | Andreas Maeder, Peter Rost, Xavier Perez-Costa, Linghang Fan, Hassan Alkanani, Nader Zein NEC | E-mail: andreas.maeder@neclab.eu peter.rost@neclab.eu xavier.perez-costa@neclab.eu linghang.fan@eu.nec.com hassan.alkanani@eu.nec.com nader.zein@eu.nec.com |
| Re: | IEEE 802.16p call for requirements |  |
| Abstract | Add requirements for low power consumption |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for additional requirements for IEEE 802.16p for low power consumption

Andreas Maeder, Peter Rost, Xavier Perez-Costa, Linghang Fan, Hassan Alkanani, Nader Zein

NEC

# Motivation

1. One-Way Traffic and Asymmetric Traffic

The current IEEE 802.16p SRD draft document [1] contains functional requirements for low-power consumption. However, one-way traffic or strongly asymmetric traffic is not yet mentioned explicitly in the document, although it is listed as a requirement in the M2M communication study report in [2]. Asymmetric traffic is expected to be one of the main characteristic of M2M traffic which originates from typical use cases like smart metering or health monitoring. An IEEE 802.16 WirelessMAN network which supports M2M traffic should be optimized for this kind of traffic in order to reduce signaling overhead, and to reduce power consumption.

We therefore propose to add a corresponding requirement for asymmetric traffic in the SRD.

2. Low complexity transmission schemes

Low power consumption is one of the key requirements for M2M communication systems. Battery-powered M2M devices depend on long battery lives to reduce maintenance time and costs. One possible solution is to employ transmission schemes with lower complexity to reduce energy consumption for signal processing.

We therefore propose to include a corresponding requirement in the SRD.

References

[1]	IEEE, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD)  (initial working document revised)”, Sept. 2010

[2]	IEEE, “Machine to Machine (M2M) Communication Study Report (Draft)”, May 2005

Remedy:  Add the following lines in IEEE 802.16ppc-10/0011r2, Section 6.1:

6.1.5 	The 802.16p system shall provide optimized operation for one-way traffic and for strongly asymmetric traffic.

6.1.6 	The 802.16p system shall support low complexity transmission schemes.