| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed IEEE 802.16p Performance Requirements for Network Entry by Large Number of Devices |  |
| Date Submitted | 2010-11-08 |  |
| Source(s) | N. Himayat, S. Talwar, K. Johnsson, Intel Corp. S. Andreev, (Tampere University of Technology) O. Galinina (SPIIRAS) A. Turlikov (SUAI) | E-mail: nageen.himayat@intel.com |
| Re: | IEEE 802.16p performance requirements related to network entry by large number of devices |  |
| Abstract | This contribution analyzes the performance of initial random access (ranging) by a large number of M2M devices. Requirements for the IEEE 802.16p Systems Requirements Document (SRD) are proposed to address the resulting performance issues observed. |  |
| Purpose | For review and adoption into the IEEE 802.16p SRD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Introduction

Contribution IEEE C80216ppc-10_0049r1, [1], discussed functional requirements relevant to network entry by large number of devices.  Focusing on the smart metering applications, the contribution highlighted several use cases that will require network access by large number of devices in a relatively short period of time.  It was shown that for some cases such as simultaneous reporting of outage alarms by a large number of smart meters can potentially strain the existing random access capacity of IEEE 802.16m standard [2]. In this contribution we extend the simple analysis presented in [1] to model the performance of the binary back-off algorithm used for network entry in 802.16 systems.  Our analysis shows that for certain scenarios, the random access performance of this algorithm can be fairly poor. Therefore, it will be important to extend the capabilities of the 802.16 standard to support a large number of M2M devices in the network. Some of these extended capabilities have been captured as functional requirements in the IEEE 802.16p Systems Requirements Document (SRD) [3].  Here we propose specific performance requirements that apply to network entry by large number of devices.

This contribution briefly reviews the uses cases and the analysis described in C80216ppc-10_0049r1. The analysis is then extended to capture the binary exponential random back-off procedure used for network entry in IEEE 802.16 systems. Performance results are included for the various assumptions on the number of devices and the arrival rate of random access attempts.  The results are then analyzed to check for resulting requirements for the 802.16p standard.  Finally, the specific text for inclusion in the performance requirements section of the IEEE 802.16p SRD is described.

# Scenarios Covering Network Entry by Large Number of Devices

Machine-to-machine applications (M2M) are expected to support very large number of devices per cell.  Using the Smart Metering application as a reference, contribution [1] estimates the number of devices supported by a sector to be in the broad range of 3000-35000 devices/sector.  In a number of applications the devices are characterized by infrequent and delay tolerant traffic [4], so appropriate use of existing network design can support network access by large number of devices. However, there are certain use cases that result in near simultaneous network entry by a very large number of devices that may easily overwhelm system capacity. In the following we describe some of these usage scenarios:

- A large number of devices can transition from idle to active state when there is a large scale outage event, for example a wide-spread power outage event for a smart metering deployment. Here, the smart meters are required to send a “last gasp” alarm notifying the network that they have lost power. Typically, this report needs to be sent out within a few hundred milliseconds, given that smart meters are expected to operate without battery backup and have limited charge to send the message before complete loss of power (see contribution [5] for more details).  This need for simultaneous network entry can result from the need to maintain a large number of devices the network in idle state. Idle state may be preferred due to a variety of reasons such as a) limited address space for supporting a large number of simultaneously active connections (10-35K devices) b) overhead associated with maintenance signaling for active connections and c) reducing device power consumption etc.

- There may also be a surge in network entry when M2M devices attempt to connect back to the network after an outage event. This can occur in a smart metering application after a power outage event.

- Surge in network access attempts can also occur for applications other than smart metering. For example a large number of sensors in a building may attempt to report an alarm event simultaneously in case of a break-in or an emergency event.

# Preliminary Analysis of Initial Random Access Capacity of IEEE 802.16m

The preliminary analysis covered in contribution [1] is reviewed in this section. The network entry procedure requires several steps, which are described in [2]. In the following we focus on the capacity of IEEE 802.16m to handle initial ranging attempts from a large number of devices.

Table 1 describes the number of initial ranging attempts with a contention probability of 1%, which may be supported by 802.16m under various ranging channel configurations supported.

| Case | Number of Contention Channels /second | Maximum (Minimum) # of Codes | Random Access Opportunities /second | Overhead | Number of ranging attempts/s supported w/ contention probability of 1% (0.15 arrival rate) |
| --- | --- | --- | --- | --- | --- |
| 1 | 200 | 32 (8) | 6400 (1600) | 2.85% | 960 (240) |
| 2 | 50 | 32 (8) | 1600 (400) | 0.71% | 240 (60) |
| 3 | 25 | 32 (8) | 800 (200) | 0.35% | 120 (30) |
| 4 | 12.5 | 32 (8) | 400 (100) | 0.18% | 60 (15) |

Table 1: Maximum number of ranging access attempts second supported with a 1% contention probability per preamble, assuming Poisson arrival (see [1] for more details).

Table 2 indicates the ranging access attempts/second, when a large number of devices attempt network entry over a short time interval of interest (the requests are assumed to be uniformly spread over the interval of interest).

| Application | Access Interval of Interest | Ranging Access Attempt/s 12K devices/sector | Ranging Access Attempt/s 35K devices/sector |
| --- | --- | --- | --- |
| Meter Reporting | 5 minute | 40 | 116.7 |
| Meter Reporting | 1 minute | 200 | 583.3 |
| Unsynchronized Alarm Reporting or Network Access | 10 second | 1200 | 3500 |
| Last Gasp Event Reporting | 500 millisecond | 24000 | 70000 |

Table 2: Possible access arrival rates for different scenarios, assuming 12000 and 35000 devices per sector. It can be seen that access attempts resulting from a surge in network arrivals will far exceed the ranging channel capacity of 802.16m.

It is clear from the table above that capacity for initial random access must be enhanced or the random access arrival rate must be controlled to ensure reliable system performance for the extreme scenarios described. However, we note that the above analysis is simplistic in that it requires the first random access attempt to be successful. In practice, a device will attempt network entry several times using the random back-off procedure mandated by the 802.16 standard. Further, the application may not require all messages to get through when faced with a surge in network access attempt. Just a few messages may be sufficient in order to initiate appropriate processing in the network [5].  If a device makes several attempts to access the network, it become important to characterize network entry performance in terms of the delays incurred, the number of tries for successful access and the probability that device will eventually be successful at network entry. In the next section, we provide more detailed simulation analysis of the random 802.16 random back-off under the challenging access rates assumption described in Table 2, and analyze the network access performance across several important metrics of interest.

# Performance Analysis of 802.16m Random Access

This section summarizes the simulation performance of IEEE 802.16m random back-off procedure for initial ranging, under the challenging network entry scenarios discussed in the previous section. Detailed performance results and simulation assumptions are included in the Appendix.  We assume optimistic processing times and work with maximum possible ranging channel capacity defined in Table 1, for most cases. Additional processing delays will be incurred if several ranging attempts are required for calibration as well as if the full extent of the ranging message protocol is include. We also assume that physical layer performance of ranging code detection does not vary with surging random access attempts and will be controlled at a decode error probability of 1%. The assumed system parameters follow the 802.16m specification as much as possible.

We first start by assessing the capacity of the system to support large number of devices by investigating the number of devices and the access rates per device that force the system into outage. Here outage is defined for two different cases a) system is in outage if even a few successful access attempts cannot be supported or conversely the system is not in outage if probability of successful access > 0, b) system is in outage if probability of successful access < 99%. The outage capacity of the system is number of devices that may be supported without experiencing outage conditions. Table 3and Table 4 show the outage capacities for criterion a) and b) respectively. Note that only a discrete set of users are considered in this computation [1K, 10K, 30K, 50K].

Table 3: Capacity of 802.16m systems to support network access by [1K, 10K, 30K, 50K] devices as a function of arrival rate per device of [1/60s, 1/10s, 1/0.5s].  Here a minimal criterion is used in that the capacity of N devices is supported if out of N devices accessing the network, more than zero are able to access the network (Probability of Access Success > 0).

Table 4: Capacity of 802.16m systems to support network access by [1K, 10K, 30K, 50K] devices as a function of arrival rate per device of [1/60s, 1/10s, 1/0.5s].  Here a more realistic criterion is used. Capacity of N devices is supported if out of N devices accessing the network, greater than 99% are able to access the network (Probability of Access Success >= 99%).

It can be seen from the results shown in Table 3 and Table 4, that as the arrival intensity increases with respect to the ranging channel capacity of the system, even 1000 users cannot be supported with 99% access success probability. In fact even increasing the number of access opportunities/second by 5 times, does not improve the outage performance. However, it can be seen that a few attempts can still go through despite the excessive load.

Table 5 shows a more detailed summary of the results obtained across varying number of users, arrival rates, and ranging channel configurations. Complete statistics capturing probability of contention, probability of access success, mean delay for all devices and only the devices that are able to access the network (conditional), and mean number of retries.

Table 5: Summary of performance of IEEE 802.16m random back-off procedure under different assumptions on the network access rates.

The results shown in the above table suggest that when a large number of devices (close to 30K for an arrival rate of 1/10s per device) attempt to access, the network there is very low probability (< 2%) that the network entry attempt will be successful even after the maximum number of retries (16) has been reached.  Results also show that even with lower number of devices (10K), a surge in network access attempts (devices access within 500ms period) will degrade the random access success rates beyond the required reliability rates. Here, even increasing the access opportunities by 5 times will not support 1000 users at 99% access success. Further, if the system is not operating at full capacity (only 400 random access opportunities per slot), a drastic degradation in random access performance is observed, where a 1000 access attempts/second will only be successful 5% of the time. Hence, appropriate methods must be designed to handle the overload caused by a surge in network attempts.  Note that the performance results shown are for fixed back-off parameters. The back-off parameters must be optimized to cope with the varying network load in the system. However, while access rates may be improved by increasing contention window size, the delays incurred by devices will also increase correspondingly.  Further analysis in this area is needed to fully characterize system behavior.

We also note that the average latency of even the devices that are successful at network entry can exceed the minimum control plane latency required for 16m devices (which is 100ms [6]).  Therefore if M2M devices co-exist with other non-M2M devices, the system design must allow for maintaining the latency of random access for non-M2M devices.  Additionally priority access must also be provided for higher priority M2M devices such that their access reliability and latency is not affected by network overload.

# Requirements for the IEEE 802.16p Standard

Several functional requirements related to system improvements needed to manage network access by a large number of devices are captured in the IEEE 802.16p SRD [3]. In particular clauses 6.2.6 and 6.2.6.1 of 802.16p SRD relate to improvements in network access for large number of devices. Requirements that allows for solutions to mitigate network access attempts by a large number of devices are also captured (see clause 6.2.2). Additionally, clause 6.2.8 relates to providing priority access for certain classes of M2M devices, which is essential if priority based M2M services, as well as non-M2M devices are to remain un-affected by the network overload caused by mass network access from M2M devices. In the following we focus on performance requirements relevant for network entry.

- Reliable network entry for large number of devices: 16p systems should provide reliable network access, by supporting an access reliability of > 99%, when the network access rate is less than 3000 access attempts/second. Note that this requirement will ensure satisfactory access performance for unsynchronized network access for up to 30,000 M2M devices. As indicated in earlier section, this requirement aims to increase access rates by 3x, which may be accomplished by increasing the maximum number of random access channels available for use.

- Maintain control plane latency for non-M2M devices: 16p systems should maintain the control plane latency for non-M2M devices (100ms specified in 16m SRD), when M2M devices share the network with other non-M2M devices. This requirement is needed to ensure that performance of non-M2M devices is not significantly degraded if such devices are deployed together with a large number of M2M devices. We note that this requirement is captured by the backward compatibility requirement on 16p systems and therefore no new requirement needs to be added to the 16p SRD.

- Provide prioritized access for certain M2M devices: 16p systems should ensure that high-priority M2M services are able to access the network with the required control plane latency.  While this is an important requirement to specify, insufficient information is available on the range of M2M applications to specify this requirement at this stage. We do note that for certain Smart Grid applications such as Distribution Automation, a tight latency of tens of milliseconds may be needed [8].

- Provide nominal level of random access success rates under network overload conditions: 16p systems should maintain a minimal level of access reliability during “high intensity alarm events” by allowing one ore more devices to access the system with a control plane latency of 100 milliseconds. This requirement is needed to ensure that sufficient number of alarms are recognized by the system to initiate appropriate processing (example power outage reporting for smart meters within 100-250 milliseconds. See [5]).

# Text Proposal

Specific text changes to the 802.16p SRD are indicated below.

---------------------------Begin Text Proposal-------------------------------

# Performance Requirements

## 7.2 Large Number of Devices

### The system shall provide reliable network access, by supporting an access reliability of > 99%, when the network access rate is less than 3000 access attempts/second.

7.1.2  The system shall maintain a minimal level of performance when the access rate exceeds 60,000 attempts/second, by allowing one ore more devices to access the system with a control plane latency of 100 milliseconds.

---------------------------End Text Proposal-------------------------------

# References

- N. Himayat, S. Talwar, K. Johnsson, X. Wang, “Functional Requirements for Network Entry and Random Access by Large Number of Devices,” IEEE C8021.6ppc-10_0049r1, Sep. 2010.

- IEEE P802.16m/D7

- IEEE 80216ppc-10_0011r2, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD),” (initial working document, revised), Sep, 2010.

- IEEE 80216ppc-10_0002r7, “Machine-to-Machine (M2M) communication study report,” May, 2010.

- R. Golla, W. Bell, M. Lin, N. Himayat, S. Talwar, K. Johnsson, “Power Outage Alarm for Smart Metering Applications,” IEEE C80216ppc-10_0054r1, Sep. 2010.

- IEEE 802.16m SRD.

- N. Himayat, S. Mohanty, K. Johnsson, S. Talwar, “Analysis of signaling overhead to establish network connectivity,” IEEE C80216ppc-10_0055r1, Sep. 2010.

- N. Himayat, S. Talwar, K. Johnsson “Smart grid requirements for IEEE 802.16 M2M networks,” IEEE C802.16ppc-10_0042r2, July 2010.

# Appendix: Detailed Simulation Assumptions & Performance Results

## Simulation Parameters

| System Parameters | Setting |
| --- | --- |
| Raging Channel Configuration for 802.16m | Periodicity 1/5ms frame, 1 per 80ms (4 super-frames); Number of codes 32 and 8; (extend to 160) Configuration for 5MHz BW, CP=1/4 |
| Probability of decoding error | 1% |
| Number of Devices | 1K, 10K, 30K, 50K |
| Mean arrival rates | 1/60s, 1/10s, 1/0.5s |
| Arrival distribution | Bernoulli |
| Initial random back-off window (power of 2) | 0 |
| Final random back-off window (power of 2) | 4 |
| Maximum Number of Retries | 16 (maximum allowed) |
| Waiting time between re-transmission | 1 frame |
| Periodicity of ranging channel information | Every 40 ms (SP1 information in S-SFH, assume back-off parameters in SP3 are available) |
| BS processing latency | 1 frame |
| Synchronization latency | Assume synchronized systems |
| Random Access Messaging Assumption | 1 single successful ranging code reception is required for network entry. The latency of RNG-REQ and RSP messages is not modeled. |

## Detailed Performance Results

Only selected results for the case where the system is configured for the maximum number of ranging channels (6400 ranging channel opportunities per second) are included in this section, to illustrate the limits on system performance as the per device arrival rate increases from 1/60s to 1/0.5 seconds. A large number of devices (10,000, 30,000) are also considered.

Figure 1: Conditional contention probability (probability of collision given that a device transmits) as a function of arrival rate per device of [1/60s, 1/10s, 1/0.5s]. The maximum number of access opportunities defined in 802.16m is assumed. Note that even if the contention probability is high, the device may still successfully access the system by employing random back-off and reattempting network access.

Figure 2: Probability of Successful Access as a function of arrival rate per device of [1/60s, 1/10s, 1/0.5s]. The maximum number of access opportunities defined in 802.16m is assumed. Note that access success probability drops sharply to zero as the access rate increases.

Figure 3: Mean delay in milliseconds function of arrival rate per device of [1/60s, 1/10s, 1/0.5s]. The maximum number of access opportunities defined in 802.16m is assumed. Mean delay is computed across all devices regardless of access success. Conditional Mean delays are shown for the devices that are successful at network entry. Note that as access success probability declines for intense arrival rates, conditional mean delays are lower as they only represent the few users that are successful at network entry.

Figure 4: Mean number of retries as a function of arrival rate per device of [1/60s, 1/10s, 1/0.5s]. The maximum number of access opportunities defined in 802.16m is assumed. Mean number of retries is computed across all devices regardless of access success. Conditional mean retries are shown for the devices that are successful at network entry. Note that as access success probability declines for intense arrival rates, conditional retries are lower as they represent the few users that are able to access the network. The unconditional value reaches its maximum allowed limit of 16.

Figure 5: CDF of access delays for 10K devices and 6400 access opportunities/second.  Delays are plotted across all users regardless of success at network access. Conditional delays are plotted for the users that are successful at network entry. An access rate of 1/10s per device is assumed. For lower access rates conditional delays are identical to the overall delay as virtually all users are able to access the network.