| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Informative text on Smart Grid applications for inclusion in IEEE 802.16p Systems Requirements Document (SRD) |  |
| Date Submitted | 2010-11-08 |  |
| Source(s) | N. Himayat, S. Talwar, K. Johnsson, Shantidev Mohanty, Xinrong Wang, Grace Wei, Eve Schooler, George Goodman (Intel) S. Andreev (Tampere University of Technology), O. Galinina (SPIIRAS), A. Turlikov (SUAI) | E-mail: nageen.himayat@intel.com |
| Re: | Informative text describing traffic characteristics and high-level requirements for Smart Grid applications. |  |
| Abstract | This contribution proposes informative text on Smart Grid use case for inclusion as an Appendix in IEEE 802.16p SRD. |  |
| Purpose | For review and adoption into the IEEE 802.16p Systems Requirements Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Background

Smart Grid is a very important machine-to-machine (M2M) use case, which is expected to drive M2M enhancements for wireless standards. First, it is an application that captures a large number of M2M specific features described in the M2M study report, [1], and hence can serve as a useful reference when developing standardized solutions across M2M applications. Second, we expect that the IEEE 802.16 family of standards will be adopted by NIST (National Institute of Standards and Technology) as one of the wireless networking solutions applicable for Smart Grid communication needs, [5]. To that end, we expect that IEEE 802.16p will develop new features to ensure that Smart Grid requirements are met by IEEE 802.16.  While the current 802.16p PAR scope [2], is focused on an initial set of features, which target a subset of Smart Grid applications, future enhancements will target additional applications and track the evolving Smart Grid requirements.

This contribution proposes to include a high-level overview of the Smart Grid application within the IEEE 802.16p Systems Requirements Document (SRD), [3], so that the 802.16p Task Group (TG) can address and track the requirements of this important use case. Specifically this contribution proposes informative text for the 802.16p SRD that covers key traffic characteristics and requirements for Smart Grid applications. The items covered in the proposed text complement the M2M study report [1] to include:

- Typical communication requirements for Smart Grid applications. References [4-10] cover these requirements extensively. A summary is included herein for reference.

- Applicability of IEEE 802.16p across key Smart Grid interfaces as defined in NIST architecture diagram [9], and the associated traffic characteristics per link.

- Estimates of expected number of devices in a smart grid deployment.

- Examples of key issues in supporting Smart Grid traffic, which may be addressed in future developments of the 802.16 standard.

# References

- IEEE 80216ppc-10_0002r7, “Machine-to-Machine (M2M) communication study report,” May, 2010.

- IEEE 80216ppc-10_0003r9, “Machine-to-Machine (M2M) communications PAR form and Five criteria,” July, 2010.

- IEEE 80216ppc-10_0011r2, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD),” Initial working document (revised), Sep. 2010.

- Department of Energy, “Communications requirements for Smart Grid Technologies,” October, 2010.

- NIST framework and roadmap for Smart Grid interoperability standards, v1.0

- NIST Priority Action Plan 2, “Wireless standards for Smart Grid,” sgs-2010-00003r000-Priority_Action_Plan2_r05, July, 2010.

- Open SG, SG Network Systems Requirements Specification v4.0.

- Open SG, SG Functional Requirements, specification, v1.0 draft.

- Open SG, SG NET diagram

- N. Himayat, S. Talwar, K. Johnsson, “Smart grid requirements for IEEE 802.16 M2M networks,” IEEE C802.16ppc-10_0042r2, July 2010.

# Text Proposal

The following text should be included as an Appendix in the 802.16p SRD, [3].

---------------------------Begin Text Proposal--------------------------------------------

# X.1 Overview of Smart Grid Use Case

Smart Grid is an important M2M application providing important benefits to consumers as well as to Utilities. By adding communications capabilities to utility (e.g. electric, power and gas) generation and to distribution and consumption infrastructure, the Utilities can automate grid operation in order to improve efficiency, cost, robustness, and security. A key M2M use case is smart metering that involves meters that autonomously report usage and alarm information to grid infrastructure to help reduce operational cost, as well as regulate customer’s utility usage based on load dependent pricing signals received from the grid. Other key smart grid applications are distribution network automation (DA), demand response (DR), distributed energy resources and storage (DER), wide area monitoring, and control and support for electric vehicles.

Table 1 provides a high level description of Smart Grid applications, the essential requirements for their communication infrastructure, as well as points to the relevance of IEEE 802.16 based communication networks for each application. The content is based on the information provided in [3].

Table 1: Smart Grid applications and associated communication requirements [3].

As can be seen from Table 1, a large number of Smart Grid applications will leverage the AMI network for their communication needs. We expect that wireless technologies, such as IEEE 802.16, will play a key role in enabling AMI. Therefore, IEEE 802.16p should target requirements stemming from Advanced Metering, Demand Response, Distributed Energy Resources, Electric Transportation and Distribution Automation. Wide Area Situational Awareness (WASA) is not included since the real-time, low-latency communication needs of WASA are not expected to be covered by wireless networks. Note that while the applications covering AMI, DR, DER, ET are characterized by low data rates and do not have tight latency and reliability requirements, some Distribution Automation applications can pose challenging latency and reliability requirements, which may not be fully covered by the current scope of the 802.16p project [1]. These challenges may be addressed in the future development of 802.16 families of standards.

In a broader context, there are several ongoing efforts to define the scope and requirements for Smart Grid applications. In addition to the industry players, there is significant interest from governments world-wide, in part due to the potential for security threats to essential infrastructure as well as the current focus on the environment. There is a desire to standardize Smart Grid interfaces to promote interoperability and to accelerate adoption of Smart Grid services. Several international government organizations, utilities, equipment manufacturers as well as research and standards organizations are focused on developing requirements for Smart Grid applications. Some examples include: US Department of Energy (DOE), NIST (National Institute of Standards & Technology), EPRI (Electric Power Research Institute), UCA International Users Group (UCAIUG), Open SG, and other international standards development organizations, such as IEEE P2030, ETSI etc.

In the US, NIST’s Smart Grid Interoperability Panel (SGIP) is driving several industry-wide “Priority Actions Plans,” (PAP) that focus on standardizing key aspects of Smart Grid communication and information infrastructure. NIST’s PAP-2 plan, focusing on wireless standards for Smart Grid, is the most relevant for IEEE 802.16 standards. NIST and the affiliated organizations have published several reference documents defining key use cases, terminology, requirements and applicable standards for the Smart Grid [4-8]. NIST’s current focus is to identify wireless technologies that are well-suited for use in different parts of Smart Grid delivery structure. Standard bodies are expected to report their performance across key metrics identified by the NIST’s requirement matrix [4]. The 802.16p project, consistent with its scope [1], will focus on the first phase of enhancements to the current 802.16 family of standards, to ensure that near-term performance gaps in meeting Smart Grid requirements are addressed. It is also expected that the near term enhancements developed by 802.16p will be more focused on enabling the AMI and the DA. However, the more challenging aspects for DA may be enabled in follow-on projects.

In addition to its value in addressing the needs of an important industry, the Smart Grid application also serves as an important reference M2M use case, as it covers a large number of M2M features described in the M2M study report [2]. The M2M study report covers several Smart Grid use cases under broader categories of Metering, Secured Access and Surveillance, Remote Maintenance and Control, and to a limited extent under Tracking, Tracing & Recovery. This informative text supplements the feature description in [2] to add details on specific areas where 802.16 is applicable for Smart Grid, the traffic characteristics across Smart Grid applications and the key challenges in supporting Smart Grid applications with IEEE 802.16 protocols. The description provided is AMI-centric, as it is viewed as a key enabler for several Smart Grid applications.

# X.2 Applicability of IEEE 802.16p in Smart Grid Reference Model

The Smart Grid conceptual reference diagram published by Open Smart Grid (OpenSG) committee, [8], provides a detailed view of key actors, interfaces, and communication flows in the Smart Grid architecture.  Figure 1 is a simplified view of OpenSG reference diagram to illustrate the network interfaces that may be applicable for 802.16 if the AMI and DA applications are considered.

- Smart meters to AMI Head-end

- Smart meter to DAP (Data Aggregation Point)

- DAP to AMI Head-end

- Smart meter to Home Area Network (HAN). Note this link is applicable from a femtocell point of view.

- Distribution device to Distribution Supervisory Control and Data Acquisition (SCADA) controller.

- Distribution device to Field Area Network (FAN) Gateway

- FAN Gateway to Distribution SCADA

Figure 1: Applicable Smart Grid (AMI and DA) interfaces for IEEE 802.16 (adapted from NIST’s reference diagram [8]).

# X.3 Traffic Characteristics for AMI Applications

NIST Smart Grid Interoperability Panel (SGIP) has initiated a broad effort to determine the communication requirements for Smart Grid applications, under the PAP-2 program [4]. The OpenSG task group has developed over 1400 requirements across 18 different uses cases as input to this program [6].  These requirements cover the information flows across the interfaces shown in Figure 1, for different usage scenarios. An example of aggregate traffic characteristics across 3 applications such as meter reading, service switching and communication to plug in hybrid vehicle (PHEV) are discussed in [4] and are included in Table 2 for illustration.

Table 2: Example statistics of aggregate Smart Grid traffic across meter reading, service switching and PHEV applications at across various links of the Smart Grid architecture.

There is a need to develop aggregate data rates and arrival models for Smart Grid traffic across the network links identified in Figure 1. However, it can be seen from Table 1 and Table 2 that data rates and the average message arrival rate for the application considered are fairly low (less than 2K bytes of data per meter every 1-2 hours).  However, several contributions, [13, 14], indicate that outage or alarm conditions can pose a significant challenge for AMI networks because of the large number of devices involved. The key challenges for AMI communications are expected to stem from the need to handle near simultaneous access by devices that are experiencing outage or alarm conditions (see reference [14]).. Additionally the efficiency of communications from a large number of devices in terms of overhead associated with transmissions also needs to be addressed.

# X.4 Estimating Number of Smart Metering Devices

It is important to estimate the typical number of devices in a smart meter deployment, as a key challenge for AMI networks will be supporting access from a large number of devices. While the number of devices per sector will depend on the particular Smart Grid deployment and the cell sizes involved, example deployment scenario are discussed in Table 3 to get a range of estimates. The estimates assume 3 meters per household and 3-sectored cells.  A uniform population density is assumed with 100% household penetration. NYC statistics assume a population density of 10K users per square Km [17]. Washington D.C statistics are based on [19]. US household statistics are based on [18]. The Urban London example is based on Vodafone’s estimates [20].

Table 3: Estimated Number of Meters/Sector for representative geographies as a function of cell size.

# X.5 Key Issues for 802.16p in Addressing AMI Requirements

The NIST PAP2 requirements matrix for wireless standards defines performance categories for measuring the capabilities of wireless standards to meet Smart Grid requirements. It is anticipated that IEEE 802.16 family of standards will meet a large proportion of requirements set forth for AMI networks under most scenarios.  In the following a few areas requiring additional capabilities that should be addressed by the 802.16p standard are illustrated.  A number of them pertain to network access by a large number of devices. Also important is the efficiency of signaling protocols used for transmitting a small amount of data across a large number of devices.

## X.5.1 Network Entry by Large Number of Devices

There are several usage scenarios that may result in near simultaneous network access from a large number of smart metering devices. Some examples are as follows:

- Alarm reports by a large number of devices when they access the network in an un-regulated or un-synchronized manner.

- A large number of smart meters transition from idle to active state when there is a wide-spread power outage event. Here, the smart meters are required to send a “last gasp” alarm notifying the network that they have lost power. Typically, this report needs to be sent out within a few hundred milliseconds, given that smart meters are expected to operate without battery backup and have limited charge to send the message before complete loss of power (see contribution [13] for more details).  This simultaneous network entry can result from the need to maintain a large number of devices in an idle state. Idle state may be preferred due to a variety of reasons such as a) limited address space for supporting a large number of simultaneously active connections (10-35K devices) b) overhead associated with maintenance signaling for active connections etc.

- A surge in network entry attempts when smart meters try to connect back to the network after a power outage event.

- Meter reporting at regular intervals by large number of devices, when aggressive reporting rates are used (may be applicable in future). Typically, regular communications to and from meters is low data rate, infrequent traffic, so the aggressive rates included here are for illustration to assess any potential system impact. Here also it is assumed that most meters remain attached to the network in the idle state and hence attempt network entry as and when they have data to send.

Table 4 describes the number of access attempts/second for different scenarios assuming different number of smart metering devices per sector.

| Application | Access Interval of Interest | Access Attempt/second (12K devices/sector) | Access Attempt/second (35K devices/sector) |
| --- | --- | --- | --- |
| Meter Reporting | 5 minute | 40 | 116.7 |
| Meter Reporting | 1 minute | 200 | 583.3 |
| Unsynchronized Alarm Reporting or Network Access | 10 second | 1200 | 3500 |
| Last Gasp Event Reporting | 500 millisecond | 24000 | 70000 |

Table 4: Access rates for various smart metering application scenarios, assuming access attempts are uniformly distributed across the interval of interest.

To highlight one of the issues with near simultaneous network entry by large number of devices, Table 5 shows the number of initial ranging opportunities supported by IEEE 802.16m [9] to support initial network entry attempts by a large number of smart metering devices. Calculations are based on cell sizes supporting 8 or 32 codes, Poisson arrival rate with required contention probability of 1%.  Ranging channel periodicities of once per frame (5ms)-once per 4 super-frames (80ms) are assumed (see [9]). Comparing Table 4 with the maximum access rates supported by 802.16m shown in Table 5, it can be seen that significantly higher number of ranging opportunities will be required, if near-simultaneous access by a large number smart meters is to be supported. Alternately, the system design must support techniques to manage a large number of unsynchronized network access attempts.

| Number of initial ranging channels /second | Maximum (Minimum) # of Codes | Number of network entry opportunities per second | Overhead | Number of access attempts supported per second at 1% contention probability. |
| --- | --- | --- | --- | --- |
| 200 | 32 (8) | 6400 (1600) | 2.85% | 960 (240) |
| 50 | 32 (8) | 1600 (400) | 0.71% | 240 (60) |
| 25 | 32 (8) | 800 (200) | 0.35% | 120 (30) |
| 12.5 | 32 (8) | 400 (100) | 0.18% | 60 (15) |

Table 5: Number of initial ranging attempts per second supported by IEEE 802.16m.

A more detailed analysis on success rates of network entry attempts using the 802.16m initial ranging protocols is given in [15]. Results of this analysis indicate that access success rates can be dramatically lowered and access latency is increased substantially when there is a surge in near simultaneous network entry attempts by a large number of devices. Therefore, the system must also ensure that the delivery of higher priority traffic or the performance of non-M2M devices is not adversely impacted by a large number of uncontrolled network access attempts.

## X.5.2 Signaling & Protocol Efficiency of Network Access

It is likely that a large number of smart meters attached to the network will be supported in idle state due to reasons such as limited address space for supporting a large number of active connections, as well as the overhead associated with maintenance signaling for active connections.  These devices will therefore need to connect to the network on a frequent basis to send a very small amount of data. Hence it is important that 802.16p enhancements minimize the signaling and protocol overhead associated with establishing the frequent network connectivity for sending short bursts of data over the network.

The current 802.16m signaling and protocol overhead associated with using short-message service (SMS) to communicate data while maintaining idle state, is investigated in [16]. Table 6 from [16] summarizes the estimated signaling and protocol overhead associated with using SMS transmission while the device maintains idle state operation. Only the signaling overhead of the messages is assumed and overhead incurred during ranging, resource allocation overhead for sending messages as well as any PHY layer overhead is not included. It is clearly seen that overhead for small burst transmissions using SMS needs to be addressed.

| Message | Estimated Number of Bytes |
| --- | --- |
| AAI_RNG-ACK | 14 |
| AAI_RNG-REQ | 58 |
| AAI_RNG-RSP | 17 |
| AAI_DREG-REQ | 8 |
| AAI_DREG-RSP | 20 |
| Signaling overhead assuming 140 bytes of transmitted payload | 117/140= 83.5% |
| Signaling + protocol overhead (62% overhead) | 117/53 = 220% |

Table 6: Approximate signaling overhead calculation for using short message service while the device is in idle state [16].

# X.6 References

- IEEE 80216ppc-10_0003r9, “Machine-to-Machine (M2M) communications PAR form and Five criteria,” July, 2010.

- IEEE 80216ppc-10_0002r7, “Machine-to-Machine (M2M) communication study report,” May, 2010.

- Department of Energy, “Communications requirements for Smart Grid Technologies,” October, 2010.

- NIST Priority Action Plan 2, “Wireless standards for Smart Grid,” sgs-2010-00003r000-Priority_Action_Plan2_r05, July, 2010.

- NIST framework and roadmap for Smart Grid interoperability standards, v1.0.

- Open SG, SG Network Systems Requirements Specification v4.0.

- Open SG, SG Functional Requirements, specification, v1.0 draft.

- Open SG, SG NET diagram

- IEEE P802.16m/D7.

- IEEE 802.16m Systems Requirements Document.

- N. Himayat, S. Talwar, K. Johnsson, “Smart grid requirements for IEEE 802.16 M2M networks,” IEEE C802.16ppc-10_0042r2, July 2010.

- N. Himayat, S. Talwar, K. Johnsson, X. Wang, “Functional Requirements for Network Entry and Random Access by Large Number of Devices,” IEEE C8021.6ppc-10_0049r1, Sep. 2010.

- R. Golla, W. Bell, M. Lin, N. Himayat, S. Talwar, K. Johnsson, “Power Outage Alarm for Smart Metering Applications,” IEEE C80216ppc-10_0054r1, Sep. 2010.

- R. Golla, W. Bell, M. Lin, N. Himayat, S. Talwar, K. Johnsson, “Supporting power outage management for smart metering applications,” IEEE C80216p-10_0005, November, 2010.

- N. Himayat, S. Talwar, K. Johnsson, S. Andreev, O. Galinina, A. Turlikov, “Proposed IEEE 802.16p Performance Requirements for Network Entry by Large Number of Devices,” IEEE C80216p-10_0006, November, 2010.

- N. Himayat, S. Mohanty, K. Johnsson, S. Talwar, “Analysis of signaling overhead to establish network connectivity,” IEEE C80216ppc-10_0055r1, Sep. 2010.

- Demographics of New York City, http://en.wikipedia.org/wiki/Demographics_of_New_York_City

- Average US Household Size, http://www.factfinder.census.gov/servlet/ACSSAFFFacts.

- http://quickfacts.census.gov/qfd/states/11000.html.

- Vodafone, “RACH intensity of time-controlled devices,” R2-102296, 3GPP TSG RAN WG2 #69bis.

----------------------------------------------End Text Proposal---------------------------------------