| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggested Changes to M2M Security Requirements |  |
| Date Submitted | 2010-10-06 |  |
| Source(s) | Kerstin Johnsson, Nageen Himayat, Shilpa Talwar, Jesse Walker, Meiyuan Zhao, Shelagh Callahan Intel Corp. | E-mail: kerstin.johnsson@intel.com |
| Re: | Requirements for M2M Security |  |
| Abstract | Response to call for requirements |  |
| Purpose | For review and adoption into the IEEE 802.16p Systems Requirements Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Background

The current 802.16e and 802.16m security suites are comprised of a series of protocols that maintain device authentication, data confidentiality, data integrity, origin authenticity, etc. In these standards, there is little flexibility in how these protocols are applied.

Unfortunately, security requirements (and QoS requirements in general) vary a great deal depending on the type of M2M application. For example, M2M applications such as health monitoring & control, shipment/cargo security, traffic control, etc. all require the full security suite. In fact, in some of these applications, the current security suite is insufficient as it does not support a robust device validity check. As a result, there is a new security requirement in the 802.16p SRD, which states that the 802.16p standard must support device validity checks coordinated between the device and network. In the other extreme, there are M2M applications where data security and/or integrity are not important particularly when weighed against the channel resource and power consumption that maintaining the full security suite requires. This may be true for some M2M measurement applications.

Since the entire suite of security protocols may not be necessary for all M2M applications, we propose to insert a requirement stating that the 802.16p amendment should enable a security suite that is flexible and can be adapted to the security requirements of the M2M application.

# Text Proposal

Modify the functional requirements described in Section 6.1 as shown.

--------------------------  Begin Text Proposal   -------------------------------

## Security Support

### The 802.16p system shall sSupport for a device validity check between the device and the network shall be provided.

### The 802.16p system shall enable a flexible security suite comprised of protocols that can be included or omitted per the requirements of the M2M application. support less complex security for certain M2M applications with low security requirements. (Editor’s note: for example, no key exchange)

### The 802.16p system shall support efficient security for small burst transmissions.

-----------------------------  End Text Proposal   ----------------------------------