| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed performance requirements for network access with large number of devices |  |
| Date Submitted | 2011-01-03 |  |
| Source(s) | N. Himayat, S. Mohanty, R. Srinivasan, S. Talwar, K. Johnsson, Intel Corp. S. Andreev, (Tampere University of Technology) O. Galinina (SPIIRAS) A. Turlikov (SUAI) | E-mail: nageen.himayat@intel.com |
| Re: | RE: Call for comments on the 802.16p Systems Requirements Document |  |
| Abstract | This contribution proposes performance requirements relevant to network entry by large number of devices. |  |
| Purpose | For review and adoption into the performance requirements section of the 802.16p SRD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Background

Contribution C80216-10_0006r1 provided extensive analysis on the performance of 802.16m random access protocol to support a surge in network access from a large number of devices, which may be triggered by an outage or an alarm event in the network [1]. Based on the analysis presented, [1] also proposed performance requirements for Section 7 of the IEEE 802.16p Systems Requirements Document (SRD), [2].  The proposed performance requirements were based on:

- Providing reliable access (99% access success rate) to support simultaneous access attempts for 30,000 devices over a period of 10 seconds (3000 access attempts/second).

- Maintain a nominal access rate during network overload conditions, which occurs when 30,000 devices attempt simultaneous access to the network during a 500 millisecond period. This surge can occur for, example, in a power outage event (see [1]) and corresponds to 60,000 access attempts/second.

The above requirements were derived based on the assumption of 30K devices/sector, which was proposed by Vodafone as a dense deployment in Urban London [3].  If we consider a more typical US Urban deployment (say New York City [4]) then ~10K devices/sector may be considered to be a more reasonable number. In this contribution we re-calculate the target performance requirement based on the assumption of 10K devices/sector.

Table 1 reproduced from [1] presents the preliminary analysis of 802.16m random access channel. It summarizes the performance of random access when 1K, 10K, or 30K devices attempt near simultaneous network access at an average rate of 1/60s, 1/10s or 1/0.5s, assuming different configurations on random access channels available for contention. It should be noted that 802.16m can support a maximum of 6400 access opportunities/second. It can be seen that > 99% access success rate is possible for 10K devices accessing the network over a 10 second period (entry 13 in the table corresponding to 1000 access attempts/second). However, an increase in random access opportunities may be required for the network to support a surge in network access attempts by 10K devices over a 500 millisecond period (20K access attempt/s) as table entry 18 shows that the access success rate is negligible even when the maximum number of random access channels are configured.

Table 1: Summary of performance of IEEE 802.16m random back-off procedure under different assumptions on the network access rates.

Hence we propose that requirement (b), revised for 10K devices, be included as a target performance requirement for 802.16p systems.

# Text Proposal

Add the following to section 7 of the 802.16p system requirements.

---------------------------Begin Text Proposal-------------------------------

# 7.2 Large Number of Devices

---------------------------Begin Text Proposal----------------------------------

7.2.1 The system shall maintain a minimal level of performance when there is a surge in network access attempt (access rate exceeding 20,000 attempts/second may be considered as “surge” event) by allowing one ore more devices to access the system with a control plane latency of 100 milliseconds.

---------------------------End Text Proposal----------------------------------

# References

- N. Himayat, S. Talwar, K. Johnsson, S. Andreev, O. Galinina (SPIIRAS), A. Turlikov (SUAI), “Proposed IEEE 802.16p Performance Requirements for Network Entry by Large Number of Devices,” IEEE C80216p-10_0006r1,  November, 2010.

- IEEE 80216p-10_0004r1, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD),” November, 2010.

- Vodafone, “RACH intensity of time-controlled devices,” R2-102296, 3GPP TSG RAN WG2 #69bis.

- IEEE 80216p-10_0005, “IEEE 802.16p Machine to Machine (M2M) Technical Report,” November, 2010.