| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Data delivery for large number of devices in SRD |  |
| Date Submitted | 2011-01-10 |  |
| Source(s) | George Cummings, Guanghui Fan, Erik Colban, Bin Chen, Huawei | E-mail: gcummings@huawei.com fanguanghui@huawei.com ecolban@huawei.com binchen@huawei.com |
| Re: | IEEE 802.16p SRD |  |
| Abstract | To propose a definition modification on 16p SRD |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Data delivery for large number of devices in SRD

George Cummings, Erik Colban,Guanghui Fan, Bin Chen

Huawei

# Introduction

In M2M, some services need the devices exchange data with M2M server within a scheduled time window. When there are several services processing in a M2M system, it shall be considered to avoid network congestion and overload for their data exchanging by scheduler, so that can also reduce delay. For example, the services scheduler can schedule the report from M2M devices with different services in same area (or cell) on different time windows to avoid network congestion by the data delivery from huge number of devices simultaneously.

For this we would propose adding a requirement to 16p SRD, as below.

# Proposed Text

------------------------------------------------Start of the proposed Text----------------------------------------------------

6.2.6	The 802.16p system shall support efficient and reliable control and data delivery to and from large number of devices

6.2.6.1 The 802.16p system shall support efficient and reliable control and data delivery to and from groups of devices

6.2.6.2 The 802.16p system shall provide mechanism to reduce or avoid network congestion or overload during data delivery to and from large number of devices.

------------------------------------------------End of the proposed Text----------------------------------------------------

References

[1]	IEEE 802.16p-10/0004r1, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD)  (initial working document revised)”, 2010-11-11