| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16p Machine to Machine (M2M): Proposed Text from Power Saving (PWR) Rapporteur Group |  |
| Date Submitted | 2011-03-16 |  |
| Source(s) | Kiseon Ryu LG Electronics | Email: kiseon.ryu@lge.com |
| Re: | IEEE 802.16p-10/0016 - M2M Rapporteur Group Plan, Schedule, and Guidelines |  |
| Abstract | The proposed text is the output from the discussions in the PWR RG. |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16p Machine to Machine (M2M): Proposed Text from

Power Saving (PWR) Rapporteur Group

Kiseon Ryu

LG Electronics

# Introduction

This contribution is the output of the deliberations in the Power Saving (PWR) rapporteur group (RG).  This contribution contains proposed text from all contributions marked with the tagline PWR in their titles and the contributions related to idle mode operation (i.e. the paging operation in idle mode and network re-entry from idle mode), which were identified in DEV RG. The contributions were discussed in 3 conference calls and email discussion on the reflector with all members of the 802.16p task group. The conference calls were held on Feb 11, 2011, Feb 17, 2011 and Feb 24, 2011.

Table 1: List of contributions submitted to the PWR RG

|  | Contribution Number | Title | Contributors |
| --- | --- | --- | --- |
| 1 | C80216p-rg-11_0007.doc | Recommended idle mode operation for a device with machine to machine application | Hyunjeong Kang, Rakesh Taori, Chiwoo Lim, Kaushik Josiam, Ying Li, Sudhir Ramakrishna, Lei Zhou, Hai Wang¦ Samsung Electronics |
| 2 | C80216p-rg-11_0014.doc | [PWR] Amendment Text for 802.16p location update procedure | Giwon Park, Youngsoo Yuk, Jin Lee, Jeongki Kim, Kiseon Ryu and Jinsam Kwak¦ LG electronics |
| 3 | C80216p-rg-11_0015.doc | [PWR] Amendment Text for 802.16p paging | Giwon Park, Youngsoo Yuk, Jin Lee, Jeongki Kim, Kiseon Ryu and Jinsam Kwak¦ LG Electronics |
| 4 | C80216p-rg-11_0019.doc | [PWR] Baseline Amendment Text for 802.16p Device Collaboration | Kyujin Park, Jinsoo Choi, HanGyu Cho, Kiseon Ryu, Jin Sam Kwak¦ LG Electronics |

Table 2: List of contributions submitted to the DEV RG but discussed in the PWR RG

|  | Contribution Number | Title | Contributors |
| --- | --- | --- | --- |
| 1 | C80216p-rg-11_0002.doc | Proposed text of group paging message for IEEE 802.16p system (DEV for large number of devices) | Namgi Kim, Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon |
| 2 | C802.16p-rg-11_0005.doc | Text Proposal for Large Number of Devices | Yu-Chuan Fang, Zheng Yan-Xiu ITRI |
| 3 | C80216p-rg-11_0008.doc | [DEV] Proposed Text for Network Reentry for a Large Number of M2M Devices¦ | Hai Wang, Lei Zhou, Xufeng Zheng, Hyunjeong Kang, Chiwoo Lim, Kaushik Josiam, Jaehyuk Jang, Jaeweon Cho, Rakesh Taori¦ Samsung Electronics |
| 4 | C80216p-rg-11_0009.ppt | [DEV] Priority-Based Backoff Instructions for M2M Network Re-Entry¦ | Ping-Heng Kuo, Wei-Chieh Huang, Chia-Lung Tsai, Yu-Tao Hsieh, Pang-An Ting¦ ITRI |
| 5 | C80216p-rg-11_0010.ppt | [DEV] Urgent Paging for M2M Network¦ | Ping-Heng Kuo, Wei-Chieh Huang, Chia-Lung Tsai, Yu-Tao Hsieh, Pang-An Ting¦ ITRI |
| 6 | C80216p-rg-11_0017.ppt | [DEV] Network access with priority | Wei-Chieh Huang, Chia-Lung Tsai, Yu-Tao Hsieh, Chiu-Ling Chen, and Pang-An Ting¦ ITRI |
| 7 | C80216p-rg-11_0020.ppt | [NEV] Network reentry for fixed devices¦ | Wei-Chieh Huang, Chia-Lung Tsai, Yu-Tao Hsieh, and Pang-An Ting¦ ITRI |
| 8 | C80216p-rg-11_0026.doc | [DEV] Access distribution for M2M devices¦ | Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu , JinSam Kwak¦ LG Electronics |
| 9 | C80216p-rg-11_0027.doc | [DEV] Access restriction for a large number of M2M devices¦ | Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu , JinSam Kwak¦ LG Electronics |
| 10 | C80216p-rg-11_0028.ppt | [DEV] Random Access Resource Allocation for Large Number of Devices | Andreas Maeder, Peter Rost, Linghang Fan, Hassan Alkanani, Nader Zein¦ NEC |

The following procedure was used in consolidating the proposed texts from all contributions.

- We build based on the Section numbers – Lower numbered Sections go first.  Existing Section Numbers go first, followed by new Section numbers

- Existing Section numbers are in black color with a bold face.

- Existing text are colored black

- All proposed sections that have consensus are colored blue with an underline and bold face.

- All harmonized proposed text that have consensus are colored blue with an underline.

- All proposed texts without consensus are [bracketed and have grey highlights].

---- Start of proposed text ----

6.3.23.10.1 Network reentry from idle mode for M2M devices

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in 6.3.10.3

16.2.18 Idle Mode

16.2.18.7 Idle mode for M2M application

When an M2M device enters idle mode, the M2M device shall follow the procedures defined in 16.2.18 except the procedures described in this section.

If an M2M device requires the longer inactive interval, longer paging cycle can be provided for the M2M device during the idle mode initiation through AAI-DREG-REQ/AAI-DREG-RSP messages.

Idle mode timer for the M2M device may be assigned during idle mode initiation. In this case, the AAI-DREG-RSP message includes M2M device-specific Idle Mode Timer. When the M2M device receives the AAI-DREG-RSP message with M2M device-specific Idle Mode Timer, the M2M device shall perform location update prior to the expiration of the M2M device-specific Idle Mode Timer. At every location update including the paging group location update, the M2M device-specific Idle Mode Timer is reset to 0 and restarted.

16.2.18.7.1 Paging operation

Group paging may be used for M2M devices. For this, M2M Group Identifier (MGID) defined in 16.2.1.3.1 is included in a paging message instead of an individual identifier to identify the group of M2M devices.

16.2.18.7.2 Network re-entry from idle mode for M2M devices

For network reentry from Idle Mode, ranging parameters may be different for M2M devices or M2M groups.

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in Table 833 in 16.3.5.5.1.2.