| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for network access entry for a large number of M2M devices |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Lei Zhou, Hai Wang, Xufeng Zheng China Samsung Telecom R&D Center Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Call for contributions for PWR RG or 802.16p WG |  |
| Abstract | This contribution proposes to network access entry for a large number of M2M devices |  |
| Purpose | To be discussed and adopted by PWR RG or 802.16p for 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for network access entry for a large number of M2M devices

Lei Zhou, Hai Wang, Xufeng Zheng

China Samsung Telecom R&D Center

Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori

Samsung Electronics Co., Ltd.

- Introduction

Based on IEEE 802.16p SRD [1] and our RG contribution [3], this contribution further proposes to clarify the procedure of network reentry from idle more for M2M group. This contribution supports two trigger modes for M2M group including network trigger and M2M group trigger. When a M2M group is expected to report their data or is paged, group delegates of the group send a ranging code based on multicast group ID (MGID) from the above ranging code set to BS. After BS decoded this ranging code, BS sends AAI-RNG-ACK to respond to this ranging code, which includes three ranging status (success, abort and continue) for group.

- References

[1] IEEE 802.16p-10/0004r2: IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD)

[2] IEEE 802.16p-10/0005r1: Machine to Machine (M2M) Communications Technical Report

[3] IEEE 802.16p-rg-11/0008: Proposed Text for Network Reentry for a Large Number of M2M Devices

- The Proposed Text in AWD

------------------------------- Text Proposal Start ---------------------------------------------------

------------------------------- Text Proposal 1 Start ---------------------------------------------------

[Remedy1: Add proposed text from line# 53 Page 16 in P80216p-10_0018 with the followings:]

16.2.18.X Idle mode for M2M devices

16.2.18.X.X Network reentry from idle mode for M2M devices

In order to reduce network congestion produced by a large number of M2M devices, network reentry based on group from idle mode procedure for M2M devices should be used. M2M device in a M2M group is called as group member (GM). M2M device which is authorized to act as representative for one M2M group to initiate the first ranging access of the M2M group is called as group delegate (GD). BS should assign a ranging code set and ranging opportunities to M2M groups. When a M2M group is expected to report their data or is paged, group delegates of the group send a ranging code based on multicast group ID (MGID) from the above ranging code set to BS.

After BS decoded this ranging code, BS sends AAI-RNG-ACK to respond to this ranging code, which includes three ranging status (success, abort and continue) for group.

All GMs of this group listen to AAI-RNG-ACK in the ranging procedure based on group. After received AAI-RNG-ACK including that ranging code, GM acts according to ranging status.

  - If ranging status is “success”, all GMs in this group start its network reentry procedure by transmitting a ranging code. The ranging code chosen may be in legacy code sets for ranging or may be a new one and the legacy initial ranging opportunity or a new one may be used for transmitting the ranging code. When A GM receives ”success” of ranging status for ranging code for the group in AAI-RNG-ACK, the GM shall randomly select a backoff value within the backoff window for group specified by S-SFH SP3. This random backoff value within the backoff window for group indicates the number of initial ranging opportunities that the GM shall defer before transmitting a ranging code.

  - If ranging status is “abort”, all GM in this group shall start the ranging abort timer and abort the ranging process until the ranging abort timer expires. Similar as the 802.16m system, after abort timer is expired, this group should restart the ranging procedure based on group.

  - If ranging status is “continue”, the GD in this group shall adjust its parameters accordingly and continue the ranging process as done on the first entry.

------------------------------- Text Proposal 1 End ----------------------------------------------------

------------------------------- Text Proposal 2 Start ---------------------------------------------------

[Remedy2: Replace the table 680 from line 16 on Page 16 in P80216p-10_0018 with the followings:]

Table 680—AAI-RNG-ACK message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| }//end of for |  |  |  |
| For (j=0; j<N_Received_Codes for M2M group; j++) { |  | The number of ranging preamble code indices(i.e. N_Received_Codes)[1..64] detected in this corresponding ranging opportunity. |  |
| Ranging preamble code index for M2M group | 5 | Ranging preamble code index for M2M group received in this ranging opportunity. |  |
| Ranging Status | 2 | Indicate whether ranging preamble code or UL burst is received within acceptable limits by BS. 0b00 = success for M2M Group 0b01 = abort for M2M Group 0b10 = continue for M2M group |  |
| If (Ranging Status == 0b01) { |  |  |  |
| Ranging Abort Timer | 16 | Timer defined by an BS to prohibit the M2M group from attempting network entry at this BS, for a specific time duration. Value: 0 (Do not try ranging again at the BS.) Value: 1-65535, In units of seconds |  |
| }else if (Ranging Status ==0b10) { |  |  |  |
| Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication |  |
| If( API Bit#0==0b1 ) { |  |  |  |
| Timing offset adjustment | 15 | Amount of time required to adjust AMS transmission (in units of 1/Fs). MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 14 bits represent timing offset correction value of [1..16384] that corresponds to 0x0000 ~ 0x3FFF,respectively The GD shall advance its transmission time if the value is negative (i.e. MSB = 0b1) and delay its transmission time if the value is positive (i.e. MSB = 0b0). |  |
| } |  |  |  |
| If( API Bit#1==0b1 ) { |  |  |  |
| Power level adjustment | 4 | Power level adjustment which expresses the change in power level (in multiples of 1 dB) that the GD shall apply to its current transmission power for initial ranging. MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 3 bits represent power level correction value of [1..8] that corresponds to 0b000 ~ 0b111, respectively |  |
| } |  |  |  |
| If (API Bit#2==0b1) { |  |  |  |
| Frequency offset adjustment | 9 | Frequency offset adjustment. Relative change in transmission frequency.The correction is 2% of the subcarrier spacing (•ƒf) multiplied by the 9-bit number interpreted as a signed integer. MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 8 bits represent frequency offset correction value of [1..256] that corresponds to 0x00 ~ 0xFF, respectively. |  |
| } |  |  |  |
| }//end of (Ranging Status == 0b10) |  |  |  |
| }//end of for |  |  |  |
| }//end of for each |  |  |  |
| }//end of for |  |  |  |
| ... | ... | ... | ... |

------------------------------- Text Proposal 2 End ---------------------------------------------------

------------------------------- Text Proposal 3 Start ---------------------------------------------------

[Remedy3: Replace the table 835 from line 2 on Page 18 in P80216p-10_0018 with the followings:]

Table 835—S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ... | ... | ... |
| Reserved |  |  |
| Initial ranging backoff start for M2M Group | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 |
| Initial ranging backoff end For M2M Group | 4 | Final backoff window size for initial ranging contention, expressed as a power of 2. Valuesof n range 0-15 |
| Reserved |  |  |
| } |  |  |

------------------------------- Text Proposal 3 End ---------------------------------------------------

------------------------------- Text Proposal End ---------------------------------------------------