| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimizing Location update for stationary M2M devices |  |
| Date Submitted | 2011-3-5 |  |
| Source(s) | Ritesh Kumar Kalle, Debabrata Das International Institute of Information Technology-Bangalore Shantidev Mohanty Intel Corp. | E-mail: riteshkumar.k@iiitb.ac.in Shantidev.mohanty@intel.com |
| Re: | 802.16p amendment texts |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimizing Location update for stationary M2M devices

# Background

IEEE 802.16p Machine to Machine (M2M): Proposed Text from Power Saving (PWR) Rapporteur Group has submitted the consolidated text of output from the RG discussions in contribution number IEEE C802.16p-11/0002 , IEEE 802.16p-rg-11/0032

It has contributed the following section on Location Update Mechanism for M2M devices:

“

16.2.18.4 Location update

16.2.18.4.3 M2M location update

An M2M device in idle mode shall perform the location update based on its mobility capability (i.e., Mobility, No mobility) and traffic pattern which were negotiated in capability negotiation phase of the network entry. For example, the time controlled M2M device may not perform the timer based update. No mobility M2M device may not perform the paging group based update.

16.2.18.4.3.1 Location update for time controlled M2M device

Time controlled M2M device may not perform the timer based update because the time controlled M2M device periodically transmits the uplink traffic to the base station at the pre-scheduled time. Time controlled M2M device can assign the idle mode timer via explicit signaling (i.e., AAI-REG-REQ/RSP) based on its traffic pattern (e.g., Time controlled or Non time controlled) to prevent the timer based update.

16.2.18.4.3.2 Cell based update support

Cell based update is intended as a mechanism to allow the M2M device for tracking service to report its own location information, typically when the M2M device detects that the preferred BS is changed.

For cell based update, M2M device shall negotiate the capability for cell based update during the capability negotiation phase of the network entry. When an M2M device that has the capability of the cell based update detects that the selected preferred cell does not support its currently attached cell, it shall perform the cell based update. When an M2M device that has the capability of cell based update enters the idle mode, paging group is not assigned to the M2M device.

“

“[16.2.18.7.2 Location update

16.2.18.7.2.1 Cell based update support

Cell based update is intended as a mechanism to allow the M2M device for tracking service to report its own location information, typically when the M2M device detects that the preferred BS is changed. For cell based update, M2M device shall negotiate the capability for cell based update during the capability negotiation phase of the network entry. When an M2M device that has the capability of the cell based update detects that the selected preferred cell does not support its currently attached cell, it shall perform the cell based update. When an M2M device that has the capability of cell based update enters the idle mode, paging group is not assigned to the M2M device.]”

Currently the RNG-REQ message for 16m devices includes the following mobility information for devices

AMS Mobility Information

0b00 = Slow (0-10km/h)

0b01 = Medium (10-120km/h)

0b10 = Fast (above 120km/h)

0b11 = Reserved

The M2M devices used for smart metering would come under the Slow mobility category above currently.

There are four location update trigger evaluation conditions:

- paging group based update

- timer based update,

- power down update and

- MBS update.

Plus AMS may conduct a Location Update at Will whenever required.

Since the assigned paging group doesn’t change, Trigger Condition 1 is not applicable

There are two possible schemes for Timer based update

Since the device is stationary there is no effect of PG_LU_TIMER on this device for location update as it doesn’t switch the PAGING_GROUP.

The Idle mode timer has a minimum value of 128 sec and a default value of 4096 sec.

The MS has to perform a location update before the expiry of the Idle mode timer and so we can assume the frequency of location updates for stationary M2M devices to be of the order of 60 min.

SMART GRID TRAFFIC MODEL (Example statistics of aggregate Smart Grid traffic IEEE 802.16p-10/0005)

For above traffic patterns in the smart meter example we see that the M2M device is not expected to wakeup from idle states for several 1000s of seconds during which the M2M device is forced to perform a location update before the Idle mode timer expiry as in the current draft.

So we propose the following changes:

Add1 bit of information more to AMS Mobility Information in AAI-RNG-REQ message Field Description

0b00 = Slow (0-10km/h)

0b01 = Medium (10-120km/h)

0b10 = Fast (above 120km/h)

0b11 = Fixed M2M Device

If (AMS Mobility ==0b11)

AMS may disable Location Update trigger based on Idle Mode Timer Expiry

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Table 678  AAI-RNG-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| AMS Mobility Information | 2 | 0b00 = Slow (0-10km/h) 0b01 = Medium (10-120km/h) 0b10 = Fast (above 120km/h) 0b11 = Fixed M2M Device | Optional |

16.2.18.4.3.3 Location update for Fixed M2M device

The Fixed M2M device may disable the Location Update trigger based on Idle Mode Timer Expiry.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------