| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Distribution of Initial Ranging Access for Network Reentry |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Seokki Kim, Chulsik Yoon ETRI | Email: jscha@etri.re.kr |
| Re: | Call for Comment on IEEE 802.16p AWD |  |
| Abstract | This contribution propose texts to distribute initial ranging access of a large number of M2M devices for network reentry. |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Distribution of Initial Ranging Access for Network Reentry

Jaesun Cha, Soojung Jung, Seokki Kim, Chulsik Yoon

ETRI

# Introduction

This contribution includes some comments on PWR RG output (C802.16p-11/0002), especially on access control during network reentry.

Some contributions propose a prioritized access concept during network reentry to resolve UL access congestion. But, it’s not easy for an ABS to assign a priority to an M2M group or an M2M device because of following reasons.

- An ABS can’t know when M2M devices will perform network reentry to transmit UL data.

- A priority level can’t be decided based on the type of M2M application. Generally, Smart Grid and healthcare are classified into a time-controlled M2M service and they are not time-critical service. But, some events such as power outage alert and sudden heart attack are time-critical. Then, do we have to give those applications a higher priority? Otherwise, how can an ABS give those events a higher priority?

We think that distribution of potential ranging accesses is a more practical scheme to avoid UL access congestion.

We differentiated any texts modified by this contribution from those modified by PWR RG contribution by using a different color as follows.

- Texts added by DEV RG contribution: blue underlined texts

- Texts deleted by DEV RG contribution: red strikeout texts

- Texts added by this contribution: green underlined texts

- Texts deleted by this contribution: green strikeout texts

# Proposed Texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

3. Definitions

[3.x Source M2M device: The end device whom the data traffic to the BS is generated from or the data traffic generated from the BS is targeting for

3.x Cooperative M2M device: The device who helps the source M2M device(s) in UL by relaying or co-transmitting data and control information]

[6.3.23.10 MS idle mode for M2M application

When an MS which supports M2M application enters idle mode, the MS follows the procedure defined in 6.3.23.

If an MS has subscribed to an M2M application requiring longer inactive interval, the operation in 6.3.23.10 should be applied for the MS.

During idle mode initiation transaction through DREG-REQ and DREG-CMD messages, the paging information in 11.1.8.3.1 is applied for the MS.

If Location Update Response is set to 0x00 (Success of Idle Mode Location Update) in RNG-RSP message and Paging information has changed, the Paging information in 11.1.8.3.1 shall be included in the RNG-RSP message.

A paging controller may assign idle mode timer for the MS only. In this case, the DREG-CMD message includes MS-specific Idle mode timer as in 11.14.1. The paging controller shall maintain idle mode system timer corresponding to MS-specific Idle mode timer to retain the MS’s service and operational information. When the MS receives the DREG-CMD message with MS-specific Idle mode timer, the MS shall periodically perform location update prior to the expiration of the MS-specific Idle mode timer. At every location update including the paging group location update, the MS-specific Idle mode timer is reset to 0 and restarted.]

11.1.8.3 Paging information

[11.1.8.3.1 Paging information for M2M application

In case of RNG-RSP message, Paging information shall be included if Location Update Response is set to 0x00 (Success of Idle Mode Location Update) and Paging information has changed.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 134.1 | 9 | Bits 0-23: PAGING_CYCLE – Cycle in which the paging message is transmitted within the paging group. Bits 24-47: PGING_OFFSET – Determines the frame within the cycle from which the paging interval starts. Shall be smaller than PAGING_CYCLE value. Bits 47-63: Paging-group-ID – ID of the paging group to which the MS with M2M application is assigned. Bits 63-71: Paging Interval Length – Max duration in frames of Paging Listening interval. Used in calculation of Paging listening interval; value shall be between 1 and 5 frames (default=2). | RNG-RSP DREG-CMD |

]

11.14 DREG-CMD/REQ message encodings

[11.14.1 MS-specific Idle mode timer

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| MS-specific Idle mode timer | 53 | 3 | MS timed interval in frame to conduct location update. Timer recycles on successful idle mode location update. | DREG-CMD |

]

16.2.3.22 AAI-DREG-RSP [

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Paging cycle | 4 | Used to indicate Paging cycle for the AMS 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08: 30000 superframes 0x09: 180000 superframes 0x10: 4194304 superframes 0x1108-0x15: reserved |  |

]

16.2.3.31 AAI-System Configuration Descriptor (SCD) Message [

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| MSB of superframe number for AMS with M2M application | 10 | 10 MSB of the remaining bit of SFN except LSB of SFN in P-SFH |  |

]

16.3.5.5.1.2 S-SFH IE [

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH For AMS with M2M application, this field is interpreted as 8 LSB of the remaining bit of SFN except LSB of SFN in P-SFH. |

]

16.2.3.22 AAI-DREG-RSP [

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| MS-specific Idle Mode Timer | 24 | AMS timed interval in frame to conduct location update. Timer recycles on successful idle mode location update. | May present when an AMS with M2M application enters idle mode |

]

16.2.3.22 AAI-DREG-RSP message

Table 699. AAI-DREG-RSP message Format [

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If (Action code == 0x07) { |  |  |  |
| … |  |  |  |
| Paging offset | 12 | Used to indicate Paging offset for the AMS. Determines the superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than Paging cycle value. or Used to indicate paging offset of M2M Group Paging Message for M2M device. Determines the superframe within paging cycle from which the paging listening interval start. Shall be smaller than Paging cycle value. |  |

]

[

16.2.3.64 M2M Group Paging (AAI-M2M-GRP) Message

Table xxx. M2M Group Paging (AAI-M2M-GRP) Message Format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| M2M Group paging message format { |  |  |  |
| Num_M2M Group | 4 |  |  |
| for (i=0; i< Num_M2M Group; i++) { |  |  |  |
| M2M Group Paging ID (MGPID) | TBD | Used to indicate M2M Group Paging ID for the M2M Group to be paged. |  |
| m | 2 | Time domain hash parameter (1 ~ 4) used to determine the frame index of a superframe for M2M Individual Paging Message transmission of an idle mode M2M device. | Shall be present when the Bit #0, 2 in Action Bitmap are set to 1 and Action type is set to 1. |
| Action Bitmap | 4 | Bit #0: Perform the network reentry Bit #1: Receives the multicast traffic Bit #2: Perform ranging for location update Bit #3: Updates control parameters |  |
| if (Bit #0 in Action Bitmap == 1) { |  |  |  |
| Action Type | 1 | 0: It indicates the group network reentry. All M2M devices belong to M2M Group shall perform the network reentry. 1: It indicates the individual paging message. M2M device monitors the M2M Individual Paging Message based on M2M Device ID mod m. |  |
| } |  |  |  |
| if (Bit #2 in Action Bitmap == 1) { |  |  |  |
| Action Type | 1 | 0: It indicates the group location update. All M2M devices belong to M2M Group shall perform the location update. 1: It indicates the individual paging message. M2M device monitors the M2M Individual Paging Message based on M2M Device ID mod m. |  |
| } |  |  |  |
| if (Bit #3 in Action Bitmap == 1) { |  |  |  |
| Paging Unavailable Interval | TBD | Paging Unavailable Interval to the M2M device | Presented if it needs to be updated. |
| Paging Cycle | TBD | Paging cycle applied to the M2M device | Presented if it needs to be updated. |
| M2M Group Paging ID (MGPID) | TBD | Used to indicate M2M Group Paging ID for the M2M Group to be paged. | Presented if it needs to be updated. |
| } |  |  |  |
| } //End of for (i=0; i< Num_M2M Group; i++) |  |  |  |
| }//End of M2M Group Paging Message format |  |  |  |

16.2.3.65 M2M Individual Paging (AAI-M2M-INP) Message

Table xxx. M2M Individual Paging (AAI-M2M-INP) Message Format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| M2M Individual Paging Message format { |  |  |  |
| M2M Group Paging ID (MGPID) | TBD | Used to indicate M2M Group Paging ID for the M2M Group to be paged. |  |
| Num_M2M devices |  |  |  |
| for (i=0; i < Num_M2M devices; i++) { |  |  |  |
| M2M Device ID (MDID) | TBD | Used to indicateM2M Device ID for the M2M device to be paged |  |
| Action code | 2 | 0b00: Perform the network reentry 0b01: Performs ranging for location update 0b10~0b11: Reserved |  |
| } |  |  |  |
| } |  |  |  |

]

[16.2.3.64 M2M Group Paging (AAI-M2M-GRP) Message

Table xxx. M2M Group Paging (AAI-M2M-GRP) Message Format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| M2M Group paging message format { |  |  |  |
| Num_M2M Paging Group | 4 |  |  |
| For (i=0; i< Num_M2M Paging Group; i++) { |  |  |  |
| M2M Group Paging ID (MGPID) | TBD | Used to indicate M2M Group Paging ID for the M2M Paging Group to be paged. |  |
| Group waiting time | TBD | The period of time that an M2M paging group of M2M devices should wait before selecting ranging opportunities | Present if reentry is needed. |
| K | TBD | The length of ranging opportunity window. This should be applied after group waiting time expires | Present if reentry is needed. |
| } |  |  |  |
| } |  |  |  |

]

[16.2.3.64 M2M Group Paging (AAI-M2M-GRP) Message

Table xxx. M2M Group Paging (AAI-M2M-GRP) Message Format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| M2M Group paging message format { |  |  |  |
| Num_M2M Paging Group | 4 |  |  |
| For (i=0; i< Num_M2M Paging Group; i++) { |  |  |  |
| M2M Group Paging ID (MGPID) | TBD | Used to indicate M2M Group Paging ID for the M2M Paging Group to be paged. |  |
| UL forbidden time | TBD | Time duration to prohibit M2M devices from ranging request for re-entry or location update within paging unavailable interval(s). |  |
| Start offset | TBD | This field is start offset of UL forbidden time from the end of paging listening interval(s). |  |
| } |  |  |  |
| } |  |  |  |

]

16.2.3.31 AAI-System Configuration Descriptor (SCD) Message[

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<Num_Ranging_Time_OffsetAccess_Prio_index; i++) { |  |  |  |
| Ranging Time OffsetAccess Priority Index | TBD | Ranging Time OffsetAccess Priority Index |  |
| Ranging Time Offsetparameters | TBD | TBDIndicates when an M2M device shall start random backoff. A time offset value is calculated by multiplying Ranging Time offset by Initial Backoff Window. |  |
| } |  |  |  |

16.2.3.22 AAI-DREG-RSP

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Time offsetAccess Priority Index | TBD | Index which indicates ranging time offsetparameters defined in SCD message. An AMS with M2M application shall start random backoff after time offset indicated by Ranging Time Offset Indexuse ranging parameters during network reentry from idle mode. indicated by this index. | Presents when an AMS with M2M application enters idle mode |

16.2.3.23 AAI-PAG-ADV

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (j=0; j<Num_Group_ID; j++0 { |  |  |  |
| Group ID | TBD | M2M Group ID |  |
| Ranging Time OffsetAccess Priority Index | TBD | Index which indicates ranging time offsetparameters defined in SCD message. An AMS with M2M application shall start random backoff after time offset indicated by Ranging Time Offset Indexuse ranging parameters during network reentry from idle mode. indicated by this index. | May present when an AMS with M2M application enters idle mode |
| } |  |  |  |

]

16.2.11.1.1 Contention-based random access bandwidth request

[M2M devices may be restricted to use dedicated BR opportunities assigned to an M2M group.]

16.2.15 Network entry and initialization

[16.2.15.X Traffic regulation for the M2M devices

The BS may manage these M2M devices by sending MAC message to these M2M devices for the purpose of traffic regulation. The M2M device should be capable of decoding these regulation messages. The BS may transmit regulation message to command or configure a group of devices temporarily or specifically. The regulation message shall be capable of stopping network entries and ranging for these M2M devices. The regulation message shall be capable of reconfiguring the ranging parameters for M2M devices. The regulation message shall be capable of terminating the configured message different to system configuration. ]

[16.2.15.X Network access with priority

The devices shall support to adjust the opportunity periodicity according to the opportunity configuration and priority factor. ]

16.2.16 Periodic ranging

[16.2.16.Y Ranging allocation for large number of machine to machine devices

The BS may interleave M2M communications with existing AMSs or MSs. Additional ranging opportunities may be allocated for specific M2M devices. When the ranging opportunities are allocated for a group of M2M devices, M2M devices shall not send ranging signals on the opportunities which has been allocated to AMSs or MSs.]

[16.2.16.x Ranging allocation for large numbers of machine to machine devices

Dedicated ranging opportunities may be allocated for specific M2M groups. If dedicated ranging opportunities are allocated to a group of M2M devices, said M2M devices shall not send ranging signals on the opportunities which has been allocated to AMSs or MSs. ]

16.2.18 Idle Mode

16.2.18.7 Idle mode for M2M application

When an M2M device enters idle mode, the M2M device shall follow the procedures defined in 16.2.18 except the procedures described in this section.

If an M2M device requires the longer inactive interval, longer paging cycle can be provided for the M2M device during the idle mode initiation through AAI-DREG-REQ/AAI-DREG-RSP messages.

A paging controller may assign idle mode timer for the M2M device. In this case, the AAI-DREG-RSP message includes M2M device-specific Idle Mode Timer. The paging controller shall maintain idle mode system timer corresponding to M2M device-specific Idle Mode Timer to retain the M2M device’s service and operational context information. When the M2M device receives the AAI-DREG-RSP message with M2M device-specific Idle Mode Timer, the M2M device shall perform location update prior to the expiration of the M2M device-specific Idle Mode Timer. At every location update including the paging group location update, the M2M device-specific Idle Mode Timer is reset to 0 and restarted.

16.2.18.7.1 Paging operation

Group paging is a mechanism for reducing a paging overhead and may be used for the purpose of multicast traffic indication, requesting a group of users to perform network reentry [, updating the group control parameter (e.g., paging cycle, paging offset, etc)]. In this case, M2M Group Identifier (MGID) defined in 16.2.1.x is included in a paging message instead of an individual identifier to identify the group of M2M devices.

[When the M2M device receives the group paging message during the paging available interval, it can perform the following procedures:

- Perform the network reentry: when the M2M device receives the group paging message which includes the identification of its M2M group to be notified of requesting the network reentry, it shall perform the network reentry. If the M2M device receives the group paging message which does not request the network reentry, it shall not perform the network reentry.

- Receive the multicast traffic: when the M2M device receives the group paging message which includes the identification of its M2M group to be notified of pending DL multicast traffic, it shall receive the multicast traffic.

- Update control parameters: when the M2M device receives the group paging message which includes the identification of its M2M group to be notified of updating the control parameters, it shall update indicated control parameters in the group paging message.

An M2M Group Identifier (MGID) and an M2M Device Identifier (MDID) are used to indicate an M2M group and each M2M device within the M2M group. An M2M Group Identifier (MGID) and an M2M Device Identifier (MDID) are unique within the paging group of M2M device. An M2M Group Identifier (MGID) and an M2M Device Identifier (MDID) shall be assigned to an M2M device during the network entry. The paging group ID is assigned to an M2M device during the idle mode initiation through AAI-DREG-RSP message. The ABS shall advertise the paging group ID (PGID) in the PGID-Info message.

In order to monitor the group paging message, the M2M device derives the start of the paging listening interval based on the paging cycle and paging offset. The paging listening interval shall comprise of the superframe whose superframe number meets the condition.

PAGING_OFFSET ==N superframe modulo PAGING_CYCLE

The length of the paging listening interval is one superframe per paging cycle. The M2M devices that have same MGID have the same paging cycle.

An M2M device monitors the group paging message at the first AAI subframe of the first frame of the superframe of its Paging Available Interval. If an M2M device does not detect the group paging message at the mentioned AAI subframe, it assumes that there is no paging message (i.e., group paging message or individual paging message) for itself and enters the Paging Unavailable Interval. When an M2M device receives the group paging message and checks that individual paging message will be transmitted, it additionally monitors the individual paging message over the pre-determined frame whose frame offset is implicitly determined as following equation:

N paging frame offset = M2M Device ID mod m,

where m=1 or 2 or 3 or 4, and is transmitted in the group paging message.]

[In order to control access congestion caused by large numbers of M2M devices, ranging from the M2M devices may be restricted during paging unavailable interval(s).

If an M2M group receive UL forbidden time by an M2M Group paging message, the group of M2M devices shall not transmit ranging request for re-entry or location update during the UL forbidden time duration within upcoming paging unavailable interval(s). Start offset of the UL forbidden time is transmitted by the M2M Group paging message.]

[16.2.x. Urgent Paging Mode

      - If the server has an urgent need to communicate with a specific group of devices, the ABS should broadcast a waiting request message to all other groups prior to initiating paging. The devices in other groups should not initiate network re-entry until the delay period indicated in the message has elapsed.

      - The devices could terminate the delay period early and return to normal operation if an urgent uplink/downlink data traffic is required.]

[16.2.18.7.2 Location update

16.2.18.7.2.1 Cell based update support

Cell based update is intended as a mechanism to allow the M2M device for tracking service to report its own location information, typically when the M2M device detects that the preferred BS is changed. For cell based update, M2M device shall negotiate the capability for cell based update during the capability negotiation phase of the network entry. When an M2M device that has the capability of the cell based update detects that the selected preferred cell does not support its currently attached cell, it shall perform the cell based update. When an M2M device that has the capability of cell based update enters the idle mode, paging group is not assigned to the M2M device.]

16.2.18.7.X Network re-entry from idle mode for M2M devices

[An M2M device [which is individual or belongs to one M2M group] may perform network reentry from idle mode voluntarily for UL transmission. Network also may request the M2M device [or M2M group] to perform network reentry from idle mode by transmitting a paging message.] For network reentry from Idle Mode, ranging parameters may be different for [distinct] M2M devices or M2M groups [to avoid the network access congestion].

[16.2.18.7.1 Network reentry from idle mode for M2M devices

In order to reduce network congestion produced by a large number of M2M devices, network reentry from idle mode procedure for M2M devices should be based on group.

After a M2M group is established, BS should allocate a ranging code set to M2M groups based on M2M group ID. When a M2M group is expected to report their data, a group member which is a delegate of the M2M group sends a ranging code from the above ranging code set to BS. A M2M device in a M2M group is called as a group member (GM). The GM which sends the ranging code is called as group delegate (GD). This ranging code is called as GDC (group delegate code).

After BS decoded this ranging code, BS sends AAI-RNG-ACK to respond to this ranging code, which includes three ranging status (success, abort and continue), as in the 802.16m system (see 16.2.3.3).

All GM in this group listen to AAI-RNG-ACK in the ranging procedure based on group. After received AAI-RNG-ACK including that ranging code, GM acts according to ranging status.

  - If ranging status is “success”, all GM in this group start its network reentry procedure by transmitting a ranging code. The ranging code chosen may be in legacy code sets for ranging or may be a new one;

  - If ranging status is “abort”, all GM in this group shall start the ranging abort timer and abort the ranging process until the ranging abort timer expires. Similar as the 802.16m system, after abort timer is expired, this group should restart the ranging procedure based on group.

  - If ranging status is “continue”, the GD in this group shall adjust its parameters accordingly and continue the ranging process as done on the first entry using the same GDC.

16.2.18.7.1 Network reentry from idle mode for M2M devices

BS assigns dedicated ranging codes and ranging opportunities to a M2M group.]

[16.2.18.X  Network reentry from idle mode with priority

The devices shall support to adjust the opportunity periodicity according to the opportunity configuration and priority factor. ]

[16.2.18.X  Network reentry from idle mode

For the network reentry from idle mode, the fixed M2M devices shall support to initiate network reentry with the ABS by sending a ranging sequence from Reentry Ranging domain.]

[16.2.18.5.1 Network reentry from idle mode for M2M

If an M2M device decodes a paging message that contains its identification and indication for network reentry, it shall initiate network reentry to the network.

For the network reentry indicated by an M2M group paging message, an M2M device shall follow the procedure described below.

An M2M group paging message may include a group waiting time which is assigned to each M2M group that requires network re-entry. Upon reception of the group waiting time, the M2M devices belonging to the M2M group shall wait the period of time before selecting ranging opportunities. In this case, the M2M devices determine ranging opportunities using the following equation:

Ranging opportunity offset == M2M Device ID mod K, the value of K is the length of ranging opportunity window.

The value of K may be determined based on the number of M2M devices and indicated by an M2M group paging message. Start of the ranging opportunity offset should be right after a group waiting time expires.

If the M2M device receives neither AAI-RNG-ACK nor UL resource allocation until T31 timer expires, the M2M device considers its ranging request is failed and restarts ranging procedure as defined in 16.2.15.3 with the exception as following:

When the M2M device restarts ranging procedure, it should wait for its priority time duration which is indicated during initial network entry. The priority time duration indicates a period of time that the M2M device waits before selecting ranging random backoff value.]

16.2.18.5 Network reentry from idle mode

[An ABS may assign dedicated ranging opportunities to an M2M group for network re-entry.]

[16.2.18.7 Idle mode for M2M application

When an AMS which supports M2M application enters idle mode, the AMS follows the procedure defined in 16.2.18 with exception of procedures described in this section.

During idle mode initiation through AAI-DREG-REQ/AAI-DREG-RSP messages, ranging time offsetaccess priority index is assigned to the AMS by the ABS through AAI-DREG-RSP message. The ranging time offsetaccess priority index indicates ranging time offsetparameters defined in SCD message. An AMS with M2M application shall start random backoff after time offset indicated by ranging time offset indexuse ranging parameters and the ranging parameters are used for ranging process during network reentry from idle mode. If a paging message includes a ranging time offsetaccess priority index which is different from one assigned during idle mode initiation, it shall use ranging time offsetparameters indicated by the ranging time offsetaccess priority index in the paging message for ranging process during network reentry from idle mode.]

[16.2.x Group Congestion Control

      - In the event of network congestion, the BS should send an backoff instruction index (BII) to the group that is to be paged.

      - The device should map the BII of its group to initiate the random backoff procedure. The backoff instruction list is shown in Table XXX.

      - In Table XXX, S denotes the size of ranging backoff window, and U(0,1) represents a uniform random variable between 0 and 1.

]

[16.2.X Priority factor

The M2M devices shall support to be labeled with various priority factors. In addition, an M2M device shall know its own priority factor.]

[16.2.29 Device collaboration for M2M

Device collaboration is a technique where a certain device helps the other device’s transmission. Especially, the device collaboration is supported for lower power consumption of a specific M2M source device by reducing the UL pathloss and relevant Tx power of the source M2M device. For supporting device collaboration, the cooperative M2M device shall stop UL transmission during the interval when listening to the UL signal from the source M2M device.

16.2.29.1 Device collaboration mode

The Device Collaboration mode is configured to support the cooperative M2M device receiving the UL data & control of the source M2M device and relaying them to the BS. 	Once the device collaboration is triggered, the Device Collaboration mode is configured to both of the source and cooperative M2M devices.

16.2.29.1.2 Frame based on-off switching

During the device collaboration mode, both of the source and cooperative devices operate with switched manner of two different device states; On-state and off-state. The interval of the ‘on-state’ and ‘off-state’ is identically equal to a frame interval (i.e. 5ms) respectively. The device state of the source device shall be cross-paired with the device state of the cooperative device, i.e. the ‘on-state’ of the source device shall be overlapped with the ‘off-state’ of the cooperative device and vice versa. Fig. x shows the example of the frame based on-off switched operation of the source and cooperative devices in device collaboration mode.

- On-state

During the ‘on-state’ interval, the source device shall receive the DL data and control from the BS directly in DL subframes, and transmit UL data and control to the cooperative device in UL subframes. The cooperative device shall also receive DL data and control for itself from the BS in DL subframes, and transmit its own UL data and control to the BS in UL subframes. And additionally, it shall relay the UL data and control of the source device, received in the D2D listening interval of the previous ‘off-state’ interval, to the BS.

- Off-state

During the ‘off-state’ interval, the source device may conduct as power saving mode. It may not receive any DL signal from the BS and transmit any UL signal. The cooperative device shall receive UL data and control from the source device through the UL subframe(s) (i.e. D2D listening interval) in ‘off-state’ interval.

Figure x – Example of source and cooperative device operation during device collaboration mode.]

16.3. 8.1.4 Ranging channel

16.3.8.1.4.2 Ranging channel for synchronized AMSs

[The ranging preamble (RP) codes are classified into periodic ranging and reentry ranging preamble codes. The periodic RP codes shall be used for periodical synchronization between ABS and AMS. The reentry RP codes shall be used for fixed devices performing network reentry from idle mode. For a ranging code opportunity, each AMS randomly chooses one of the RP codes from the available RP codes set in a cell.]

----------------- End of the text proposal ---------------------------------------------------------------------------------------