| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Connection management for multicast operation of M2M group |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon ETRI | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p-10/0018 |  |
| Abstract | This contribution proposes texts for connection management for multicast operation of M2M group. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Connection management for multicast operation of M2M group

Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon

ETRI

# Introduction

This contribution includes some comments on DEV RG output [1] to clarify the connection management for multicast operation of M2M group.

IEEE 802.16p BS shall provide the multicast service for M2M devices in idle state as well as connected state.

A M2M group identifier (MGID) and DSx procedures are used for multicast traffic transmission. Multiple service flows for multicast traffic can be configured on a M2M device. But, the same number of service flows shall be configured on the M2M device that belongs to same M2M group. To receive the multicast data in idle mode without network reentry, the M2M device is requested to retain the associated information of all service flows set by successful AAI-DSA and AAI-DSC transactions for the multicast connection.

# References

[1] IEEE 802.16p-11/0001, “IEEE 802.16p Machine to Machine (M2M): Proposed Text from Large number of devices (DEV) Rapporteur Group,” Feb. 2011.

# Proposed Text

We differentiated any texts modified by this contribution from those modified by DEV RG contribution by using a different color as follows.

- Texts added by DEV RG contribution: blue underlined texts

- Texts deleted by DEV RG contribution: red strikeout texts

- Texts added by this contribution: green underlined texts

- Texts deleted by this contribution: green strikeout texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

3. Definitions

M2M group: a group of devices that share one or more features in common and belong to same M2M subscriber

16.2.3.9 AAI-REG -RSP

Table 686—AAI-REG-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| M2M group ID(MGID) | TBD | M2M group identifier which the M2M device belongs to |  |

16.2.3.47.1 AAI-DSA-REQ

Table 734—AAI-DSA-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Multicast indicator | 1 | Indicates this service flow is a multicast service flow 1: multicast | Optional |
| [MMGID | 12 | M2M Multicast group ID | Present if associated flow has M2M Multicast Group ID] |

16.2.3.47.2 AAI-DSA-RSP

Table 735—AAI-DSA-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Multicast indicator | 1 | Indicates this service flow is a multicast service flow 1: multicast | Optional |
| [MMGID | 12 | M2M Multicast group ID | Present if associated flow has M2M Multicast Group ID] |

16.2.3.47.4 AAI-DSC-REQ

Table 737—AAI-DSC-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Multicast indicator | 1 | Indicates this service flow is a multicast service flow 1: multicast | Optional |
| [MMGID | 12 | M2M Multicast group ID | Present if associated flow has M2M Multicast Group ID] |

16.2.3.47.5 AAI-DSC-RSP

Table 738—AAI-DSC-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Multicast indicator | 1 | Indicates this service flow is a multicast service flow 1: multicast | Optional |
| [MMGID | 12 | M2M Multicast group ID | Present if associated flow has M2M Multicast Group ID] |

### 16.2.28 Support for Multicast Service

16.2.28.4 Multicast Operation for machine to machine (M2M) applications

When an IEEE 802.16p BS receives DL multicast data for M2M application from the network, it shall send DL multicast data to M2M devices belonging to a M2M Group.  A M2M Group Identifier (MGID) defined in 16.2.1.x is used for multicast traffic transmission.

[An ABS may initiate the DSA procedure for multicast connections. The AMS's discovery and registration of multicast services with an ABS through upper layer signaling are outside the scope of this standard. The AAI-DSA, AAI-DSC and AAI-DSD messages are used to establish, change, and delete multicast service flows respectively. The ABS shall send the AAI-DSA-REQ/RSP to the AMS with the relevant multicast parameters including M2M Multicast Group ID and FID.]

The DSx procedures are used to establish, change and delete multicast service flows respectively. The 16p BS shall send the AAI-DSA-REQ/RSP to the M2M device with the relevant multicast parameter including FID and multicast indicator. And the number of service flows configured on the M2M device within a M2M group shall be same.

[For multicast connections, ARQ is not applicable, and the Null SAID is used as the target SAID field in AAI-DSA-REQ/RSP message.]

[The multicast data assignment is indicated using Broadcast Assignment A-MAP IE with Function index 0b11 as described in Table 860.]

[In this case, 16-bit CRC of the A-MAP IE is masked with 0b0011 (4-bit) plus 12-bit MMGID.]

16.2.28.4.1 M2M Multicast operation in idle mode

IEEE 802.16p BS shall provide the multicast service for M2M devices in idle mode.

Before an 802.16p BS sends DL multicast data, the 802.16p BS shall transmit the paging message including the multicast traffic indication to M2M devices during the paging listening intervals of the M2M devices. When an M2M device receives the multicast traffic indication through the paging message during its paging listening interval, the M2M device shall start receiving the DL multicast data without the idle mode termination [except for the case of UL feedback] [regardless of UL feedback].

[The multicast transmission start time shall be included in the paging message in order to indicate when the DL multicast data is sent. The M2M device shall start receiving the multicast data from the multicast transmission start time.]

[When the multicast data transmission ends, an IEEE 802.16p BS shall transmit the multicast transmission end (MTE) indicator. Upon receiving the MTE indicator the device shall not perform the process of receiving the multicast data any more.]

To receive the multicast data in idle mode without network reentry, the M2M device is requested to retain the associated information of all service flows set by successful AAI-DSA and AAI-DSC transactions for the multicast connection. In particular it includes FIDs and related description.

----------------- End of the text proposal ----------------------------------------------------------------------------------------