| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Access Distribution of Network Reentry M2M Device |  |
| Date Submitted | 2011-3-06 |  |
| Source(s) | Guanghui Fan Erik Colban George Calcev Huawei Technologies | E-mail: fanguanghui@huawei.com ecolban@huawei.com George.Calcev@huawei.com |
| Re: | IEEE 802.16p call for AWD Content |  |
| Abstract | Proposal for access distribution of network reentry M2M device using group information. |  |
| Purpose | To be discussed and adopted in IEEE 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Acess Distribution of Network Reentry M2M Device

Guanghui Fan, Erik Colban, George Calcev

Huawei Technologies

# Introduction

For M2M devices, grouping mechanism is used for the purpose of group paging, multicasting and etc. When members of a group are paged and need to perform network reentry, large number of devices will perform random ranging procedure which will bring severe collisions.

As discussed in 16p RG group, an M2M group identifier, which is unique in the entire network and named MGID, will be defined for each M2M group. For each grouped M2M device, an identifier, named M2M_ID also needs to be defined to uniquely identify it in its group. The M2M group is managed by the network side and may have a large size. For each BS, it may only serves part members of each M2M group. For example, one BS may serve N members of an M2M group with total member size of M (N<M). This contribution proposes how to use N, M2M_ID and other info to control the mass accesses from the N members in that BS. The solution is described in the following proposed text.

# Text Proposal

------------------------------------------------Start of the proposed Text Part 1------------------------------------------------

16.2.18.5.1 Network reentry from idle mode for M2M device

16.2.18.5.1.1  Network reentry from idle mode for individually paged M2M device

When an M2M device receives a individual paging message with its paging group ID and action code = [TBD], the M2M shall do network reentry procedure as described below.

[TBD]

16.2.18.5.1.2  Network reentry from idle mode for group paged M2M device

When an M2M device receives a group paging message with its group ID and action code is [TBD], the M2M shall do network reentry procedure as described below.

The M2M shall wait for a period ranging offset time before starting sending the ranging code for uplink synchronization. The ranging offset value is a relative value from the frame where the group paging message is transmitted and derived by equation:

Ranging_Offset_Time = ( M2M_ID mod M )*floor(Initial_Access_Window_Length/M)

Initial_Access_Window_Length is the length of the window for initial access (i.e. send the ranging code for the first time) of paged M2M devices and indicated in group paging message.

The number of M2M devices which paged in the M2M group in the ABS and indicated by Num of Paged Members field in group paging message.

After the ranging offset waiting time, the M2M device will perform network reentry by randomly selecting a ranging channel from the closet frame which contains ranging opportunities and a ranging sequence from the Handover Ranging domain. The following random access procedure is the same as AMS.

------------------------------------------------End of the proposed Text Part 1-------------------------------------------------

------------------------------------------------Start of the proposed Text Part 2------------------------------------------------

16.2.3.x AAI-GRP-PAG-ADV

This advertising message is used for paging M2M group members.

Table xxx —AAI-GRP-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Num_Paging_Group | TBD | The number of paged groups in this message |  |
| For (i=0; i< Num_Paging Group; i++) { |  |  |  |
| M2M Group ID (MGID) | TBD | MGID of the each paged group in this message |  |
| Action Code | TBD | TBD |  |
| Initial_Access_Window | TBD | The length of the window in unit of frames for initial access (i.e. send the ranging code for the first time) of paged M2M devices for each paged group | Shall be present when the action code is [TBD] |
| Num of paged Members | TBD | The number of M2M devices which paged in each M2M group in this ABS. | Shall be present when the action code is [TBD] |
| } |  |  |  |
| ………. |  |  |  |

------------------------------------------------End of the proposed Text Part 2-------------------------------------------------