| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handling Normal Operation Congestion in M2M Networks |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | George Calcev, Erik Colban, Bin Chen, George Cummings Huawei Technologies | E-mail: George.Calcev@huawei.com ecolban@huawei.com binchen@huawei.com gcummings@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments on the M2M AWD, as announced by the Task Group chair, on January 18th, 2011. |  |
| Abstract | This contribution proposes discussion of M2M congestion in normal operation |  |
| Purpose | To be discussed and adopted by 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Handling Normal Operation Congestion in M2M networks

George Calcev, Erik Colban, Bin Chen, George Cummings

Huawei Technologies

## Introduction

One of the most important features of M2M communication is that it needs to handle a large number of devices (there may be as many as 30,000 devices in a cell). Most of the M2M devices will be programmed to upload periodic reports to the central servers or to report asynchronous events such as sudden changes in temperature, humidity, mechanical fractures, etc. Many discussions in the M2M TG were dedicated to handling congestion resulted from catastrophic events such as large power loss. However, this type of catastrophic congestion that overwhelms the wireless network is relatively rare. We think that the congestion created by the superposition of asynchronous measurements reports or by sudden changes in the environmental variables will be a very important issue to deal with on a daily or hourly basis. A rigorous forecast of this congestion might be a difficult or an impossible task due randomness and delays introduced by contention in the ranging channel, retransmissions caused by the propagation conditions or by accidental hardware failures and abrupt changes in measured variables.

We ask the M2M TG to discuss and comment on the problem of daily and hourly congestion that might occur in the M2M networks in normal conditions. If the TG sees some merit in addressing this problem we will bring a new contribution proposing a procedure that helps mitigating daily congestion.