| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on the addressing for 802.16p |  |
| Date Submitted | 2011-03-13 |  |
| Source(s) | Erik Colban, Bin Chen, George Calcev Huawei | E-mail: binchen@huawei.com ecolban@huawei.com George.Calcev@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for contributions from 802.16, M2M |  |
| Abstract | This contribution proposes to address for 802.16p device and identifier in A-MAP |  |
| Purpose | To be discussed and adopted by 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal on the addressing for 802.16p

Erik Colban, Bin Chen, George Calcev
Huawei

# Introduction

This contribution is about a discussion on addressing about Multicast Group ID and M2M Device ID (MGID).

In contribution C80216p-rg-11_0038r2, it is said that a multicast group ID (MGID) is unique in the entire network. In contribution C80216p-rg-11_0016, multiple M2M devices share the same STID, while in C80216p-rg-11_0043r1.doc, a second level M2M device ID (MDID) is used to expand the address domain for M2M service. It is a good idea to expand the address domain to support large number of devices and, at the same time, support group multicast for M2M service easily.

However, there are some issues we are concerned about.

- In 0038r2, a broadcast/multicast A-MAP IE is used for the M2M multicast, and it looks like that there are just 12 bits for MGID. The question is whether that is enough.

- Sharing STID for multiple M2M devices would cause very high complexity in handling and extra overhead on payload.

- 0043r1 is using a second level M2M device ID to identify a M2M device in the network. The problem is that it needs a supplementary A-MAP to indicate the second MDID, which would cause too much overhead.

To address the above issues, we would like to propose some solutions for discussion in the group.

- If we want to define the MGID/MDID on the same address domain of current STID address domain, we need to define them exactly on the reserved identifiers from 0b001/0b0110/0b0111 and may leave 0b0001/0b0010 part for future expanding. So that we can use 12288 identifiers for MGID/MDID. Here we are proposing to the group to discuss whether this is enough for uniquely identifying an M2M group/device. If we want ensure sufficient support for M2M, we may need more addresses for M2M devices. To define a new address domain for the M2M devices, we would like to suggest 2 options.

- Option 1: Define a new A-MAP type other than the existing A-A-MAP and put the new A-MAP on another A-MAP resource. This will create a new address domain dedicated for M2M in 16m (and 16e). Of course, we do not need to define totally new A-MAP contents, but just re-use the existing A-A-MAP contents.

- Option 2: use a reserved bit in the A-A-MAP to define a new address domain for M2M devices. For example, when the 1 bit on the A-A-MAP is 0b0, the CRC is masked by the STID (the old address domain defined in table 843/844/845.

We would like to ask our 16p colleagues to discuss this proposal at the F2F.

# Text Proposal

For future discussion and addition. Here we just put an example to explain the solution of option 2.

------------------------------------------------------ Start of Proposed Text -----------------------------------------------------

Table 846— DL Basic Assignment A-MAP IE

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| DL_Basic_Assignment_AMAP_ IE() { |  | … |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| …… |  |  |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Reserved AddressDomain | 1 | Reserved bit Indicator of address domain used for this A-MAP. 0b0: the address domain of legacy STID is used to mask the CRC, which is defined in table 843/844/845 0b1: the address domain of M2M devices is used to mask the CRC, including MGID and MDID. |
| } |  |  |