| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Addressing for M2M Devices and Corresponding Masking Schemes of A-A-MAP IEs |  |
| Date Submitted | 2011-3-6 |  |
| Source(s) | Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kenchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen Institute for Information Industry (III) | E-mail: eating@iii.org.tw |
| Re: | Call for Comments on IEEE P802.16p AWD |  |
| Abstract | This contribution proposes addressing for M2M devices |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Addressing for M2M Devices and Corresponding Masking Schemes of A-A-MAP IEs

Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kanchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen

Institute for Information Industry (III)

# Introduction

During the rapporteur group discussion, we have discussed the capacity of the addressing space in 16p system. However, there is still no consensus on the addressing space of an M2M device and an M2M group. This contribution proposes the addressing for an M2M device and for an M2M group based on the efficiency of the masking schemes of the assignment A-MAP IEs. Grouping is an efficient scheme in transmitting multicast data to a group of devices instead unicasting a copy of data to each device individually. We propose to separate the addressing of an M2M device for unicasting transmission and the group addressing of an M2M group for multicasting transmission. Our consideration has following three:

- Each unicast A-MAP IE does not need adds overhead from the group addressing. The overhead is bad especially for transmission by large number of devices.

- If we should follow the identification scheme and resource block assignment of WirelessMAN-Advanced Air Interface, which means that an M2M device ID and radio resource for an M2M device are assigned by the Base Station, there are no devices with the same M2M device ID in a cell. Therefore an M2M device ID can be solely used to identify a M2M device in a cell.

- For multicast transmission, grouping is efficiency in transmitting “a data” to all the devices within a group, and it is not necessary to identify each M2M device within a M2M group.

So we shall separately discuss the unicast transmission for an M2M device and the multicast transmission for an M2M group. In suggested remedy 1, a masking scheme for both information and CRC is proposed to let M2M device exclusively identify the unicast Assignment A-MAP IEs with 16-bit M2M_ID. In suggested remedy 2, a masking scheme for an M2M group is proposed to let M2M group exclusively identify the Broadcast Assignment A-MAP IE with 16-bit MGID.

# Reference

[1] IEEE P802.16m/D12, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” Feb. 2011.

- Text proposal for inclusion in the 802.16p AWD

Suggested Remedy 1

[Add the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.1.3.2 M2M Device Identifier (M2M_ID)

A 16-bit M2M_ID uniquely identifies an M2M device in the domain of the ABS supporting M2M service.

16.3.5.3.2.4 Assignment A-MAP

The Assignment A-MAP (A-A-MAP) shall include one or multiple A-A-MAP IEs and each A-A-MAP IE is

encoded separately. Figure 540 describes the procedure for constructing A-A-MAP symbols.

The 40 bits of information contained in the A-A-MAP IE are masked by a sequence generated by a PRBS generator. The mask, when XOR'ed with the A-A-MAP IE, randomizes the A-A-MAP IE. When a MAPMask Seed is used to initialize the PRBS generator, the mask also conceals the contents of the A-A-MAP IE from unintended users. The PRBS generator and the randomization process are described in 16.3.10.1.3.

When the 16-bit CRC Mask Type Indicator is 0b000, the 15-bit initial vector of the PRBS generator shall be the value of the MAPMask Seed that was transmitted to the AMS in the same MAC control message as the STID or TSTID value used as the 16-bit CRC Mask Masking Code. Otherwise, the 15-bit initial vector of the PRBS generator shall be the same as the 16-bit A-A-MAP CRC mask in Table 843, excluding the 1 bit masking prefix.

For 16p devices, the consecutive 4 bits from the second bit at the right end of the randomized 40 bit A-A-MAP IE is further masked by the 4 bit MSBs of 16 bit STID. A 16-bit CRC, for which CRC16-CCITT as defined in ITU-T recommendation X.25 is used, is generated based on the randomized sequence of information bits of the assignment A-MAP IE. Denote the randomized assignment A-MAP IE for 16m device or 4 bit masked assignment A-MAP IE after randomization for 16p device by m(x) , the 16-bit CRC by p(x), and the 16-bit CRC mask by q(x) which is generated according to 16.3.5.5.2.4. The 16-bit CRC mask is applied to the 16-bit CRC by bitwise XOR operation.

The masked CRC is then appended to the randomized sequence of information bits of the assignment A-MAP IE, resulting in a bit sequence of m(x)  x16 + p(x)  q(x). The resulting sequence of bits shall be encoded by the TBCC described in 16.3.10.2.

16.3.5.5.2.4 Assignment A-MAP IE

Table 843.—Description of CRC Mask

| Masking Prefix (1 bitMSB) | Remaining 15 bit LSBs |  |
| --- | --- | --- |
| 0b0 | Type Indicator | Masking Code |
|  | 0b000 | 12 bit STID for AMSs or 12 bit LSBs M2M_ID for M2M devices or TSTID |
|  | 0b001 | Refer to Table 844 |
|  | 0b010 | Refer to Table 845 |
| 0b1 | 15 bit RA-ID: The RA-ID is derived from the AMS' random access attributes (i.e., superframe number (LSB 5bits), frame_index (2 bits), preamble code index for ranging or BR (6 bits) and opportunity index for ranging or BR (2 bits)) as defined below: RA-ID = (LSB 5bits of superframe number ¦ frame_index ¦ preamble_code_index ¦ opportunity_index) |  |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

Suggested Remedy 2

[Add the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.1 Addressing

16.2.1.3 Address for machine to machine application

16.2.1.3.1 M2M Group Identifier (MGID)

A TBD15-bit value that uniquely identifies an M2M group which one or more M2M devices belong to in the domain of the entire network. This ID shall be used to control a group of devices (e.g., group paging) or to transmit the multicast data to the M2M devices.

The 15 bit space is used to assign MGID to M2M devices. The CRC for MAP_IE is masked with this 15 bit MGID for M2M Devices.

The CRC mask for unicast MAP IE and multicast MAP IE for M2M devices and M2M Group is set as follows.

  - M2M Device

    - CRC mask = 0000 | 12-bit LSBs M2M_ID (4-bit MSBs M2M_ID is used to mask IE contents)

  - M2M Group

    - CRC mask = 0 | 15 bit MGID

      - 4 bit MSBs is 0011~0111

16.3.5.3.2.4 Assignment A-MAP

When the 16-bit CRC Mask is generated with masking prefix 0b0 and 15 bit MGID, the 15-bit initial vector of the PRBS generator shall be the value of the MAPMask Seed that was transmitted to the AMS in the same MAC control message as the 15 bit MGID value used for generating the 16 bit CRC mask. Otherwise, the 15-bit initial vector of the PRBS generator shall be the same as the 16-bit A-A-MAP CRC mask in Table 843, excluding the 1 bit masking prefix.

16.3.5.5.2.4 Assignment A-MAP IE

Table 843.—Description of CRC Mask

| Masking Prefix (1 bit MSB) | Remaining 15 bit LSBs |  |
| --- | --- | --- |
| 0b0 | Type Indicator | Masking Code |
|  | 0b000 | 12 bit STID or TSTID |
|  | 0b001 | Refer to Table 844 |
|  | 0b010 | Refer to Table 845 |
|  | 0b011 – 0b111 | 12 LSBs of 15 bit MGID |
| 0b1 | 15 bit RA-ID: The RA-ID is derived from the AMS' random access attributes (i.e., superframe number (LSB 5bits), frame_index (2 bits), preamble code index for ranging or BR (6 bits) and opportunity index for ranging or BR (2 bits)) as defined below: RA-ID = (LSB 5bits of superframe number ¦ frame_index ¦ preamble_code_index ¦ opportunity_index) |  |

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

Table 860—Broadcast Assignment A-MAP IE

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: reservedThis IE carries multicast assignment information for M2M application |
| … |  |  |
| } else if {Function Index == 0b11} { |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 958 |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number fo AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 16 | Reserved bits |
| } |  |  |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------