| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Evaluation Methodology profile for 802.16p |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Belal Hamzeh Rui Huang Honggang Li Yang-Seok Choi Shantidev Mohanty | E-mail: { Belal.Hamzeh, Rui. Huang, Honggang.Li, Yang-Seok.Choi, Shantidev.Mohanty}@intel.com |
| Re: | IEEE 802.16p-10/0018 (EMD for 16p) |  |
| Abstract | This contribution proposes an evaluation methodology profile for machine type communications. |  |
| Purpose | To be discussed and adopted either as an separate document or can be added to IEEE 802.16m-08/004r5 (the EMD of 16m) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Evaluation Methodology for 802.16p

Belal Hamzeh, Rui Huang, Honggang Li, Yang-Seok Choi, Shantidev Mohanty

Intel Corporation

# Introduction

Due to the specific nature and requirements of machine type communications, special considerations need to be considered when evaluating the performance of various system aspects related to machine to machine (M2M) communications. This contribution proposes the evaluation methodology for the 16p system.

# Proposed Text

==================Start of proposed text=====================================

To evaluate various aspects related to machine type communication, we propose an evaluation methodology based on the previously adopted evaluation methodology for IEEE 802.16m; with special extensions to accommodate for nature of M2M devices and expected deployment scenarios.

To this end, we propose the following profile for the evaluation of IEEE 802.16p contributions

- Mobile device model: The current 802.16m EMD only considers devices with a single received antenna, we propose the inclusion of single receiver antenna devices, in the evaluation.

- Channel model: Current 802.16m EMD considers only mobile devices, we recommend the inclusion of channel models specific to fixed devices, such as the SUI-3 and SUI-4 channel models

- Channel mix: Given that most of devices are expected to be fixed devices, and given that various mobile aspects of 802.16m have already been verified in 802.16m, we propse that the channel mix for 802.16p be 100% fixed devices, 50% SUI-3 and 50% SUI-4 channels.

- Scheduling: To support large volume of M2M devices with low power transmission, group-based resource scheduling should be considered, e.g. the ABS can allocate the specific channel resource to a group of M2M devices to transmit and receive packets. Within the group, the allocated channel can be accessed by M2M devices based on contention or contention free mechanism. The group-based proportional fairness scheduler is based on the equivalent proportional fairness among the multiple M2M groups.

- Performance metrics: performance metrics specific to M2M devices need to be included, specifically:

- The average power consumption; defined as

Where k=1…K is the index of user, j=1… is the index of packet transmitted for the kth user, is the transmit power for UL or  the receiving power consumption for DL for the jth packet and kth user, is the time consuming for the jth packet and kth user.

For simplicity, only the UL can be considered for general evaluation, because it is related to the transmit power of M2M device. The DL can be considered if the power consumption for receiving processing can be known.

- The CDF of averaged power consumption per user

- 13.2.5.2 The averaged power consumption per user

- 13.2.5.3 The averaged power consumption at 5% percentile of coverage

- Deployment model: M2M devices will be mostly a mix of in-home devices and commercial-use devices, with varying traffic characteristics; for this, we propose the following as a basis for a deployment model:

In home devices

| Appliances/ Devices | Average Message Transaction Rate/s | Message period (s) | Average Message Size (Bytes) | Number Per Home | Average Date Rate per home (b/s) |
| --- | --- | --- | --- | --- | --- |
| Home Security System | 0.001666667 | 600 | 20 | 1 | 0.266666667 |
| Elderly Sensor Devices | 0.016666667 | 60 | 128 | 0.1 | 1.706666667 |
| Refrigerator | 0.000277778 | 3600 | 30 | 1 | 0.066666667 |
| Clothes Washer | 1.15741E-05 | 86400 | 8 | 1 | 0.000740741 |
| Clothes Dryer | 1.15741E-05 | 86400 | 8 | 1 | 0.000740741 |
| Dishwasher | 1.15741E-05 | 86400 | 8 | 1 | 0.000740741 |
| Freezer | 1.15741E-05 | 86400 | 30 | 1 | 0.002777778 |
| Stoves/Ovens | 1.15741E-05 | 86400 | 8 | 1 | 0.000740741 |
| Microwaves | 1.15741E-05 | 86400 | 8 | 1 | 0.000740741 |
| Coffee Makers | 1.15741E-05 | 86400 | 8 | 1 | 0.000740741 |
| Toaster Ovens | 1.15741E-05 | 86400 | 8 | 1 | 0.000740741 |
| PHEV to SM | 2.31E-04 | 4320 | 97.6 | 2 | 0.361481481 |
| Smart Meter | 1.16E-04 | 8640 | 2017 | 3 | 5.602777778 |
| Aggregated data rate per home |  |  |  |  | 6.476222222 |

| Scenario | Cell Radius(m) | Cell Area (sq. meters) | Population Denisty (per sq. meters) | Household Size | Average number of homes within cell |
| --- | --- | --- | --- | --- | --- |
| Urban(New York City) | 1000 | 3140000 | 0.01 | 2.6 | 12076.92308 |
| Sub-urban(Washington D.C.) | 1500 | 7065000 | 0.003846154 | 2.6 | 10451.18343 |

Commercial usage devices

| Appliances/ Devices | Average Message Transaction Rate/s | Message period (s) | Average Message Size (Bytes) | Date Rate (b/s) |
| --- | --- | --- | --- | --- |
| Credit Machine in grocery | 0.008333333 | 120 | 24 | 1.6 |
| Credit Machine in shop | 0.000555556 | 1800 | 24 | 0.106666667 |
| Credit Machine in Restaurant | 2.77778E-05 | 36000 | 24 | 0.005333333 |
| Roadway Signs | 0.033333333 | 30 | 1 | 0.266666667 |
| Traffic Lights | 0.016666667 | 60 | 1 | 0.133333333 |
| Traffic Sensors | 0.016666667 | 60 | 1 | 0.133333333 |
| Movie Rental Machines | 1.15741E-05 | 86400 | 152 | 0.014074074 |

| Scenario | Land Area (sq. meters) |  | Number of grocery stores |  | Number of shops |  | Number of restaurants |  | Number of Movie Rental Machine | Number of roadway signs | Number of traffic lights | Number of traffic sensors | Aggregated Data Rate in Cell(b/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(New York City) | 3140000.00 |  | 657.73 |  | 4011.91 |  | 2993.95 |  | 328.87 | 142.32 | 47.58 | 47.58 | 1609.96 |
| Sub-urban(Washington D.C.) | 7065000.00 |  | 163.36 |  | 736.38 |  | 1735.49 |  | 81.68 | 444.27 | 44.43 | 44.43 | 436.74 |
| Scenario |  | In Home: Aggregated Data Rate in Cell(b/s) |  | In City: Aggregated Data Rate in Cell(b/s) |  | Total: Aggregated Data Rate in Cell(b/s) |  |  |  |  |  |  |  |
| Urban(New York City) |  | 78212.84 |  | 1609.96 |  | 79822.80 |  |  |  |  |  |  |  |
| Sub-urban(Washington D.C.) |  | 67684.19 |  | 436.74 |  | 68120.93 |  |  |  |  |  |  |  |

==================End of proposed text====================================