| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Ranging Channel Design Considerations for M2M in 16e |  |
| Date Submitted | 2011-05-18 |  |
| Source(s) | Shantidev Mohanty Belal Hamzeh Yang-Seok Choi Xiangying Yang Yi Hsuan Joey Chou Nageen Himayat Kerstin Johnsson Intel Corporation Inbar Anson Bratspiess Sequans Erik Colban Huawei Chun-Yen Hsu, Tsung-Yu Tsai Institute for Information Industry (III) Jaesun Cha ETRI Hassan Alkanani NEC Hyunjeong Kang, Anil Agarwal Samsung | E-mail: { shantidev.mohanty}@intel.com |
| Re: | IEEE 802.16p-10/0018r1 |  |
| Abstract | This contribution proposes methods required to address the large number of ranging contentions arising from M2M devices |  |
| Purpose | To be discussed and adopted into 16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M Ranging Channel Design Considerations for M2M in 16e

Shantidev Mohanty, Belal Hamzeh, Yang-Seok Choi, Xiangying Yang,

Yi Hsuan, Joey Chou, Nageen Himayat, Kerstin Johnsson

Intel Corporation

Inbar Anson Bratspiess

Sequans

Erik Colban

Huawei

Chun-Yen Hsu, Tsung-Yu Tsai

Institute for Information Industry (III)

Jaesun Cha

ETRI

Hassan Alkanani

NEC

Hyunjeong Kang, Anil Agarwal

Samsung

# Introduction

In the previous meeting it was agreed to have dedicated ranging channel for M2M devices to address the large ranging load coming from the M2M devices.

This contribution proposes changes to the 16e standard to implement the dedicated ranging channel for M2M devices.

# Proposed Text

==================Start of proposed text=====================================

[Add the following text after line 14 in page 2.]

Table 568—UCD channel encodings

| Name | Type (1 byte) | Length | Value |
| --- | --- | --- | --- |
| M2M Ranging Region | TBD | 6 | Bit 0: dedicated ranging indicator Bits 1–2: ranging method Bits 2–9: num subchannels Bits 10–16: num OFDMA symbols Bits 17–23: subchannel offset Bits 24–31: OFDMA symbol offset Bits 32–34, Parameter d that defines periodicity of 2d frames Bits 35–39, Allocation phase expressed in frames, 0 ≤ Allocation Phase < periodicity (=2d) Bit 40, When this bit is set to 1, the ranging allocation defined by Ranging Region TLV Type = 212 shall not be used by M2M devices Bits 41-47, Reserved |

==================End of proposed text====================================