| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16p Machine to Machine (M2M): Proposed Text on M2M Group Identifier |  |
| Date Submitted | 2011-03-16 |  |
| Source(s) | Kaushik Josiam Samsung Electronics Ming-Hung Tao ITRI DEV-RG chairs | Email: kjosiam@sta.samsung.com, mhtao@itri.org.tw |
| Re: | IEEE 802.16p-10/0016 - M2M Rapporteur Group Plan, Schedule, and Guidelines |  |
| Abstract | The proposed text is the output from the discussions in the DEV RG. |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16p Machine to Machine (M2M):

Proposed Text on M2M Group Identifier

Kaushik Josiam Ming-Hung Tao

DEV RG Chairs

# Introduction

This contribution proposes the harmonized text regarding to the M2M Group Identifier (MGID).

# Proposed text

---- Begin of proposed text ----

16.2.1.3 Address for machine to machine application

16.2.1.3.1 M2M Group Identifier (MGID)

A 15-bit value that uniquely identifies an M2M group which one or more M2M devices belong to. This ID shall be used to send control messages to a group of devices (e.g., group paging) or to transmit the multicast data to the M2M devices.

An MGID is assigned by a network entity during initial network entry and released during an explicit network exit (e.g., power down location update) or when the device enters DCR mode. The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned; the re-assignment procedure is TBD.

The MGID is obtained by pre-pending each 12 bit STID with values 100 ~111 to generate 4 new IDs from each 12 bit STID. The newly generated addressing space is used by the network entity to assign MGID to M2M devices.

16.3.5.5.2.4 Assignment A-MAP IE

Table 843—Description of CRC Mask

| Masking Prefix (1 bit MSB) | Remaining 15 bit LSBs |  |
| --- | --- | --- |
| 0b0 | Type Indicator | Masking Code |
|  | 0b000 | 12 bit STID or TSTID |
|  | 0b001 | Refer to Table 844 |
|  | 0b010 | Refer to Table 845 |
|  | 0b100 – 0b111 | 12 LSBs of 15 bit MGID |

---- End of proposed text ----