| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast MAP IE modification to support multicasting for M2M |  |
| Date Submitted | 2011-05-12 |  |
| Source(s) | Anil Agiwal, Hyunjeong Kang, Rakesh Taori Samsung Electronics | E-mail: anilag@samsung.com, hyunjeong.kang@samsung.com |
| Re: | Call for Comments on the 802.16p Amendment Working Document , IEEE 802.16p-10/0018r1 |  |
| Abstract | This contribution proposes modification to Multicast MAP IE to allocate resources for a downlink PHY burst carrying the data for an M2M group |  |
| Purpose | To be discussed and adopted in IEEE 802.16p-10/0018r2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multicast MAP IE modification to support multicasting for M2M

Anil Agiwal, Hyunjeong Kang, Rakesh Taori

Samsung Electronics

# Introduction

AWD [1] defines the multicast operation. A 15 bit MGID is also defined in AWD and it identifies an M2M group. The method to allocate resources for multicast burst carrying data for an M2M group is not defined in AWD.

# Proposed Solution

We propose to modify the Broadcast Assignment A-MAP IE to allocate resource for a multicast burst carrying data for an M2M group. This is similar to method used in 16m for multicast traffic. No change is required in CRC masking.

# Proposed text with change marks

16.3 Physical layer

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast or multicast burst or NS-RCH. A broadcast burst contains one or more broadcast MAC control messages. Table 860 describes the fields in a broadcast assignment A-MAP IE.

Table 860—Broadcast Assignment A-MAP IE

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: reservedThis IE carries multicast assignment information for M2M Group identified by M2M Group Identifier (see 16.2.1.3.1) |
| if {Function Index == 0b00} { |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 958 |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Reserved | 13 | Reserved bits |
| }else{ |  |  |
| Reserved | 15 | Reserved bits |
| } else if {Function Index == 0b01} { |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH 1: two NS-RCHs |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity index | 1 | Indicates 2-bit Opportunity index of the ranging channel specified in 16.2.15.3. 0b0: 0b01 0b1: 0b10 |
| } |  |  |
| Reserved | 29/25 | Reserved bits. If Number of Ranging Opportunities is set to 0b0, Size in bits is 29. Otherwise, Size in bits is 25. |
| } else if {Function Index == 0b10} { |  |  |
| Multicast Group ID | 12 | ID of a group that receives multicast assignment |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 958 |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number fo AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 4 | Reserved bits |
| }else if {Function Index == 0b11} { |  |  |
| MGID | 15 | ID of an M2M group that receives multicast assignment |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 958 |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number fo AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 1 | Reserved bits |
| } |  |  |

# References

[1] 80216p-10_0018r1: Enhancements to Support Machine-to-Machine Applications