| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 16m uplink scheduling enhancement to support periodic M2M reporting |  |
| Date Submitted | 2011-05-07 |  |
| Source(s) | Joey Chou Intel | Joey.Chou@intel.com |
| Re: | P802.16p AWD |  |
| Abstract | Propose the 16m uplink scheduling enhancement to support periodic M2M reporting |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

16m uplink scheduling enhancement to support periodic M2M reporting

Joeyn Chou,

Intel

- Introduction

As described in contribution C802.16p-11-0017, M2M applications have a strong bias towards the uplink, and most of the uplink traffic is dedicated to provide non-realtime periodic report.

This contribution proposes uplink scheduling enhancement to support M2M periodic report. Figure 1 shows an example of M2M service creation.

Figure 1: Example of M2M service ceration

The M2M server sends a message to access network (i.e. BS) to pre-provision the M2M services.

- UL periodic reporting interval = 24 hour – indicates the device is to send a report every 24 hours

- UL QoS parameter – indicate the QoS parameter of the uplink connection

- DL QoS parameter – indicate the QoS parameter of the downlink connection

When the device enters the network, the BS should send DSA messages to create uplink and downlink connections.

- M2M periodic reporting interval = 86400 – this new attribute is defined to indicate the interval of periodic report is 86400 seconds, equivalent to 24 hours

- Grant scheduling type = m2mPS – this new scheduling type indicates the delay-tolerant, and long reporting cycle nature of certain M2M traffic

- QoS parameters – QoS parameters, such as

  - Traffic Priority

  - Maximum Sustained Traffic Rate

  - Maximum Traffic Burst

  - Minimum Reserved Traffic Rate

  - Vendor-specific QoS parameters

  - Tolerated Jitter

  - Maximum Latency

- References

- IEEE 802.16p-11/0017, “Optimization of UL grant scheduling for M2M,” March. 2011.

- Proposed Text

3.1 Proposed Text #1

----------------------------------------------  Text Start -----------------------------------------------

16.2.3.47.1 AAI-DSA-REQ

[Change Uplink grant scheduling type at page 227, Line 5, as follows:]

Table 734—AAI-DSA-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| S2: Uplink grant scheduling type | 3 | 0: m2mPS Reserved 1: Undefined (BS implementation-dependent) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service |  |

[Change at page 227, Line 52 as follows:]

Table 734—AAI-DSA-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| } else if (Uplink grant scheduling type == UGS) { |  |  |  |
| L1: Tolerated Jitter | 6 | Extensible look-up table 188 of 6.3.14.4.1. |  |
| S4: Unsolicited grant interval | 6 | Extensible look-up table 189 of 6.3.14.4.1. |  |
| } else if (Uplink grant scheduling type == m2mPS) { |  |  |  |
| M2M periodic reporting interval | 24 | Indicate the interval of M2M periodic report in seconds [0 – 16777215] |  |
| } |  |  |  |
| } // End if (Global Service Class Name) |  |  |  |

[Change at page 229, Line 10 as follows:]

Table 734—AAI-DSA-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Uplink grant scheduling type | 3 | 0: m2mPS Reserved 1: Undefined (BS implementation-dependent) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service |  |

[Change at page 229, Line 56 as follows:]

Table 734—AAI-DSA-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| else if (UL Grant Scheduling Type == aGPS) { |  |  |  |
| : : |  |  |  |
| Adaptation Method | 1 | 0:ABS-initiated adaptation | 1:AMS-initiated adaptation |
| } else if (Uplink grant scheduling type == m2mPS) { |  |  |  |
| M2M periodic reporting interval | 24 | Indicate the interval of M2M periodic report in seconds [0 – 16777215] |  |
| } |  |  |  |
| Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1. | Present if needed |

----------------------------------------------  Text End -----------------------------------------------

3.2 Proposed Text #2

----------------------------------------------  Text Start -----------------------------------------------

16.2.3.47.2 AAI-DSA-RSP

[Change at page 242, Line 48 as follows:]

Table 735—AAI-DSA-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| If (Confirmation Code == 0 && AMS initiated AAI-DSA-REQ) { | FID | 4 |  |
| FID | 4 |  |  |
| If (M2M device) { |  |  |  |
| M2M periodic reporting interval | 24 | Indicate the interval of M2M periodic report in seconds [0 – 16777215] |  |
| } |  |  |  |
| } |  |  |  |

----------------------------------------------  Text End -----------------------------------------------

3.3 Proposed Text #3

----------------------------------------------  Text Start -----------------------------------------------

16.2.47.4 AAI-DSC-REQ

[Change AAI-DSC-REQ at page 249, Line 44, as follows:]

Table 737—AAI-DSC-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| else if (UL Grant Scheduling Type == aGPS) { |  |  |  |
| : : |  |  |  |
| Adaptation Method | 1 | 0:ABS-initiated adaptation | 1:AMS-initiated adaptation |
| } else if (Uplink grant scheduling type == m2mPS) { |  |  |  |
| M2M periodic reporting interval | 24 | Indicate the interval of M2M periodic report in seconds [0 – 16777215] |  |
| } |  |  |  |
| If (Uplink/Downlink Indicator == 0) { |  |  |  |

----------------------------------------------  Text End -----------------------------------------------

3.4 Proposed Text #4

----------------------------------------------  Text Start -----------------------------------------------

16.2.3.47.5 AAI-DSC-RSP

[Change at page 242, Line 48 as follows:]

Table 738—AAI-DSC-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Confirmation Code | 1 | 0 = request was successful. 1 = request was failure |  |
| If (Confirmation Code == 0 && AMS initiated AAI-DSC-REQ && M2M device) { |  |  |  |
| M2M periodic reporting interval | 24 | Indicate the interval of M2M periodic report in seconds [0 – 16777215] |  |
| } |  |  |  |
| } |  |  |  |

----------------------------------------------  Text End -----------------------------------------------