| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MOB_PAG-ADV enhancement to support M2M periodic report |  |
| Date Submitted | 2011-05-07 |  |
| Source(s) | Joey Chou Shantidev Mohant Kerstin Johnsson, Xiangying Yang, Intel Inbar Anson Bratspiess Sequans | Joey.Chou@intel.com inbar@sequans.com |
| Re: | P802.16p AWD |  |
| Abstract | Propose MOB_PAG-ADV enhancement to support M2M periodic report |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MOB_PAG-ADV enhancement to support M2M periodic report

Joeyn Chou,

Shantidev Mohant

Kerstin Johnsson,

Xiangying Yang

Intel

Inbar Anson Bratspiess

Sequans

- Introduction

As described in contribution C802.16p-11-0017, M2M applications have a strong bias towards the uplink, and most of the uplink traffic is dedicated to provide non-realtime periodic report.

This contribution proposes enhancement to MOB_PAG-ADV message in order to support M2M periodic report.

Figure 1 shows an example of paging listening windows that is used to describe how a smart meter can send a report every 24 hours.

Figure 1: Paging Listening Windows for Smart Meter Applications

The smart meter application has the following configuration:

- Frame duration = 5ms

- Paging cycle = 20000 frames = 100 seconds

- 864 paging cycles = 17280000 frames = 86400 seconds = 24 hours

Therefore, the smart meter will wake up every 100 seconds to listen to the MOB_PAG-ADV message. The BS will send a new M2M report code, as defined below, to indicate the opportunity for the smart meter to send the perioci report.

0b0 = No action required

0b1 = Send uplink report

- References

- IEEE 802.16p-10/0017r3, “Optimization of UL grant scheduling for M2M” March. 2011.

- IEEE 802.16p-10/0005, “Machine to Machine (M2M) Communications Technical Report”, Jan 2011

- Proposed Text

3.1 Proposed Text #1

----------------------------------------------  Text Start -----------------------------------------------

6.3.2.3.51 MOB_PAG-ADV (BS broadcast paging) message –

Table 154— MOB_PAG-ADV message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| : |  |  |
| For (j = 0; j < Num_MACs; j++) { | – | – |
| MS MAC Address hash | 24 | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |
| Action Code | 2 | Paging action instruction to MS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network 0b11 = Reserved |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 0b0: No action required 0b1: Send uplink report |
| Reserved | 65 |  |
| } |  |  |
| : |  |  |

M2M periodic code

Indicate the opportunity for the M2M device to send the uplink report

0b0 = No action required

0b1 = Send uplink report

----------------------------------------------  Text End -----------------------------------------------

3.2 Proposed Text #2

----------------------------------------------  Text Start -----------------------------------------------

6.3.23.6 BS Broadcast Paging message

The BS Broadcast Paging message shall also include an action code directing each MS notified via the inclusion of its MS MAC address hash as appropriate:

0b00: No action required

0b01: Perform ranging to establish location and acknowledge message

0b10: Perform initial network entry

0b11: Reserved

The BS broadcast paging message shall include the M2M report code to ask each M2M device identified by MS MAC Address hash to perform the following actions:

0b0 = No action required

0b1 = Send uplink report

----------------------------------------------  Text End -----------------------------------------------