| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M_POLL-ADV to support M2M periodic report |  |
| Date Submitted | 2011-05-19 |  |
| Source(s) | Joey Chou, Shantidev Mohant, Kerstin Johnsson, Xiangying Yang, Intel Inbar Anson Bratspiess Sequans Jin Lee, Kiseon Ryu LGE Hassan Alkanani NEC | Joey.Chou@intel.com inbar@sequance.com jin1.lee@lge.com Hassan.Alkanani@EMEA.NEC.COM |
| Re: | P802.16p AWD |  |
| Abstract | Propose M2M_POLL-ADV to support M2M periodic report |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M_POLL-ADV to support M2M periodic report

Joeyn Chou,

Shantidev Mohant

Kerstin Johnsson,

Xiangying Yang

Intel

Inbar Anson Bratspiess

Sequans

Jin Lee, Kiseon Ryu

LGE

Hassan Alkanani

NEC

- Introduction

As described in contribution C802.16p-11-0017, M2M applications have a strong bias towards the uplink, and most of the uplink traffic is dedicated to provide non-realtime periodic report.

This contribution proposes M2M_POLL-ADV message in order to support M2M periodic report.

Figure 1 shows an example of paging listening windows that is used to describe how a smart meter can send a report every 24 hours.

Figure 1: Paging Listening Windows for Smart Meter Applications

The smart meter application has the following configuration:

- Superframe duration = 20ms

- Paging cycle = 512 SF = 10.24 seconds

- 8437 paging cycles = 4319744 Superframes = 86394.88 seconds  24 hours

Therefore, the smart meter will wake up every 10.24 seconds to listen to the AAI-PAG-ADV message. The ABS will send a new M2M report code, as defined below, to indicate the opportunity for the smart meter to send the perioci report.

0b0 = No action required

0b1 = Send uplink report

- Load control using paging message

When the BS does not poll M2M device, the M2M device shall defer to send the data and see if it is allowed to send the data at next paging cycle. However, if the BS does not poll the M2M device continuously, the data from the M2M device could be outdated and never get to perform ranging to send data. Therefore, the following procedure is recommended.

Figure 2: Load control with paging message

- During idle mode initiation, ‘Transmission type’ and ‘Max time duration to wait for paging message’ can be negotiated. Transmission type is to notify the M2M device of uplink data transmission only after receiving a paging indication. Max time duration is for M2M device to wait for a paging message to send data.

- If the M2M device is in idle and it sends data based on the polling from paging message (Transmission type is set to 1), the M2M device shall check if the paging message includes its identification with uplink report indication

- References

- IEEE 802.16p-10/0017r3, “Optimization of UL grant scheduling for M2M” March. 2011.

- IEEE 802.16p-10/0005, “Machine to Machine (M2M) Communications Technical Report”, Jan 2011

- Proposed Text

3.1 Proposed Text #1

---------------------------------------------- Text Start -----------------------------------------------

[Remedy1] Add the following texts at the end of section 16.2.18.7.1, in page 6 line 65.

16.2.29 M2M UL polling operation

AAI-POLL-ADV with M2M report code set to 0b1 may be used to poll M2M devices for periodic uplink non-realtime data transmission for fixed devices to give an opportunity to transmit UL data. When an M2M device receives the AAI-POLL-REQ message with the Transmission Type set to 1 and Waiting time for UL opportunity attribute, the device may wait for the AAI-POLL-ADV with M2M report code = 1 at Waiting time for UL opportunity. If the M2M device does not receive the AAI-POLL-ADV with M2M report code = 1, it should not send the uplink data.

[Remedy2] Add a new subclause in 16.2.3.

16.2.3.xx AAI-POLL-ADV (M2M UL polling) Message

An AAI-POLL-ADV message is transmitted to give an opportunity for a fixed M2M device to transmit UL data that has been in power saving mode.

[Add a table XXX as follows:]

Table XXX—AAI-POLL-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| MSID | 24 | The hash is obtained by computing a CRC24 on the AMS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 0b0: No action required 0b1: Send uplink report | Present if M2M UL polling is supported |

[Remedy3] Add a new subclause in 16.2.3

16.2.3.yy AAI-POLL-REQ

Table yyy– AAI-POLL-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Transmission Type | 1 | 0 : Reserved 1 : Allowed to send data only after receiving AAI-POLL-ADV with M2M report code 0b1 | Present if M2M UL polling is needed. |
| Waiting time for UL opportunity | 16 | This is for M2M device to wait for AAI-POLL-ADV with M2M report code 0b1. See 16.2.29. | May present if Transmission Type is set to 1. |

----------------------------------------------  Text End -----------------------------------------------

3.3 Proposed Text #3

---------------------------------------------- Text Start -----------------------------------------------

Add a new subclause 6.3.2.3.60 after page 2 line 14.

6.3.2.3.60 M2M_POLL-ADV (M2M UL polling) message

An M2M_POLL-ADV message is transmitted to give an opportunity for a fixed M2M device to transmit UL data that has been in power saving mode.

The M2M_POLL-ADV message shall be sent on the Broadcast CID.

The format of the message is shown in Table 167.

Table 167— M2M_POLL-ADV message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| M2M_POLL-ADV_Message_format() { |  |  |
| Management Message Type = 70 | – | – |
| Num_M2M_Devices | 8 | Number of M2M devices |
| For (j = 0; j < Num_M2M_Devices; j++) { |  |  |
| M2M device MAC Address hash | 24 | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |
| M2M action code | 1 | Indicate the opportunity for the M2M device to send the uplink data 0b0: No action required 0b1: Send uplink data |
| Reserved | 7 |  |
| } |  |  |

[Remedy4] Add a new subclause 6.3.28 after page 2 line 14.

6.3.28 M2M UL polling operation

M2M_POLL-ADV with M2M action code set to 0b1 may be used to poll M2M devices for periodic uplink non-realtime data transmission for fixed devices.

----------------------------------------------  Text End -----------------------------------------------