| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging Instructions for M2M Network Re-Entry |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Ping-Heng Kuo, Wei-Chieh Huang, Yu-Tao Hsieh, Pang-An Ting ITRI Hsi-Pin Ma, Jen-Ming Wu, Yuan-Hao Huang, Yeong-Luh Ueng, Shih-Yu Chang NTHU | E-mail: pinghengkuo@itri.org.tw |
| Re: | Call for contributions on the 802.16p Amendment Working Document |  |
| Abstract | This contribution provides a method to distribute ranging opportunities among different M2M devices and groups, in order to alleviate the anticipated congestions of network re-entry. |  |
| Purpose | To be discussed and adopted by 802.16p WG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ranging Instructions for M2M Network Re-Entry

Ping-Heng Kuo, Wei-Chieh Huang, Yu-Tao Hsieh, Pang-An Ting

ITRI

Hsi-Pin Ma, Jen-Ming Wu, Yuan-Hao Huang, Yeong-Luh Ueng, Shih-Yu Chang

NTHU

# Introduction

Severe congestions can be envisaged in M2M network re-entry, since potentially a large number of devices would attempt to perform ranging simultaneously. Actuated by this anticipation, a number of contributions submitted to 802.16p PWR RG suggested that the access opportunities should be distributed among M2M devices and groups. Specifically, the network could assign different ranging parameters to devices using paging or some other MAC messages.

This contribution proposes a ranging parameter assignment mechanism, based on a look-up table that shall be pre-configured at both ABS and M2M device. The look-up table is consisted of multiple sets of ranging parameters, and each of the sets is associated with an index dubbed as ranging instruction index (RII). By including the index value in the paging message, the ABS could provide M2M devices the ranging information that is to be used. In particular, the chance of collision could be reduced by altering the probability density functions that devices leverage when selecting their random backoff times.

- Text proposal for inclusion in IEEE 802.16p-10/0018r1 (802.16p AWD)

--------------------------------- Start of Proposed Text Part 1 ---------------------------------

16.2.18.X AAI-PAG-ADV

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Num_PAG_GRP | TBD | The number of M2M groups to be paged. |  |
| For (j=0; j<Num_PAG_GRP; j++) { |  |  |  |
| Group ID | TBD | M2M Group ID |  |
| Action Code | 1 | 0b0: Perform network re-entry 0b1: Perform location update |  |
| Ranging Instruction Index (RII) | TBD | The devices should use the ranging parameters correspond to the assigned RII in accordance to Table xxx. | Present if network re-entry is required (Action Code = 0b0). |
| } |  |  |  |

--------------------------------- End of Proposed Text Part 1 ---------------------------------

--------------------------------- Start of Proposed Text Part 2 ---------------------------------

16.2.18.7.2 Network re-entry from idle mode for M2M devices

For network reentry from Idle Mode, ranging parameters may be different for M2M devices or M2M groups.

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in Table 833 in 16.3.5.5.1.2.

For the network reentry indicated by a paging message that contains ranging configuration, the M2M device shall select a ranging opportunity according to the ranging configuration. Ranging configuration may include differentiated waiting offset time and backoff window size. The configurations are notified by including the information of ranging instruction index (RII) in the paging message. Each RII is associated to a set of parameters, encompassing a delay probability, backoff window size, and waiting interval length; the detailed values of which can be extracted by mapping with Table XXX. The parameters of different RIIs would result in distinct probability density functions for random backoff selection among devices.

Once the information of RII is received, the device should generate a random number between 0 and 1, and then check if this random number is larger or smaller than the delay probability value associated with its assigned RII. If the randomly generated value is larger, the device should initiate the network reentry operation based on the backoff window size indicated in its assigned RII. Conversely, if the randomly generated value is smaller than the delay probability, the device should delay its ranging attempt in accordance to the waiting interval length indicated in its assigned RII.

| Ranging Instruction Index (RII) | Delay Probability | Backoff Window Size | Waiting Interval Length |
| --- | --- | --- | --- |
| 0 | TBD | TBD | TBD |
| 1 | TBD | TBD | TBD |
| 2 | TBD | TBD | TBD |
| …. | TBD | TBD | TBD |
| TBD | TBD | TBD | TBD |

Table XXX: Ranging instructions for network re-entry

--------------------------------- End of Proposed Text Part 2 ---------------------------------